3GPP TSG SA WG2 Meeting #119	S2-171033

13 - 17 February 2017, Dubrovnik	(revision of S2-17xxxx)

Source:	Huawei, Hisilicon

Title:	TS 23.501: Architecture for network slicing

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:  5GS_ph1/Rel-15

Abstract of the contribution: This contribution proposes architecture reference model for network slicing to support UE accessing one slice and multiple slices use cases.

# Introduction

This document proposes non-roaming service-based architecture reference model with network slicing support for both UE accessing one slice and multiple slices use cases to address the impacts of network slicing on architecture.

# 2. Proposal

It is proposed to capture the following clause in the system architecture TS 23.501.

* * * Start of changes * * *

### 5.15	Network Slicing

### 5.15.x	Non-roaming architecture with network slice support

# Figure 5.13.x-1: Non-roaming service based architecture for one UE accessing one slice

# Figure 5.13.x-2: Non-roaming service based architecture for one UE accessing multiple slices

NOTE 1: Multiple SMF instances and UPF instances within one network slice may be involved to serve one UE.

NOTE 2: The SMF may be either a common network function or a slice-specific network function, which depends on the network slice deployment of the operator.

Editor’s note: Whether the PCF can be deployed as a slice-specific network function is FFS.

Editor’s note: Whether the NEF can be deployed as a slice-specific network function is FFS.

Editor’s note: Whether some security related network functions (in current AMF) can be deployed as slice-specific network functions depends on the discussion in SA3 and is FFS.

* * * End of changes * * *