SA WG2 Meeting #119	S2-171080

13 - 17 Feb 2017   Dubrovnik, Croatia	(revision of S2-17xxxx)

Source:	Samsung

Title:	For MO only – rename and add implicit deregistration

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Propose to rename MO only mode to Mobile On-demands mode and additional description for implicit deregistration.

# Discussion

#### 1) re-name of MO only

The name of MO only mode (Mobile-Originated only mode) has been introduced to cover the case of the UE moves to CM-Connected only when the UE wants to send uplink data. This mode can be characterized as there is no paging during the CM-IDLE, but it does not mean the UE cannot receive MT data. It was agreed as the MO only mode UE can receive MT data during the UE in CM-Connected. Therefore, the name of MO only has contradiction itself with the operation for this feature.

Rather than use MO only, this paper proposes to use the name as “Mobile On-demands” to reflect the concept for which the UE requests connection only when the UE wants to send data.

Proposal 1: Rename MO only to Mobile On-demands

#### 1) Implicit deregistration for Mobile On-demands mode UE

In the TR conclusion, following has been captured:

5.	If the UE needs to be de-registered at the end of CN-Connected mode, the UE can provide the preference e.g.:

-	CN-IDLE mode is required, or CN-IDLE mode is not required.

It was introduced to capture the UE can perform deregistration at the end of communication. This feature is useful along with MO only mode because the UE in MO only mode does not perform specific IDLE mode procedure (e.g., paging, registration area update due to mobility) except periodic registration procedure. Therefore, it is proposed to capture this feature in terms of MO only mode support.

Proposal 2: The UE can perform implicit deregistration at the end of CM-Connected state based on the negotiation during the registration signalling.

# Proposal

Add the following to the TS 23.501.

*************** Start of changes *********************

#### 5.4.1.3	Mobile On-demands mode

A UE may indicate preference for Mobile On-demands mode during registration and/or registration update. The AMF, based on subscription, local configuration based on UE indicated preferences, UE subscription information and network policies, or any combination of them, determines whether Mobile On-demands mode is applied for the UE and indicates it to the UE during registration signalling.

The UE and core network re-initiates or exits the Mobile On-demands mode at subsequent registration signalling. Mobile On-demands mode needs to be applied explicitly in every registration and registration update.

When the AMF applies Mobile On-demands mode to a UE, the registration area is not constrained by paging area size.

Editor's note:	It is FFS whether AMF should provide a registration area to UE in Mobile On-demands mode. If the registration area is provided, the registration area size may be bound to Mobility restrictions (i.e. the registration may be contained by the allowed area) and by requirements to perform tracking for the UE.

When the AMF applies Mobile On-demands mode to a UE, the AMF considers the UE always unreachable while in CM-IDLE. The UE in Mobile On-demands mode is only reachable for mobile terminated data or signalling when the UE is in CM-CONNECTED mode for the PDU sessions that are resumed. The CN rejects any request for downlink data delivery for an Mobile On-demands UE in idle mode. The CN also does not allow downlink transport over NAS for SMS, location services, etc.

A UE in Mobile On-demands mode performs periodic registration as determined by the periodic registration timer.

A UE in Mobile On-demands mode does not listen to paging while in CM-IDLE. A UE in Mobile On-demands mode may stop any access stratum procedures in CM-IDLE, until the UE initiates CM-IDLE to CM-CONNECTED mode procedures due to one of the following triggers:

-	A change in the UE (e.g. change in configuration) requires an update its registration with the network.

-	Periodic registration timer expires

-	MO data pending

-	MO signalling pending (e.g. SM procedure initiated)

Editor's note:	If registration area is allocated to UE in Mobile On-demands mode. Then the UE determines if it is within the registration area or not when it has MO data or MO signalling.

If a UE may indicate preference for implicit deregistration along with the preference for Mobile On-demands mode during registration and/or registration update, the UE in Mobile On-demands mode can perform implicit deregistration at the end of CM-CONNECTED state.

Editor’s note: It is FFS whether and how AMF and UE negotiates implicit deregistration timer to determine deregistration of the UE.

*************** End of changes *********************