SA WG2 Meeting #119	S2-171507

13 - 17 Feb 2017   Dubrovnik, Croatia	(revision of S2-171080)

Source:	Samsung

Title:	for deregistration at the end of communication.

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Propose to resolve FFS for name of MO only mode and additional description for deregistration at the end of communication.

# Discussion

1) Implicit deregistration for Mobile On-demands mode UE

In the TR conclusion, following has been captured:

5.	If the UE needs to be de-registered at the end of CN-Connected mode, the UE can provide the preference e.g.:

-	CN-IDLE mode is required, or CN-IDLE mode is not required.

It was introduced to capture the UE can perform deregistration at the end of communication. Therefore, it is proposed to capture this feature

Proposal 2: The UE can perform deregistration at the end of CM-Connected state based on the negotiation during the registration signalling.

# Proposal

Add the following to the TS 23.501.

*************** Start of changes *********************

5.4.1.4	Mobile Deregistration at the End of Communication

Editor’s Note: It is FFS to define the name of this functionality.

A UE can perform deregistration at the end of communication without additional NAS signalling. The UE may indicate preference for the deregistration at the end of communication during registration procedure. The AMF determines whether it is supported for the UE, and indicates during the registration signalling. When the AMF applies the deregistration at the end of communication to the UE, the AMF considers the UE enters RM-DEREGISTERED at the release of N2 connection for the UE, and the UE moves to RM-DEREGISTERED when leaving CM-CONNECTED.