SA WG2 Meeting #119	S2-171576

13-17 February 2017, Dubrovnik, Croatia	(was S2-171224)

Source:	Sony

Title:	Fast Connection Set-up solution

Document for:	Approval

Agenda Item:	6.10

Work Item / Release:	FS-REAR/Rel-15

Abstract of the contribution: This solution proses a solution to speed up the PC5 connection set-up by utilizing end-to-end AS security.

# Discussion

According to 23.303

Observation 1: Remote UE can initiate Direct Communication if the Remote UE has knowledge about the Layer-2 ID

Observation 2: In ProSe R13 it is required to set-up a secure link after the Direct Communication Request have been sent to the Relay UE.

For enhancing the connection setup, we would like to find ways to optimize this procedure. In ProSe R13 the Remote UE and Relay UE have a PDCP connection and this can be used for AS encryption of the PC5-u. For Layer-2 relay, we have the PDCP connection between the eRemote UE and the eNodeB that will provide AS security protection for all links between the eRemote UE and the eNodeB.

Observation 3: There is an AS security relation between the eRemote UE and the eNodeB. This security relation can be used to protect the data sent over PC5.

Proposal 1: Utilize the AS security protection provided on PCDP level between eRemote UE and eNodeB to provide a secure PC5 link.

According to TR33.899 study in SA3, SA3 is looking into how to authenticate remote UE via relay UE, even though this study has not concluded it could be valuable to see if any solutions in TR33.899 can be valid for REAR.

The procedure above focus on the mutual authentication UE-Network, however it would be interesting to use some of the concepts. We see that the Remote UE generates a Remote UE Indicator (RUE_indicator). We see that the Relay UE can identify and verify the message from the Remote UE by looking at the RUE_indicator. If the RUE_indicator is valid the Relay will forward the message to the network.

Observation 4: The RUE_Indicator can be used to validate that the message is coming from an authenticated and authorized Remote UE.

Proposal 2: Use the SA3 concept of RUE_indicator to verify any messages send by the eRemote UE to the eRelay is coming from an authenticated and authorized Remote UE.

# Conclusion

The following observation and proposals shall be considered.

Observation 1: Remote UE can initiate Direct Communication if the Remote UE has knowledge about the Layer-2 ID

Observation 2: In ProSe R13 it is required to setup a secure link after the Direct Communication Request have been sent to the Relay UE.

Observation 3: There is an AS security relation between the eRemote UE and the eNodeB. This security relation can be used to protect the data sent over PC5.

Proposal 1: Utilize the AS security protection provided on PCDP level between eRemote UE and eNodeB to provide a secure PC5 link.

Observation 4: The RUE_Indicator can be used to validate that the message is coming from an authenticated and authorized Remote UE.

Proposal 2: Use the SA3 concept of RUE_indicator to verify any messages send by the eRemote UE to the eRelay is coming from an authenticated and authorized Remote UE.

# Proposal

We propose to update TR 23.733 with a solution reflecting above observations and proposals.

* * * Start of changes * * * *

## 6.y	Solutions to key issue 3

### 6.y.x	Solution X: Fast Connection setup by optimizing eRemote UE security procedures

#### 6.y.x.1	Description

#### This solution proposes a way to optimize the connection setup between the eRemote UE connection and the network in order to facilitate fast connection setup. In TS 23.303 clause 5.4.5 the procedure is described how to set up One-to-one ProSe Direct Communication. It involve two steps. First, UE-1 sends a Direct Communication Request message to UE-2. The message then triggers step two, the mutual authentication and security association between UE-1 and UE-2.

This solution for Layer-2 relay proposes to utilize the fact that for traffic using indirect 3GPP communication between the eRemote UE and eNodeB, the traffic is security protected by legacy AS security procedures. This protection can be used to provide a secure PC5 link. The optimization proposed for setting up One-to-one communication between an eRemote UE and an eRelay UE, is to omit the step 2 in TS23.303 clause 5.4.5, since the PC5 is secured by the AS security between eRemote and eNodeB.

#### Editor’s Note: As secure communication between eRemote UE and eNB can be achieved with PDCP, whether the secure communication over PC5 (signalling, data or both) is still needed should be decided by SA3.

#### 6.1.1.2	Impact analysis

#### 6.1.1.3	Evaluation

* * * End of changes * * * *