SA WG2 Meeting #120	S2-172492

27 - 31 March 2017, Busan, South Korea	(revision of S2-172084)

Source:	LG Electronics

Title:	TS 23.502: Handover procedure between 3GPP and non-3GPP

Document for:	Approval

Agenda Item:	6.5.1 General aspects, concepts and reference models

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes handover procedure between 3GPP and non-3GPP accesses.

Discussion

Proposal

Add the following to the TS 23.502.

* * * * Start of 1st Change * * * *

## 4.9	Handover procedures

### 4.9.1	Handover procedures in 3GPP access

Editor's note:	(S2-167180):

1)	Intra NG RAN node,

2)	Inter NG RAN node with Xn interface,

3)	Intra AMF, Intra SMF, Inter NG RAN node without Xn interface,

4)	Intra AMF, Inter SMF, Inter NG RAN node without Xn interface,

5)	Inter AMF, Inter and Intra SMF, Inter NG RAN node without Xn interface.

"NG RAN node" naming discussed in RAN, and AMF to be changed when new name agreed.

### 4.9.1.1	Xn based inter NG RAN handover

This clause includes details regarding the Xn based inter NG RAN handover with and without UPF relocation.

Editor's note:	Impact of centralized deployment being discussed in RAN3 is FFS.

Editor's note:	It is FFS whether to define separate call flows for roaming scenarios.

Editor's note:	Definition of Xn is defined by RAN.

#### 4.9.1.1.1	Xn based inter NG RAN handover without user plane function relocation

This procedure is used to hand over a UE from a source RAN to target RAN using Xn when the AMF is unchanged and the SMF decides to keep the existing UPF. The UPF referred in this clause is the UPF which terminates N3 interface in the NGC. The presence of IP connectivity between the Source UPF and Target UPF is assumed.

The call flow is shown in figure 4.9.1.1.1-1.

Figure 4.9.1.1.1-1 – Xn based inter NG RAN handover without UPF relocation

1.	The Target RAN sends an N2 Path Switch Request message to an AMF to inform that the UE has moved to a new target cell and a list of PDU Sessions to be switched or removed. Depending on the type of target cell, the Target RAN includes appropriate information in this message.

2.	The AMF sends a N11 Message to each SMF associated with the list of PDU Sessions received in the N2 Path Switch Request.

Upon receipt of the N11 Message, each of these SMFs determines whether the existing UPF can continue to serve the UE. If the existing UPF cannot continue to serve the UE and it is not a PDU session anchor, steps 3-11 of clause 4.9.1.1.2 are performed. Otherwise, the following steps 3 to 6 are performed by each of these SMFs if their existing UPFs can continue to serve the UE:

Editor's note:	Handling of UPF relocation of the PDU session anchor is FFS.

3.	The SMF initiates release of PDU Sessions which are not requested by the Target RAN. For PDU Sessions requested by the Target RAN, the SMF sends an N4 Session Modification Request (RAN address, tunnel identifiers for downlink user plane) message to the UPF.

4.	The UPF returns an N4 Session Modification Response (Tunnel identifiers for uplink traffic) message to the SMF after requested PDU Sessions are switched.

5.	In order to assist the reordering function in the Target RAN, the UPF sends one or more "end marker" packets on the old path immediately after switching the path. The UPF starts sending downlink packets to the Target RAN.

NOTE:	Step 6 can occur any time after receipt of N4 Session Modification Response at the SMF.

6.	The SMF sends an N11 Message Ack (CN Tunnel Information) to the AMF for PDU Sessions which have been switched successfully.

7.	Once the N11 Message Response is received from all the SMFs, the AMF aggregates received CN Tunnel Information from these responses and sends this aggregated information as a part of N2 SM Information in N2 Path Switch Request Ack to the Target RAN. If none of the requested PDP Sessions have been switched successfully, the AMF shall send an N2 Path Switch Request Failure message to the Target RAN.

8.	By sending a Release Resources message to the Source RAN, the Target RAN confirms success of the handover. It then triggers the release of resources with the Source RAN.

#### 4.9.1.1.2	Xn based inter NG RAN handover with user plane function relocation

This procedure is used to hand over a UE from a Source RAN to a Target RAN using Xn when the AMF is unchanged and the SMF decides that the Source UPF is to be relocated.

Editor's Note: Impact on UL-CL and/or BP functions is FFS.

The Source UPF referred in this clause is the UPF which terminates N3 interface in the NGC. The presence of IP connectivity between the Source UPF and Source RAN, and between the Target UPF and Target RAN, is assumed.

The call flow is shown in figure 4.9.1.1.2-1.

Figure 4.9.1.1.2-1 - Xn based inter NG RAN handover with Source UPF relocation

Steps 1-2 are the same as discussed in clause 4.9.1.1.1.

3.	The SMF initiates release of PDU Sessions which are not requested by the Target RAN. The SMF then selects a new Target UPF based on UPF Selection Criteria according to clause 6.3.3 of TS 23.501 [2]. Target UPF IP address assignment and allocation of downlink and uplink tunnel identifiers are performed by the SMF. An N4 Session Establishment Request (Target RAN address, uplink and downlink tunnel identifiers) message is sent to the Target UPF.

4.	The Target UPF sends an N4 Session Establishment Response message to the SMF. The SMF starts a timer, to be used in step 10.

5.	The SMF sends N4 Session Modification message to the PDU session anchor.

6.	The PDU session anchor responds with the N4 Session Modification Response message. At this point, PDU session anchor starts sending downlink packets to the Target RAN using the address and tunnel identifiers of the Target RAN via Target UPF.

7.	The SMF sends a N11 Message Ack (CN Tunnel Information) to the AMF.

Steps 8-9 are same as steps 7-8 defined in clause 4.9.1.1.1.

10.	Once the timer has expired after step 4, the SMF initiates Source UPF release procedure by sending an N4 Session Termination Request (Release Cause).

11.	The Source UPF acknowledges with an N4 Session Termination Response message to confirm the release of resources.

4.9.X	Handover procedures between 3GPP and non-3GPP accesses

4.9.X.1	Handover procedure from untrusted non-3GPP to 3GPP access (non-roaming and roaming with local breakout)

This clause specifies how a UE can handover a PDU session from untrusted non-3GPP access to 3GPP access. It is based on the PDU session establishment procedure for 3GPP access as specified in clause 4.3.2.

Figure 4.9.X.1-1: Handover procedure from untrusted non-3GPP access to 3GPP access (non-roaming and roaming with local breakout)

1.	If the UE is not registered via 3GPP access, the UE shall initiate registration procedure as defined in clause 4.2.2.2.2.

2.	The UE performs a PDU Session Establishment procedure as specified clause 4.3.2.2.1.

4.9.X.2	Handover procedure from 3GPP to untrusted non-3GPP access (non-roaming and roaming with local breakout)

This clause specifies how a UE can handover a PDU session from 3GPP access to untrusted non-3GPP access. It is based on the PDU session establishment procedure for non-3GPP access as specified in clause 4.12.4.

Figure 4.9.X.2-1: Handover from 3GPP access to untrusted non-3GPP access (non-roaming and roaming with local breakout)

1.	If the UE is not registered via untrusted non-3GPP access, the UE shall initiate registration procedure as defined in clause 4.12.2.

2.	The UE performs PDU Session Establishment procedure as specified in clause 4.12.4.

Editor's note: The clause 4.12.14 should be updated to describe handover procedure.

* * * * End of Changes * * * *