SA WG2 Meeting #121	S2-173154

15-19 May 2017, Hangzhou, China	(was S2-17xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, Samsung, NTT DOCOMO

Title:	TS 23.501: AMF Selection

Document for:	Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes description for AMF selection

# 1	Introduction

AMF selection function is used by RAN or N3IWF or other core NFs (e.g. AMF, etc). The details of AMF selection is related to UE stickiness removal solutions, Network slicing etc.

The AMF selection function may consider the selection criteria such as NSSAI, temporary UE ID, Load conditions, Network topology, etc.

Following scenarios have to be addressed:

- the UE provides no Temp Id to the 5G AN  need to first select an AMF Group, then the AMF.

- the UE provides Temp Id to the 5G AN but the Temp Id is from another area  need to first select an AMF Group.

The selection comprises of 2 steps:

- select an AMF group

- Select an AMF within the group

AMF selection function within an AMF can use NRF for NF discovery, however, when RAN or N3IWF selects an AMF, NRF is not used.

# 2	Proposal

It is proposed to modify TS 23.501 as follows…

* * * * Start Change * * *.*

## 6.3	Network function discovery and selection

## 6.3.x	AMF selection

The AMF selection function is applicable to both 3GPP access and non-3GPP access. The AMF selection function can be supported by the 5G-AN (e.g. RAN, N3IWF) and is used to select an AMF for a given UE. An AMF supports the AMF selection function to select an AMF for relocation or because the initially selected AMF was not an appropriate AMF to serve the UE (e.g. due to change of allowed NSSAI based on subscription).

Editor’s note: whether there are other NFs that will support AMF selection function is FFS.

AMF selection function in the 5G-AN and N3IWF are not required to utilize the NRF to discover the AMF instance(s). When the 5G-AN performs AMF selection and locally valid temporary ID is not provided by the UE to the RAN, then RAN selects an AMF Group and an AMF from the AMF Group. When locally valid temporary ID is provided by the UE to the 5G-AN but the AMF Group within temporary ID is not connected to the AN (i.e. UE has moved to a new AMF Group area), then the 5G-AN selects an new AMF Group and an AMF.

Editor’s note:	The above statement will be updated based on the conclusion on whether the shortened form of temporary ID (e.g., like S-TMSI used for paging and Service Request in EPC) is used for more efficient RRC procedures.

The AMF selection function in the 5G-AN and N3IWF may consider the following factors for selecting the AMF Group:

-	Requested NSSAI.

-	Local operator policies.

It considers the following factors for selecting an AMF from AMF Group:

-	Load conditions of the candidate AMFs.

-	Availability of candidate AMFs.

When locally valid temporary ID is provided by the UE to the 5G-AN, then the 5G-AN uses the AMF Group ID for AMF selection.

## Editor’s note: any impact to AMF selection due to the solution principles for stickiness removal and network slicing is FFS.

## The AMF selection function in the AMF shall utilize the NRF to discover the AMF instance(s) unless AMF information is available by other means, e.g. locally configured on AMF. The NRF provides the IP address or the FQDN of AMF instance(s) to the AMF. In addition to the factors listed above, AMF selection function within the AMF considers the following factor for selecting any AMF from the AMF Group:

## -	Subscription information from UDM (e.g. allowed NSSAI).

* * * * End Change * * *.*