SA WG2 Meeting #121	S2-173159

27-31 March 2017, Busan, Korea	(was S2-17xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, AT&T, SK Telecom

Title:	TS 23.502: UDM services identification

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes UDM services and service operations

# 1	Background

During the last SA2, following was agreed:

- NF services are derived from the information flows that describe end to end functionality, where applicable (see 3GPP TS 23.502 [3], Annex B drafting rules).

- Complete end to end message flows can be described by a sequence of NF service invocations.

NOTE:	While the use of NF services is not restricted to the identified consumers, no specific effort will be made in this release of the specification, to develop the description of NF services beyond what is necessary to support their use in information flows.

It was also agreed that:

“Each of the NF services offered by a Network Function shall be self-contained, reusable and use management schemes independently of other NF services offered by the same Network Function (e.g. for scaling, healing, etc).”

“NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service operation: An elementary unit a NF service is composed of. “

It is proposed that we summarize the NF services in a table with service description, input/output parameters, other relevant information and have a column to determine whether the given call flow is referring to NF service operation or NF service.

# 2	Criteria

We consider the following set of criteria for defining the services:

- each NF service operates on its own set of context(s)

- services operations that operate on a common context data shall be defined within a single service. Any direct access of a context owned by a NF service are only made by the service operations of that NF service.

- One non-service based interface shall be tied to at most one NF service in an NF.

# 3	Considerations for identifying UDM services and service operations

The services exhibited by UDM should mainly include features that are essential for other Network functions would need as described in e2e call flows in section 4.

To start with, the list of functions that UDM performs need to be considered. From 3GPP TS 23.501 section 6.2.7 the following are the list of UDM functions:

The UDR stores data required for functionalities provided by UDM-FE, plus policy profiles required by PCF. The data stored in the UDR includes:

-	User subscription data, including subscription identifiers, security credentials, access and mobility related subscription data and session related subscription data

-	Policy data, as defined in A.3.1.4.

The UDM-FE accesses subscription information stored in a User Data Repository (UDR) and supports the following functionality:

-	Authentication Credential Processing.

-	User Identification Handling.

-	Access Authorization.

-	Registration/Mobility management.

-	Subscription management.

-	SMS management.A front end implements the application logic and does not require an internal user data storage. Several different front ends may serve the same user in different transactions.

NOTE 2:	The interaction between UDM and HSS is implementation specific.

It is proposed to update TS 23.502 to include the following services for UDM

If this is agreeable, then it is proposed to update the call flows in section 5 accordingly (i.e. remove the ones that are not relevant as per e2e call flows in section 4).

* * * * Start Change * * *.*

## 5.2	Network Function services

Editor's note:	Procedures that can be supported as a service by NF.

Editor's note:	The identified NF service(s) described in this clause need be aligned with the end to end call flow.

Editor's note:	The granularity and structure of NF service(s) in this clause are to be evaluated.

Editor's note:	The NF services listed in this section are non-exhaustive and new NF services will be added per progress of the normative work.

### 5.2.1	General

Editor's note:	It is expected that all control plane interactions specified in section 4 for CP NFs will be covered by final list of services and its service operations defined for CP NFs.

### 5.2.3	UDM Services

Editor's note:	The identified NF service(s) in this clause are to be evaluated. 5.2.3.1 "Register UE serving NF" service.

Editor's note:	How to define a service based procedure for "Purge of subscriber data from AMF" is to be determined.

#### 5.2.3.1	General

Table 5.2.3-1: NF Services provided by UDM

| NF service | Description | NF service operations | Consumer | Ref. |
| --- | --- | --- | --- | --- |
| Subscriber Data Management | Retrieve, update, delete, Subscriber data | Nudm_Subscriber_Data_Get | AMF | Many call flows in section 4 e.g. 4.5.2, 4.5.1 |
|  |  | Nudm_SubscriptionData_UpdateNotification | NEF, SMF, SMSF, AUSF, PCF |  |
|  |  |  | SMF, SMSF, AUSF, PCF |  |
| Serving NF management | Register, Deregister and retrieve UE’s Serving NF information. | Nudm_Serving NF_Registration, | AMF, SMF, SMSF | Many call flows in section 4, 4.5.2 |
|  |  | Nudm_Serving NF_RemoveNotification | AMF, SMF, SMSF |  |
|  |  | Nudm_SubscriberData_Get_Purge | AMF |  |
|  |  | Nudm_Serving NF_Get | NEF |  |
| UE reachability notification | UE reachability notification to the requestor. | Nudm_UE ReachabilityNotification | NEF | 4.2.5.2 |

Editor's note:	The identified NF service(s) in this clause are to be evaluated.

Next change

#### 5.2.3.2	Serving NF Management service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Nudm_Serving NF_Registration, | Register Serving NF | SUPI, Requester NF ID | Result indication |
| Nudm_Serving NF_RemoveNotification | UDM notifies the Requester NF, which has subscribed the remove notification before, the Requester NF ID has been removed from the UDM due to a new serving NF for the UE registered in the UDM. | Serving NF registered in UDM for a SUPI (implicit) | SUPI, serving NF change reason SUPI, serving NF removal reason. |
| Nudm_SubscriberData_Get_Purge | UDM deletes the information related to the Requester NF in the UE context. | SUPI | Result Indication. |
| Nudm_Serving NF_Get | Retrieve Serving NF of a certain NF type for a given UE. | SUPI, NF type. | Serving NF ID |

#### 5.2.3.3	Subscriber Data Management Service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Nudm_Subscriber_Data_Get | Retrieve Subscriber data for a certain UE | SUPI | Requested Subscription data |
| Nudm_SubscriptionData_UpdateNotification | The UDM notifies NF consumer of the updates of UE’s subscriber data | Serving NF registered in UDM implicitly/explicitly for notification | SUPI, Updated subscription Data The UDM checks the UE ID and NF type to retrieve corresponding subscriber data and providing the data to the Requester NF. In case of Requester NF is SMF, the subscriber data may contain e.g., PDU type(s), authorized SSC mode(s), Default QoS profile. |

#### 5.2.3.7	Reachability_Notification service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Nudm_UE ReachabilityNotification | Retrieve Subscriber data for a certain UE | UE ID | UDM notifies the requester NF when the corresponding UE is reachable |

End changes