SA WG2 Meeting #121	S2-173158

27-31 March 2017, Busan, Korea	(was S2-17xxxx)

Source:	Nokia, Alcatel-Lucent Shanghai Bell, AT&T, SK Telecom

Title:	TS 23.502: AMF services identification

Document for:	Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: Proposes AMF services and service operations.

# 1	Background

During the last SA2, following was agreed:

- NF services are derived from the information flows that describe end to end functionality, where applicable (see 3GPP TS 23.502 [3], Annex B drafting rules).

- Complete end to end message flows can be described by a sequence of NF service invocations.

NOTE:	While the use of NF services is not restricted to the identified consumers, no specific effort will be made in this release of the specification, to develop the description of NF services beyond what is necessary to support their use in information flows.

It was also agreed that:

“Each of the NF services offered by a Network Function shall be self-contained, reusable and use management schemes independently of other NF services offered by the same Network Function (e.g. for scaling, healing, etc).”

“NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service operation: An elementary unit a NF service is composed of. “

It is proposed that we summarize the NF services in a table with service description, input/output parameters, other relevant information and have a column to determine whether the given call flow is referring to NF service operation or NF service.

# 2	Criteria

We consider the following set of criteria for defining the services:

- each NF service operates on its own set of context(s)

- services operations that operate on a common context data shall be defined within a single service. Any direct access of a context owned by a NF service are only made by the service operations of that NF service.

- One non-service based interface shall be part at most one NF service in an NF.

# 3	Considerations for identifying AMF services and service operations

Some functionalities are mandatory while some functionalities are optional. Some optional functionalities are essential for other NFs to use for both network internal operation (e.g. SMF, PCF, SMSF) and to manage network external requests (e.g. NEF). The services that are mandatory for an AMF should mainly include those that are always or frequently used by other NFs as described in e2e call flows in section 4. Services that are optional for an AMF should include those that in some scenarios are not used, and hence need not be included in some AMF instantiations.

To start with, the list of functions that AMF performs need to be considered. From 3GPP TS 23.501 section 6.2.1 the following are the list of AMF functions:

-	Termination of RAN CP interface (N2).

-	Termination of NAS (N1), NAS ciphering and integrity protection.

-	Registration management.

-	Connection management.

-	Reachability management.

-	Mobility Management.

-	Lawful intercept (for AMF events and interface to LI System).

-	Provide transport for SM messages between UE and SMF.

-	Transparent proxy for routing SM messages.

-	Access Authentication.

-	Access Authorization.

-	Provide transport for SMS messages between UE and SMSF.

-	Security Anchor Function (SEA). It interacts with the AUSF and the UE, receives the intermediate key that was established as a result of the UE authentication process. In case of USIM based authentication, the AMF retrieves the security material from the AUSF.

-	Security Context Management (SCM). The SCM receives a key from the SEA that it uses to derive access-network specific keys.

NOTE:	Regardless of the number of Network functions, there is only one NAS interface instance per access network between the UE and the CN, terminated at one of the Network functions that implements at least NAS security and mobility management.

In addition to the functionalities of the AMF described above, the AMF may include the following functionality to support non-3GPP access networks:

-	Support of N2 interface with N3IWF. Over this interface, some information (e.g. 3GPP cell Identification) and procedures (e.g. Hand-Over related) defined over 3GPP access may not apply, and non-3GPP access specific information may be applied that do not apply to 3GPP accesses.

-	Support of NAS signalling with a UE over N3IWF. Some procedures supported by NAS signalling over 3GPP access may be not applicable to untrusted non-3GPP (e.g. Paging) access.

-	Support of authentication of UEs connected over N3IWF.

-	Management of mobility, authentication, and separate security context state(s) of a UE connected via non-3GPP access or connected via 3GPP and non-3GPP accesses simultaneously.

-	Support as described in clause 5.3.2.3 a co-ordinated RM management context valid over 3GPP and Non 3GPP accesses.

-	Support as described in clause 5.3.3.4 dedicated CM management contexts for the UE for connectivity over non-3GPP access.

It is proposed to update TS 23.502 to include the following services for AMF:

If this is agreeable, then it is proposed to update the call flows in section 5 accordingly (i.e. remove the ones that are not relevant as per e2e call flows in section 4).

* * * * Start Change * * *.*

### 5.2.2	AMF Services

Table 5.2.2-1: NF Services provided by AMF

| NF service | Description | NF service operations | Consumer | Ref. |
| --- | --- | --- | --- | --- |
| UE location reporting and subscriber state | Provides subscriber state, change in state and the current location (e.g. TAI) of the UE - 3GPP access or non-3GPP access - Registered or Deregistered - IDLE or CONNECTED - TAI, - Cell ID (Subscribe/Notify) | UE Mobility event subscribe/notification | NEF | Section 4.10 FFS |
| Collection of AMF Statistics | Provides the stats to the Requester NF may subscribed to the stats event before. | Mobility stats event Subscribe/notification | NEF | Section 5.2.2.6-1 |
| Communication service | This service supports delivery of messages over N2. This service supports delivery of NAS message for relevant functions (e.g. SMS, Device triggering, SM message delivery) | SMS transport request/response | NEF, SMSF, SMF | Many call flows in section 4 |
|  |  | Device triggering request/response |  |  |
|  |  | SM message delivery |  |  |
|  |  | N2 Session Request/Response |  |  |
| Context transfer | Provides UE Context information | Context Request/response | AMF | AMF relocation |

Editor's note:	The identified NF service(s) in this clause are to be evaluated.

* * * * Next Change * * *.*

### 5.2.2	AMF Services

Editor's note:	The identified NF service(s) in this clause are to be evaluated.

#### 5.2.2.1	The procedure of "Get UE Context" service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| Context Request/response | Context retrieval | UE Temporary ID, Reason | UE MM context (UE permanent ID, the accepted NSSAI(optional), AMF policy information. |

#### 5.2.2.2	UE location reporting and subscriber state

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| UE Mobility event subscribe/notification | Support UE mobility event notification for UE(s) that have changed location and/or changed CM states | UE ID, Filter | UE mobility event |

#### 5.2.2.3	Communication service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| SMS transport request/response | SMS transport to the UE | UE ID, SMS Payload | ACK |
| Device triggering request/response | Device trigger the UE when it is in IDLE mode | UE ID | ACK |
| SM message delivery | Deliver SM message to the UE | UE ID, SM Message payload |  |
| N2 Session Request/Response |  |  |  |

#### 5.2.2.6	The procedure of "Mobility Stats Event Notification" service

| NF service operation | Description | Input parameters | Output parameters |
| --- | --- | --- | --- |
| UE Mobility event subscribe/notification | Provides Mobility stats event to requester NF. | SUPI or External ID, Event filter, Reporting Options | Mobility stats event |

* * * * End Change * * *.*