SA WG2 Meeting #121	S2-173219

15 - 19 May 2017, Hangzhou, China	(revision of S2-17xxxx)

Source:	CATT

Title:	TS 23.501: Corrections to MICO mode

Document for:	Discussion / Approval

Agenda Item:	6.5.7.2

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to make some corrections to MICO mode section.

- Discussion

It is agreed that, when MICO mode is applied to a UE, the AMF shall reject any request for downlink data delivery, but it is not clearer whether the network function which receives the reject can initiate further request,

There are two options to solve this issue:

1. When the AMF determines to indicate MICO mode to a UE, the AMF informs the SMF of the MICO mode being applied to the UE, which prevent the SMF to send further requests for DL data delivery;

2. The AMF rejects the request with an appropriate cause, e.g. Unreachable mode, which prevents any network function receiving the reject message sending further request for DL signalling or data delivery.

This paper proposes to adopt option 2.

Other issues:

As a UE in MICO mode can not be paged if it is in CM-IDLE, not only DL data but also DL signalling can not be delivered to this UE.

Since “The UE in MICO mode is only reachable for mobile terminated data or signalling when the UE is in CM-CONNECTED mode for the PDU sessions that are resumed.”, but if a UE in CM-CONNECTED state has a PDU session that is not resumed, any DL signalling or data delivery request for this PDU session shall also be rejected by the AMF.

2.	Proposal

According to the discussion above, we would like to propose to add the following texts into TS 23.501.

* * * * Start of 1st Change * * * *

5.4.1.3	Mobile Initiated Connection only (MICO) mode

A UE may indicate preference for MICO mode during initial registration or registration update. The AMF, based on local configuration, UE indicated preferences, UE subscription information and network policies, or any combination of them, determines whether MICO mode is allowed for the UE and indicates it to the UE during Registration procedure.

The UE and core network re-initiates or exits the MICO mode at subsequent registration signalling. If MICO mode is not indicated explicitly in Registration, then both the UE and the AMF shall not use the MICO mode.

The AMF assigns a registration area to the UE during the registration procedure. When the AMF indicates MICO mode to a UE, the registration area is not constrained by paging area size. The network, based on local policy, and subscription information, may decide to provide an "all PLMN" registration area to the UE. In that case, re-registration to the same PLMN due to mobility does not apply.

Editor’s Note: whether the AMF does not allocate a periodic registration timer when providing an “all PLMN” registration area or provides a very long periodic registration timer is FFS.

If mobility restrictions are applied to a UE in MICO mode, the AMF needs to allocate an allowed area/non-allowed area to the UE as specified in clause 5.3.4.1.

When the AMF indicates MICO mode to a UE, the AMF considers the UE always unreachable while in CM-IDLE. The AMF rejects any request for downlink signalling or data delivery for an MICO UE in idle mode with an appropriate cause. The AMF also defers downlink transport over NAS for SMS, location services, etc.

The UE in MICO mode is only reachable for mobile terminated data or signalling when the UE is in CM-CONNECTED mode for the PDU sessions that are resumed.  For a PDU session that is not resumed, the AMF may reject the request for mobile terminated data or signalling corresponding to this PDU session with an appropriate cause.

In order to allow for timely delivery of mobile terminated data and/or signalling, when a UE in MICO mode transitions to CM-CONNECTED, the AMF may provide a Pending Data indication to the RAN node. If the RAN node receives this indication, the RAN node may take this information into account when determining user inactivity.

A UE in MICO mode need not listen to paging while in CM-IDLE. A UE in MICO mode may stop any access stratum procedures in CM-IDLE, until the UE initiates CM-IDLE to CM-CONNECTED mode procedures due to one of the following triggers:

-	A change in the UE (e.g. change in configuration) requires an update its registration with the network.

-	Periodic registration timer expires.

-	MO data pending.

-	MO signalling pending (e.g. SM procedure initiated).

Editor's note:	It is FFS whether the AMF indicates to the RAN that the UE is in MICO mode.

If a registration area that is not the "all PLMN" registration area is allocated to a UE in MICO mode, then the UE determines if it is within the registration area or not when it has MO data or MO signalling.

* * * * End of Changes * * * *