SA WG2 Meeting #121	S2-173245

15 - 19 May 2017, Hangzhou, China	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	TS 23.501: Proposal for Control plane protocol stack

Document for:	Discussion / Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes a protocol stack model for 5GS control plane, especially between the UE and the 5GC.

1.	Discussion

This contribution proposes a protocol stack model for 5GS control plane, especially between the UE and the 5GC. There were some related contributions on this issue, and this contributions tries to make a consolidated general solution proposal, based on the agreements in other WGs, e.g. RAN3, CT1.

Detailed discussion on the open issue is depicted in the discussion paper in S2-173244.

Some notable suggestions in this contribution are as follow:

- Structure of the clause

- As suggested in S2-172649, the subclauses under the clause 8.2 Control plane protocol stack are structured as “AN-5GC stacks”, “UE-5GC stacks” and “for N3GPP access”.

- Also for each subclause, general descriptions are introduces as suggested in S2-172649

- Alignment with RAN agreements

- In the last RAN WG3 meeting, several interface and protocol stack were agreed (according to the agreements in RAN3#95bis meeting, which is reflected in RAN3 consolidated text proposal for TS 38.300 in R3-171329).

  - Transport layer between AN and CN nodes: SCTP.

  - Name of the application protocol between RAN and AMF: NG-AP

  - 5G UP encapsulation between An and UPF: GTP-U

- Added reference regarding this agreement (TS 36.300, TS 38.300, TS 38.413 and RFC4960 for SCTP)

- UE-5GC protocol stack

- As suggested in S2-172649, protocol stack between the UE and 5GC (AMF and SMF) is similar.

- Changed the name of the NAS protocol by removing “N1” since N1 is the name of the interface, and it is not necessarily included in the name of the protocol.

- For the name of the protocol, suggested to use “NAS-MM” to be aligned with CT WG1, who are using “5GMM” functionality.

- Proposal 1: There is a single NAS protocol layer between the UE and the AMF. This NAS protocol is responsible for 5GMM functionalities, including transport of non-MM messages between UE and AMF. This layer can be referred as “NAS-MM”

- ENs are added regarding the name of the NAS protocol.

- AN-SMF protocol stack

- Proposal 2: Whether an additional (sub)layers above NG-AP in the AN and N11 in the SMF is needed for AN-SMF interaction transparent to the AMF will be determined in RAN WG3

- In this contribution, the stack is proposed without additional layer, but the following EN is added:

- Editor's note:	Whether an additional (sub)layers above NG-AP in the AN and N11 in the SMF is needed for AN-SMF interaction transparent to the AMF will be determined in RAN WG3.

This paper also proposes to update existing protocol stacks for the untrusted non-3GPP access and user plane protocol stacks according to the RAN agreement on the each protocols to use. (NG-AP, SCTP, GTP-U…)

2.	Proposal

According to the discussion above, we would like to propose to add the following texts in TS 23.501

* * * * Start of 1st Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 4282: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[xa]	IETF RFC 4960: "Stream Control Transmission Protocol".

[xb]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[xc]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[xd]	3GPP TS 38.413: " NG Radio Access Network (NG-RAN); NG Application Protocol".

* * * * Start of 2nd Change * * * *

8	Control and User plane Protocol Stacks

Editor's note:	This is a place holder to include control and user plane protocol stacks if there is a need to capture the same in stage 2 specification (similar content as TS 23.401 clause 5.1 expected). Need for this section is FFS.

8.1	General

Editor's note:	For the name of the interfaces between AN and 5GC, whether to use N2/N3 as defined in SA2 or NG-C/U as defined in RAN WG is FFS.

8.2	Control plane Protocol stacks

8.2.1	General

Editor's note:	This subclause is FFS.

8.2.2		Control Plane protocol stacks between the AN and the 5GC

8.2.2.1	General

The Control Plane interface between the AN and the 5GC has to support

-	The connection of multiple different kinds of AN (e.g. 3GPP RAN, N3IWF for Un-trusted access to 5GC) to the 5CG via an unique Control Plane protocol, which means that a single NG-AP protocol is used for both the 3GPP access and non-3GPP access;

-	The decoupling between AMF and other functions such as SMF that may need to control the services supported by AN(s) (e.g. control of the UP resources in the AN for a PDU session)

8.2.2.2	AN - AMF

Legend:

-	NG Application Protocol (NG-AP): Application Layer Protocol between the NG-RAN node and the AMF. NG-AP is defined in TS 38.413 [xd]

-	Stream Control Transmission Protocol (SCTP): This protocol guarantees delivery of signalling messages between AMF and NG-RAN node (NG-C). SCTP is defined in RFC 4960  [xa].

Figure 8.2.2.2-1: Control Plane between the AN and the AMF

8.2.2.3	AN - SMF

Figure 8.2.2.3-1: Control Plane between the AN and the SMF

Editor's note:	Whether an additional (sub)layers above NG-AP in the AN and N11 in the SMF is needed for AN-SMF interaction transparent to the AMF will be determined in RAN WG3.

Editor's note:	Detailed protocol stack and the exact name of the protocol layer on N11 interface will be determined in stage 3.

8.2.3		Control Plane protocol stacks between the UE and the 5GC

8.2.3.1	General

The NAS supports generic capabilities:

-	NAS procedures that terminate at the AMF.

-	Transfer of NAS messages for different services that do not terminate at the AMF, e.g. SM, SMS, Policy to UE. This includes :

  - Provide a secure NAS connection between the UE and the AMF;

  - Transfer of UL NAS Routing information (e.g. NAS application identifier, PDU session Id) and NAS Service Type allowing the NAS relay in AMF to determine the upper layer NAS service type and the relevant NF instance to handle the NAS message;

  - Transfer of DL NAS Routing information (e.g. NAS application identifier, PDU session Id) and NAS Service Type allowing the UE to determine the upper layer NAS service type and the relevant NAS application to handle the NAS message ;

- There is a Single NAS protocol that applies on both 3GPP and non-3GPP access.

8.2.3.2	UE - AMF

Legend:

-	NAS-MM: The NAS protocol for MM functionality supports registration management functionality, connection management functionality and user plane connection activation and deactivation. It is also responsible of ciphering and integrity protection of NAS signalling. NAS protocol is defined in TS 24.xxx [xx]

-	AN Protocol layer: This set of protocols/layers depends on the AN. In case of 3GPP RAN, the radio protocol between the UE and the AN node (eNodeB or gNodeB) is specified in TS 36.300 [xb] and TS 38.300 [xc].

Figure 8.2.3.2-1: Control Plane between the UE and the AMF

Editor's note:	The exact name of the NAS protocol between UE and AMF (e.g. NAS-AM / NAS-MM / NAS-RM) is FFS.

8.2.3.3	UE – SMF

Legend:

-	NAS-SM: The NAS protocol for 5GSM functionality supports user plane PDU session establishment, modification and release. It is transferred via the AMF, and transparent to the AMF. NAS protocol is defined in TS 24.xxx [xx]

Figure 8.2.3.3-1: Control Plane protocol stack between the UE and the SMF

Editor's note:	Whether to use the same NAS protocol between UE and AMF as 8.2.3.2 (e.g. NAS-AM / NAS-MM / NAS-RM) or to use the other NAS protocol (e.g. N1-NAS transport) is FFS.

8.2.4		Control Plane protocol stacks between the 5GC functions

Editor's note:	Whether the control plane protocol stacks between 5GC functions needs to be specified is FFS.

8.2.5	Control Plane protocol stack for untrusted non 3GPP Access

Legend:

-	NG Application Protocol (NG-AP): Application Layer Protocol between the N3IWF and the AMF.

-	N12 Application Protocol (N12-AP): Application Layer Protocol between the AMF and the AUSF.

-	The N3IWF creates a NAS Attach Request message on behalf of the UE and send this message over N2 to AMF

Figure 8.2.3-1: Control Plane for initial part of attach procedure via N3IWF

Editor's note:	The need for NAS layer between N3IWF and AMF is FFS.

Legend:

-	N2 Application Protocol (N2-AP): Application Layer Protocol between the N3IWF and the AMF.

-	IPsec transport mode and GRE is used to encapsulate NAS payload between UE and N3IWF.

Figure 8.2.3-2: Control Plane for NAS when CP IPsec SA is established

Editor's note:	It is FFS whether GRE encapsulation of NAS is needed or not.

Legend:

-	N2 Application Protocol (N2-AP): Application Layer Protocol between the N3IWF and the AMF.

-	IKEv2 is used to establish the Child SA between UE and N3IWF for each PDU session.

Figure 8.2.3-3: Control Plane for user plane establishment via N3IWF

Editor's note:	The name of N12-AP and the protocol stack for N12 is FFS.

8.3	User Plane Protocol stacks

8.3.1	User Plane Protocol stack for a PDU session

This clause illustrates the protocol stack for the User plane transport related with a PDU session.

Legend:

-	PDU layer: This layer corresponds to the PDU carried between the UE and the DN over the PDU session. When the PDU session Type is IPV6, it corresponds to IPv6 packets ; When the PDU session Type is Ethernet, it corresponds to Ethernet frames ; etc.

-	GPRS Tunnelling Protocol for the user plane (GTP-U): This protocol supports multiplexing traffic of different PDU sessions (possibly corresponding to different PDU session Types) by tunnelling user data over N3 (i.e. between the AN node and the UPF) in the backbone network. GTP shall encapsulate all end user IP packets. It provides encapsulation on a per PDU session level. This layer carries also the marking associated with a QoS Flow defined in clause 5.7.

-	5G Encapsulation: This layer supports multiplexing traffic of different PDU sessions (possibly corresponding to different PDU session Types) over N9 (i.e. between different UPF of the 5GC). It provides encapsulation on a per PDU session level. This layer carries also the marking associated with a QoS Flow defined in clause 5.7.

Figure 8.3.1-1: User Plane Protocol Stack

Editor's note:	The details of encapsulation header should be updated according to the protocol chose/defined in stage 3.

-	AN protocol stack: This set of protocols/layers depends on the AN:

-	When the AN is a 3GPP RAN, these protocols/layers are defined by the 3GPP RAN

Editor's note:	The details of RAN protocol stacks depend on the decision of RAN WG and will be updated later.

-	When the AN is an Untrusted non 3GPP access to 5GC the 5G AN interfaces with the 5GC at a N3IWF defined in clause 4.3.2 and the AN protocol stack is defined in clause 8.3.3.

-	UDP/IP: These are the backbone network protocols.

NOTE 1:	The number of UPF in the data path is not constrained by 3GPP specifications: there may be in the data path of a PDU session 0, 1 or multiple UPF that do not support a PDU session anchor functionality for this PDU session.. For the IP type PDU session, the UPF acting as PDU session anchor is the IP anchor point of the IP address/prefix allocated to the UE.

NOTE 2:	The "non PDU anchor session" UPF depicted in the Figure 8.3.1-1 is optional.

NOTE 3:	The N9 interface may be intra-PLMN or inter PLMN (in case of Home Routed deployment).

In case there is an UL CL (Uplink Classifier) or a Branching Point (both defined in clause 5.6.4) in the data path of a PDU session, the UL CL or Branching Point acts as the non PDU session anchor UPF of Figure 8.3.1-1. In that case there are multiple N9 interfaces branching out of the UL CL / Branching Point each leading to different PDU session anchors.

NOTE 4:	Co-location of the UL CL or Branching Point with a PDU session anchor is a deployment option.

8.3.2	User Plane for untrusted non 3GPP Access

Legend:

-	PDU Layer: As defined in Figure 8.3.1-1

-	GPRS Tunnelling Protocol for the user plane (GTP-U): This protocol supports multiplexing traffic of different PDU sessions (possibly corresponding to different PDU session Types) by tunnelling user data over N3 (i.e. between N3IWF and the UPF) in the backbone network. This tunnel is per PDU session.

-	The N3IWF relays the user data between per PDU session IPsec tunnel over NWu and corresponding N3 tunnel.

Figure 8.3.2-1: User plane via N3IWF

* * * * End of Changes * * * *