# SA WG2 Meeting #S2-121	S2-173938

# 15 - 19 May 2017, Hangzhou, P. R. China

# 3GPP TSG CT WG 1 Meeting 104								C1-172475

Zhangjiajie, P.R of China, 15-19 May 2017

Title:	LS on NAS registration request from 5G UE through non-3GPP access

Response to:	- -

Release:	Rel-15

Work Item:	5GS_Ph1-CT

Source:	CT1

To:	SA2

Cc:	RAN3

Contact person:	Chen-Ho CHIN

chen-hox.chin@intel.com

Tel:  +32 477 413371

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 has discussed the scenario described in TS 23.502, subclause 4.12.2, of UE registration to 5G system through non-3GPP access. In that subclause (and in Figure 4.12.2-1), in step 3b the N3IWF creates a NAS Registration Request message (presumably the ATTACH REQUEST message) after an IKE message exchange between UE and N3IWF where the UE has to provide its 'Identity' and 'Registration params' but not the ATTACH REQUEST.

CT1 would like that the UE does send a NAS registration request message (REGISTRATION REQUEST) and not have the N3IWF create this NAS Registration Request message (in step 3). CT1 feels that in doing it this way, there will be, amongst other things less impacts and less divergence from any 5G NAS that CT1 will have to design out of existing NAS. This will also mean less exceptions have to be built in, all of which should lead to better and more timely completion of CT1's work on the UE NAS to core network protocol.

Thus CT1 kindly request SA2 to revisit the UE's registration procedures though non-3GPP access and re-consider if the UE should "pass" the NAS registration request message (REGISTRATION REQUEST) to the N3IWF function for it to forward to the AMF. Naturally, if the UE's Identity is required for the N3IWF to perform its AMF selection, that (or other parameters) can be provided to the N3IWF separate from the NAS registration request message, so that the N3IWF need not open this message for processing.

# 2	Actions

To SA2

ACTION: 	CT1 kindly request SA2 to re-consider that the registration to the 5G system through non-3GPP access starts with the UE sending a NAS registration request message (REGISTRATION REQUEST).

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 105	21 - 25 August 2017	Krakow (Poland)

TSG CT WG1 Meeting 106	23 - 27 October 2017	TBC, India