SA WG2 Meeting #121	S2-174028

May 15 – 19, 2017, Hangzhou, China	(revision of S2-173226)

Source:	CATT

Title:	23.501 Mobility Pattern FFS removal

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	NextGen / Rel-15

Abstract of the contribution: This contribution proposes to remove two FFS in Section 5.3.4.2.

Discussion:

Discussion A: Appropriate Name for the concept Mobility Pattern.

During the TR phase, the wording “Mobility Pattern” was considered better than “Mobility Level” and hence been chosen for such concept. In TS23.501, the definition of “Mobility Pattern” has been captured, and such concept has been well understood by companies. Till now, we believe that the “Mobility Pattern” is the most appropriate name for such concept, hence related EN should be removed.

Discussion B: Specific parameters for determining Mobility Pattern are left to implementation.

Proposal:

The following changes are proposed to be applied to TS 23.501 v0.4.0.

**************************************Start of 1st change*****************************************

### 5.3.4.2	Mobility Pattern

The Mobility Pattern is a concept that may be used by the 5G system core network to characterise and optimise the UE mobility. The 5G system core network determines and updates Mobility Pattern of the UE based on subscription of the UE, statistics of the UE mobility, network local policy, and the UE assisted information, or any combination of them. The statistics of the UE mobility can be historical or expected UE moving trajectory.

UE mobility pattern can be used by the AMF to optimize mobility support provided to the UE, for example, registration area list allocation.

**************************************End of the changes*****************************************