SA WG2 Meeting #122bis	S2-175389

21 - 25 August 2017, Sofia Antipolis, France	(revision based on S2-174807)

Source:	Ericsson

Title:	UE reachability improvement for RRC Inactive

Document for:	Approval

Agenda Item:	6.5.7.5

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Proposes to add logic in AMF for RRC inactive to CM-IDLE transition handling in case of RAN indicating UE unreachable

# Discussion

In SA2 #122, CM-CONNECTED with RRC inactive state and the UE reachability in this state was introduced. 
Below text is extracted from TS 23.501 v 1.2.0 (however, a previous  ED in the text was removed by S2-175254 without addressing the real issue behind the ED – Editor's note: Further actions, e.g. when to start the implicit deregistration timer, in AMF in CM-IDLE state after unreachable notification is received, are FFS.):

### 5.4.2	UE reachability in CM-CONNECTED

For a UE in CM-CONNECTED state:

-	the AMF knows the UE location on a serving (R)AN node granularity.

-	the NG-RAN notifies the AMF when UE becomes unreachable from RAN point of view.

UE RAN reachability management is used by RAN for UEs in RRC Inactive state, see TS 38.300 [27]. The location of a UE in RRC Inactive state is known by the RAN on a RAN Paging Area granularity. A UE in RRC Inactive state is paged in cells of the RAN Paging Area that is assigned to the UEs . The RAN Paging Area can be a subset of cells configured in UE's Registration Area or all cells configured in the UE's Registration Area. UE in RRC Inactive state performs Paging Area Update when entering a cell that is not part of the RAN Paging Area that is assigned to the UE.

Editor's note:	Whether the RAN Paging Area term needs to be aligned with the one used in RAN specifications is FFS.

At transition into RRC Inactive state RAN configures the UE with a periodic PAU timer value and the timer is restarted in the UE and in the RAN with this initial timer value. After the expiry of the periodic PAU timer in the UE, the UE in RRC Inactive state performs periodic PAU, as specified in TS 38.300 [27].

Editor's note:	The value of the periodic PAU timer will be defined together with RAN WGs.

To aid the UE reachability management in the AMF, upon the expiry of the periodic PAU timer in RAN, the RAN releases the N2 signalling connection for that UE indicating to the AMF that the UE is unreachable due to expiry of the periodic PAU time. RAN may optionally delay the release of the N2 signalling connection. If the RAN releases the N2 signalling connection, the AMF shall enter CM-IDLE state see clause x.y.z.

However, as mentioned, there are still some open issues, one of them is the implicit deregistration timer handling in AMF when N2 signalling connection is released and UE changed state from CM-CONNECTED with RRC inactive to CM-IDLE. This was captured in a previous version in the following EN:

Editor's note:	Further actions, e.g. when to start the implicit deregistration timer, in AMF in CM-IDLE state after unreachable notification is received, are FFS.

Without any synchronization between RAN and CN the UE may be regarded by the AMF as reachable either too long or too short. Too long may be the case if AMF just starts the reachable timer regardless of the length of the RAN periodic PAU timer. Too short may be the case if AMF instead directly starts the implicit deregistration timer assuming that the RAN periodic PAU timer is in parity with the reachable timer. Without knowledge in AMF about elapsed time of the periodic PAU timer in RAN, AMF will not be able to make the correct adjustment of either its reachable timer or of its implicit deregistration timer. The total time that AMF holding the UE in RM-Registered state after RRC-INACTIVE can be one of the following two options:

- The sum of the time elapsed in RAN, mobile reachable timer and implicit detach timer, which probably will be too long in certain cases,

- the sum of the time elapse in RAN and implicit timer, which probably will be too short and MT service may be impacted unnecessary.

See below figure for detailed illustration.

It is therefore proposed that RAN at UE transition to CM-IDLE, through N2 connection release indicating of UE unreachable, provides AMF with the time since last in contact with the UE (i.e. when UE enters RRC inactive state). When AMF receives the time from RAN, mobile reachable timer and implicit detach timer can be adjusted accordingly. Hence the total time for AMF to hold the UE in RM-registered state after RRC-INACTIVE is always the sum of mobile reachable timer and implicit detach timer and therefore consistent with the use case where RRC Inactive state is not used, i.e. a consistent behaviour is achieved.

The figure below illustrates the case when the time elapsed in RAN is less than the sum of mobility reachable timer and implicit detach timer in AMF.

For the case when the time elapsed in RAN is greater than the sum of mobility reachable timer and implicit detach timer in AMF, AMF can simply put the UE in deregistered state directly. But it’s normally an un-usual case.

# Proposal

***** 1st Change *****

### 5.4.2	UE reachability in CM-CONNECTED

For a UE in CM-CONNECTED state:

-	the AMF knows the UE location on a serving (R)AN node granularity.

-	the NG-RAN notifies the AMF when UE becomes unreachable from RAN point of view.

UE RAN reachability management is used by RAN for UEs in RRC Inactive state, see TS 38.300 [27]. The location of a UE in RRC Inactive state is known by the RAN on a RAN Paging Area granularity. A UE in RRC Inactive state is paged in cells of the RAN Paging Area that is assigned to the UEs. The RAN Paging Area can be a subset of cells configured in UE's Registration Area or all cells configured in the UE's Registration Area. UE in RRC Inactive state performs Paging Area Update when entering a cell that is not part of the RAN Paging Area that is assigned to the UE.

Editor's note:	Whether the RAN Paging Area term needs to be aligned with the one used in RAN specifications is FFS.

At transition into RRC Inactive state RAN configures the UE with a periodic PAU timer value and the timer is restarted in the UE and in the RAN with this initial timer value. After the expiry of the periodic PAU timer in the UE, the UE in RRC Inactive state performs periodic PAU, as specified in TS 38.300 [27].

Editor's note:	The value of the periodic PAU timer will be defined together with RAN WGs.

To aid the UE reachability management in the AMF, upon the expiry of the periodic PAU timer in RAN, the RAN releases the N2 signalling connection for that UE indicating to the AMF that the UE is unreachable due to expiry of the periodic PAU time. RAN may optionally delay the release of the N2 signalling connection. The time elapsed since RAN’s last contact with the UE is provided to AMF. If the RAN releases the N2 signalling connection, the AMF shall take the elapsed time from RAN into consideration when the UE enters CM-IDLE state see clause 5.4.1.1.

***** End of 1st Change *****

***** 2nd Change *****

### 5.4.1	UE reachability in CM-IDLE

#### 5.4.1.1	General

Reachability management is responsible for detecting whether the UE is reachable and providing UE location (i.e. access node) for the network to reach the UE. This is done by paging UE and UE location tracking. The UE location tracking includes both UE registration area tracking (i.e., UE registration area update) and UE reachability tracking ((i.e. UE periodic registration area update)). Such functionalities can be either located at 5GC (in case of CM-IDLE state) or NG-RAN (in case of CM-CONNECTED state).

The UE and the AMF negotiate UE reachability characteristics in CM-IDLE state during registration and registration update procedures.

Two UE reachability categories are negotiated between UE and AMF for CM-IDLE state:

1.	UE reachability allowing Mobile Terminated data while the UE is CM-IDLE mode.

-	The UE location is known by the network on a Tracking Area List granularity

-	Paging procedures apply to this category.

-	Mobile originating and mobile terminated data apply in this category for both CM-CONNECTED and CM-IDLE mode.

2.	Mobile Initiated Connection Only (MICO) mode:

-	Mobile originated data applies in this category for both CM-CONNECTED and CM-IDLE mode.

-	Mobile terminated data is only supported when the UE is in CM-CONNECTED mode.

Whenever a UE enters CM-IDLE state, it starts a periodic registration timer according to the periodic registration timer value received from the AMF during a registration procedure.

The AMF allocates a periodic registration timer value to the UE based on local policies, subscription information and information provided by the UE. After the expiry of the periodic registration timer, the UE shall perform a periodic registration. If the UE moves out of network coverage when its periodic registration timer expires, the UE shall perform a registration update when it next returns to the coverage.

The AMF runs a Mobile Reachable timer for the UE. The timer is started with a value longer than the UE's periodic registration timer when the CM state for the UE changes to CM-IDLE. Alternatively, if the AMF receives an elapsed time from RAN when RAN releases the N2 signalling connection indicating UE unreachable and the CM state for the UE changes to CM-IDLE, the AMF shall deduce a Mobile Reachable timer value based on the UE's periodic registration timer value and the elapsed time received from RAN. If the elapsed time received from RAN is significantly longer than a normal Mobile Reachable timer, the AMF should also use a reduced value for the Implicit De-registration timer. The AMF stops the Mobile Reachable timer, if the AMF moves to CM-CONNECTED state. If the Mobile Reachable timer expires, the AMF determines that the UE is not reachable.

However, the AMF does not know for how long the UE remains not reachable, thus the AMF shall not immediately de-register the UE. Instead, after the expiry of the Mobile Reachable timer, the AMF should clear the PPF flag and shall start an Implicit De-registration timer, with a relatively large value. The AMF shall stop the Implicit Deregistration timer and set the PPF flag if the AMF moves to CM-CONNECTED state.

NOTE:	the AMF considers the UE in CM-IDLE state always unreachable if the UE is in MICO mode (refer to clause 5.4.1.3).

If the PPF is not set, the AMF does not page the UE and shall reject any request for delivering DL signalling or data to this UE.

If the Implicit De-registration timer expires before the UE contacts the network, the AMF implicitly de-register the UE.

As part of deregistration for a particular access (3GPP or non-3GPP), the AMF shall request the UE's related SMF to release the PDU sessions established on that access.

***** End of 2nd Change *****