SA WG2 Meeting #122Bis	S2-175653

21- 25 Aug 2017, Sophia Antipolis, France	(revision of S2-17xxxx)

Source:	Huawei, HiSilicon

Title:	TS 23.501: Registration procedure initiated by UE after NSSAI changes

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_ph1/Rel-15

Abstract of the contribution: This contribution proposes to update the description of Registration procedure initiated by UE after NSSAI changes in TS 23.501.

# 1.     Discussion

In current clause 5.15.5.2.3, for a registered UE, the AMF relocation due to changing of supported network slices is mentioned as following:

For a UE that is already registered, the system shall support a redirection initiated by the network of a UE from its serving AMF to a target AMF due to Network Slice(s) considerations. Operator policy determines whether redirection between AMFs is allowed.

For the redirection between AMFs, there were corresponding descriptions in clause 5.15.5.2.2:

The network, based on local policies, subscription changes and/or UE mobility, may change the set of permitted Network Slice(s) to which the UE is registered. The network may perform such change during a Registration procedure or trigger a notification towards the UE of the change of the supported Network Slices using an RM procedure (which may trigger a Registration procedure). The new Allowed NSSAI is determined as described in clause 5.15.5.2.1, and the AMF provides the UE with the new Allowed NSSAI and Tracking Area list.

However, the above description does not consider the case that “Operator policy does not allow the redirection between AMFs”. This may happen e.g. the subscription changes and the UE need to be served by another set of network slices which are isolated from current network slices. In this case, the serving AMF cannot discover and connect to any target AMF that can serve the new NSSAI. Hence the serving AMF have to send the new allowed NSSAI to the UE and trigger the UE to perform another initial registration.

In addition, to avoid the RAN node routing Registration request to current AMF based on current 5G-S-TMSI, the serving AMF shall indicate to the UE that current 5G-S-TMSI becomes invalid.

Proposal: If the redirection between AMFs is not allowed by the operator policy, i.e. the serving AMF cannot discover or connect to any target AMF that can serve the new NSSAI, the serving AMF shall provide the new allowed NSSAI to the UE and also indicates UE that current 5G-S-TMSI is invalid. This triggers UE initiated Registration procedure using the new NSSAI and UE SUPI, so that a new AMF can be selected to serve the UE as described in clause 5.15.5.2.

# 2	Proposal

It is proposed to update the following text into clause 5.15.5 for TS 23.501

***** START OF CHANGE *****

##### 5.15.5.2.3	AMF Relocation due to Network Slice(s) Support

During a Registration procedure in a PLMN, in case the network decides that the UE should be served by a different AMF based on Network Slice(s) aspects, then the AMF that first received the Registration Request shall redirect the Registration request to another AMF via the RAN or via direct signalling between the initial AMF and the target AMF. The redirection message sent by the AMF via the RAN shall include information for selection of a new AMF to serve the UE.

For a UE that is already registered, the system shall support a redirection initiated by the network of a UE from its serving AMF to a target AMF due to Network Slice(s) considerations. Operator policy determines whether redirection between AMFs is allowed.

If the redirection between AMFs is not allowed by the operator policy, i.e. the serving AMF cannot discover or connect to any target AMF that can serve the new NSSAI, the serving AMF shall provide the new allowed NSSAI to the UE and also indicates UE that current 5G-S-TMSI is invalid. This triggers UE initiated Registration procedure using the new NSSAI and UE SUPI, so that a new AMF can be selected to serve the UE as described in clause 5.15.5.2.

* * * * End of Changes * * * *