SA WG2 Meeting #122bis	S2-175737

21 - 25 August 2017, Sofia-Antipolis, France	(revision of S2-17xxxx)

Source:	NTT DOCOMO

Title:	Removing EN about optimization for no-N26 SR UE mobility

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Removes editor's note that was introduced to investigate optimization for no-N26 SR UE mobility.

# Introduction

For no-N26 IWK with SR UEs, the following FFS still remains:

Editor's note:	Additional optimization to enable UE to skip the TAU request/reject when UE is in connected mode in 5GC when mobility to EPC occurs is FFS.

Since this is a network that does not support N-26 interface, seamless mobility is not possible for SR UEs. There will be interruption. The agreed procedure is for the UE to perform TAU in LTE, get a TAU reject with new cause and then do Attach following with PDU session handover. The only steps that can possibly be removed is the TAU procedure. TAU reject is a very short procedure. No authentication, no HSS interaction needed. There is no benefit in skipping the TAU procedure. Providing LTE cell information in RRC redirection is possible even with TAU reject.

Hence, it is proposed to remove the FFS and not add any optimization, which is of dubious value.

# 2.	Updates to 23.501

### **************  START CHANGES **********************

#### 5.17.2.3	Interworking Procedures without N26 interface

#### ...

##### 5.17.2.3.2	Mobility for UEs in single-registration mode

When the UE supports single-registration mode and network supports interworking procedure without N26 interface:

-	For mobility from 5GC to EPC, the UE performs TAU procedure with 4G-GUTI mapped from 5G-GUTI. The MME determines that old node is an AMF, rejects the TAU with a "Handover PDN Connection Setup Support" indication to the UE. Based on this indication, the UE may perform Attach in EPC with "handover" indication in PDN Connection Request message (TS 23.401 [26], clause 5.3.2.1) and it subsequently moves all its other PDU session using the UE initiated PDN connection establishment procedure with "handover" flag (TS 23.401 [26] clause 5.10.2).

Editor's note:	It is FFS if it is optional for UE in single-registration mode to perform the procedure mentioned in bullet above and what will be the impacts.

NOTE:	The first PDN connection may be established during with the E-UTRAN Initial Attach procedure (see TS 23.401 [26]).

-	For mobility from EPC to 5GC, the UE performs Registration of type "mobility registration update" in 5GC with 5G-GUTI mapped from 4G-GUTI. The AMF determines that old node is an MME, but proceeds as if the Registration is of type "initial registration". The Registration Accept includes "Handover PDU Session Setup Support" indication to the UE. Based on this indication, the UE may subsequently move all its PDN connections from EPC using the UE initiated PDU session establishment procedure with "Existing PDU Sessions" flag (TS 23.502 [3], clause 4.3.2.2.1).

### **************  END CHANGES    **********************