SA WG2 Meeting #122bis	S2- 175788

21 - 25 August 2017, Sophia Antipolis, France	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	TS 23.502: Clarification on handover between 3GPP and N3GPP

Document for:	Approval

Agenda Item:	6.5.10 Non-3GPP access specific functionality and flows

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: It is proposed to clarify that PDU Session information is stored in the UDM per PDU Session ID.

Discussion

5GS allows to establish multiple PDU sessions to the same DNN. Also there is no restriction that the same SMF shall be selected when there are multiple PDU session to the same DNN. So there may exist multiple SMFs to the same DNN when multiple PDU sessions have been established for the same DNN.

According to the current description related to handover between 3GPP access and non-3GPP access in the PDU session establishment procedure described in clause 4.3.2.2.1: step 2 of TS 23.502, the AMF checks whether it stores the PDU Session ID to select SMF for the PDU session based on context in the AMF. If the AMF does not recognize the PDU Session ID, it checks SMF ID corresponding to the DNN in the UE context from the UDM. In this case, when multiple SMFs exist for the same DNN as explained above, the AMF cannot decide which SMF is correct one for the PDU session because, PDU Session ID is not provided when the SMF registers its information to the UDM (see step 19).

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on that Request Type indicates "initial request" and that the PDU Session ID that is not used for any existing PDU Session(s) of the UE. If the NAS message does not contain an S-NSSAI, the AMF may determine a default S-NSSAI for the requested PDU session according to the UE subscription. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. The AMF stores an association of the PDU session ID and the SMF ID.

The case where the Request Type indicates "Existing PDU Session", and either the AMF does not recognize the PDU Session ID or the subscription context from UDM does not contain an SMF ID corresponding to the DNN constitutes an error case.

Also the SMF registers its information only when the SMF identity is not stored in the UDM. However, there may exist multiple PDU sessions controlled by the same SMF, the SMF shall register its session information whenever new PDU session is established.

So it is proposed to clarify that the SMF registers PDU session information including PDU Session ID whenever new PDU session is established.

Proposal

Based on the above discussion the following changes to TS 23.502 are proposed.

* * * * Start of 1st Change * * * *

4.3.2.2.1	Non-roaming and Roaming with Local Breakout

This clause specifies PDU Session establishment in the non-roaming and roaming with local breakout cases. The procedure is used to establish a new PDU session as well as to hand over an existing PDU Session between 3GPP access and non-3GPP access.

In case of roaming, the AMF determines if a PDU session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI, DNN, PDU Session ID, Request type, N1 SM information).

In order to establish a new PDU session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM information. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The Request Type indicates "Initial request" if the PDU Session Establishment is a request to establish a new PDU Session and indicates "Existing PDU Session" if the request refers to an existing PDU Session between 3GPP access and non-3GPP access.

The NAS message sent by the UE is encapsulated by the AN in a N2 message towards the AMF that should include User location information and Access Technology Type Information.

The N1 SM information may contain SM PDU DN Request Container containing information for the PDU session authorization by the external DN.

The AMF receives from the AN the NAS SM message (built in step 1) together with User Location Information (e.g. Cell Id in case of the RAN).

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on that Request Type indicates "initial request" and that the PDU Session ID that is not used for any existing PDU Session(s) of the UE. If the NAS message does not contain an S-NSSAI, the AMF may determine a default S-NSSAI for the requested PDU session according to the UE subscription. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. The AMF stores an association of the PDU session ID and the SMF ID.

The case where the Request Type indicates "Existing PDU Session", and either the AMF does not recognize the PDU Session ID or the subscription context from UDM does not contain an the PDU Session ID corresponding to the DNN constitutes an error case.

3.	From AMF to SMF: Nsmf_PDUSession_CreateSMRequest (Subscriber Permanent ID, DNN, S-NSSAI, PDU Session ID, AMF ID, N1 SM information (PDU Session Establishment Request), User location information, Access Technology Type, PEI).

The AMF ID uniquely identifies the AMF serving the UE. The AMF forwards the PDU Session ID together with the N1 SM information containing the PDU Session Establishment Request received from the UE. The PEI (Permanent Equipment Identifier) is defined in TS 23.501 [2] clause 5.9.3.

4.	SMF invokes: Nudm_SubscriberData_GetRequest  (Subscriber Permanent ID, DNN).

If the Request Type in step in step 3 indicates "Existing PDU Session" the SMF determines that the request is due to handover between 3GPP access and non-3GPP access. The SMF identifies the existing PDU Session based on the PDU Session ID.

If the SMF has not yet retrieved the SM-related subscription data for the UE related with the DNN, the SMF requests this subscription data.	UDM responds to SMF: with Nudm_SubscriberData_GetResponse.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), Default QoS profile.

Editor's note:	Further detailing of SM-related subscription data is FFS.

The SMF checks whether the UE request is compliant with the user subscription and with local policies. If that is not the case the SMF rejects the UE request via NAS SM signalling by responding to the AMF with Nsmf_PDUSession_CreateSMResponse including a relevant SM rejection cause. The SMF indicates to the AMF that the PDU session ID is to be considered as released and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs perform secondary authorization/authentication during the establishment of the PDU session by a DN-AAA server as described in clause 5.6.6 of TS 23.501 [2], the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU session establishment authentication/authorization as described in clause 4.3.2.3.

If the PDU session establishment authentication/authorization fails, the SMF terminates the PDU session establishment procedure and indicates a rejection to the UE.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection.

6b.	The SMF may invoke Npcf_PolicyControl_PolicyCreateRequest including PDU Session ID to establish a PDU CAN session and get the default PCC Rules for the PDU Session. If the Request Type in step 3 indicates "Existing PDU Session", the PCF initiates a PDU-CAN Session Modification instead, by invoking the Npcf_PolicyControl_PolicyUpdateNotification service.

Editor's note:	The purpose of step 5 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 5 can be skipped.

7.	If the Request Type in step 3 indicates "Initial request", the SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1. For Unstructured PDU Type the SMF may allocate an IPv6 prefix for the PDU session and N6 point-to-point tunnelling (based on UDP/IPv6) as described in TS 23.501 [2] clause 5.6.9.4.

8a.	SMF invokes the Npcf_PolicyControl_PolicyCreateRequest: If dynamic PCC is deployed and the PDU-CAN Session Establishment has not yet been done, the SMF invokes PCF service to establish PDU-CAN session to get the default PCC Rules for the PDU Session.

8b. SMF provides EventExposure_Notify service operation: Otherwise, if Request Type is "initial request" and dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF notifies the PCF with the allocated UE IP address/prefix.

9.	If Request Type indicates "initial request" and step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If CN Tunnel Info is allocated by the UPF, the CN Tunnel Info is provided to SMF in this step.

10.	SMF to AMF: Nsmf_PDUSession_CreateSM Response (Cause, N2 SM information (PDU Session ID, QoS Profile(s), CN Tunnel Info, S-NSSAI), N1 SM information (PDU Session Establishment Accept (Authorized QoS Rule, SSC mode, S-NSSAI, allocated IPv4 address))).

Editor's note:	It is FFS how the AMF and SMF associate the N11 transactions with a corresponding UE context.

The N2 SM information carries information that the AMF shall forward to the (R)AN which includes:

-	The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU session.

-	The QoS Profile provides the (R)AN with the mapping between QoS parameters and QoS Flow Identifiers. Multiple QoS profiles can be provided to the (R)AN. This is further described in TS 23.501 [2] clause 5.7.

-	The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU session for the UE.

S-NSSAI is corresponding to the PDU session.

The N1 SM information contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple Authorized QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM information and in the N2 SM information.

The SM Response further contains PDU Session ID and information allowing the AMF to know which the target UE as well to determine which access towards the UE to use.

NOTE 1:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to determine which access towards the UE to use.

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, NAS message (PDU Session ID, PDU Session Establishment Accept)).

The AMF sends the NAS message containing PDU Session ID and PDU Session Establishment Accept targeted to the UE and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the Authorized QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN N3 tunnel information for the PDU Session. In case of Dual Connectivity, the Master RAN node may assign some (zero or more) QFIs to be setup to a Master RAN node and others to Secondary RAN node. The RAN tunnel information includes a tunnel endpoint for each involved RAN node, and the QFIs assigned to each tunnel endpoint. A QFI can be assigned to either the Master RAN node or the Secondary RAN node and not to both.

(R)AN forwards the NAS message (PDU Session ID, N1 SM information (PDU Session Establishment Accept)) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Response (PDU Session ID, Cause, N2 SM information (PDU Session ID, (R)AN Tunnel Info, List of accepted/rejected QoS profile(s))).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU session.

14.	AMF to SMF: Nsmf_PDUSession_ UpdateSMContext Request (N2 SM information).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

15a.	If the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and CN Tunnel Info. The CN Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

NOTE 2:	If the PDU Session Establishment Request was due to mobility between 3GPP and non-3GPP access, the downlink data path is switched towards the target access in this step.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

16.	SMF to AMF: Nsmf_PDUSession_ UpdateSMContext Response (Cause).

After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

18.	If the PDU Session Establishment request is due to a handover between 3GPP access and non-3GPP access, i.e. the Request type is set to "existing PDU Session" the SMF executes the steps X-Y specified in clause 4.3.4 in order to release the user-plane over the source access (3GPP or non-3GPP access).

Editor's note:	The exact reference to steps of network initiated PDU session release procedure will be defined when the procedure is available.

19.	If the PDU Session ID is not included in step 4b by the UDM in the UE context, the SMF invokes the Nudm_UEContextManagement_Registration service operation including the SMF address, the PDU Session ID and the DNN. The UDM stores the SMF identity, address, the PDU Session ID and the associated DNN.

If during the procedure the PDU session establishment is not successful, the SMF informs the AMF. Once the AMF associates an SMF with a PDU session ID, the SMF is automatically subscribed to notifications of incoming N1 signalling associated with this PDU session ID. These notifications automatically provide any User Location information and Access Type that the AMF has received from the (R)AN in association with N1 signalling.

* * * * End of Changes * * * *