SA WG2 Meeting #122bis	S2-175794

21 - 25 Aug 2017, Sophia Antipolis, France	(revision of S2-17xxxx)

Source:	ETRI

Title:	TS 23.502: Updating UE context release procedure for Non-3GPP

Document for:	Approval

Agenda Item:	6.5.10

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes to update UE context release in the AN procedure for non-3GPP access in TS 23.502.

Introduction

It is proposed to update the section 4.2.6 UE context release in the AN procedure for untrusted non-3GPP access in TS 23.502 because only AN link failure has been considered as the cause of initiating UE context release procedure in non-3GPP access.

Proposal

It is proposed to update the following texts into TS 23.502.

* * * Start of changes* * * *

4.2.6	UE context release in the AN

This procedure is used to release the logical N2-AP signalling connection and the associated N3 User Plane connections.

When the N2-AP signalling connection is lost due to (R)AN or AMF failure, the UE context release in the AN procedure is performed locally by the AMF or the (R)AN as described in the procedure flow below without using or relying on any of the signalling shown between (R)AN and AMF. The UE context release in the AN causes all PDU sessions of the UE to be deactivated.

The initiation of UE context release in the AN procedure may be due to:

-	(R)AN-initiated with cause e.g. O&M Intervention, Unspecified Failure, AN (e.g. Radio) Link Failure, User Inactivity, Release due to UE generated signalling connection release, etc.; or

-	AMF-initiated with cause e.g. Unspecified Failure, etc.

Both (R)AN-initiated and AMF-initiated UE context release in the AN procedures are shown in Figure 4.2.6-1. In non-3GPP access case, (R)AN-initiated UE context release procedure is considered due to AN Link Failure.

Editor's note: It is FFS whether AMF-initiated UE context release procedure is considered for non-3GPP access.

Figure 4.2.6-1: UE context release in the AN Procedure

1.	If there is some confirmed AN conditions (e.g. Radio Link Failure) or for other (R)AN internal reason, the (R)AN may decide to initiate the UE context release in the AN. In this case, the (R)AN sends a N2 UE Context Release Request (Cause) message to the AMF. Cause indicates the reason for the release (e.g. AN Link Failure, O&M intervention, unspecified failure, etc.).

The decision to initiate a UE context release in the AN Procedure may also be triggered by an internal AMF event

2.	[Conditional] For each of the PDU sessions of the UE whose UP is active, the AMF requires the SMF to deactivate the PDU session: the AMF sends a N11 PDU session deactivation Request (PDU session ID, Release Cause).  The AMF also stores the deactivated state of the PDU session

Editor's note: The  handling of the GBR QoS flows upon specific Release Causes will be determined as part of QoS discussions .

Editor's note: This step could also be Namf_ UE Mobility Event Notification (SMF): The AMF sends (N11 message) UE Mobility Event Notification, which indicates the N2 Release event, to the SMF(s) who serves active PDU session(s) of the UE as defined in clause 5.2.2.2. The message includes SUPI, the RAN information, and cause code.

3a	[Conditional] SMF to UPF: N4 Session Modification Request (AN tunnel info to be removed, Buffering on/off).

The SMF initiates a N4 Session Modification procedure indicating the need to remove AN Tunnel Info.  Buffering on/off indicates whether the UPF shall buffer incoming DL PDU or not.

See clause 4.4 for more details.

3b.	[Conditional] UPF to SMF: N4 Session Modification Response acknowledging the SMF request.

See clause 4.4 for more details.

4.	[Conditional] SMF to AMF: N11 PDU session deactivation Response.

Editor's note: This step could also be Nsmf_ PDU Session Event Report Notification (AMF): The SMF sends (N11 message) PDU Session Event Report Notification as defined in clause 5.2.8.1. The notification includes a UE Identifier (SUPI), PDU Session ID(s), and event trigger (deletion of (R)AN tunnel information).

5.	After the AMF collects all the PDU Session Event Report Notification in step 4 from the SMF(s) that were notified in Step 2.The AMF sends to the (R)AN a N2 UE Context Release Request. In case the AN is a RAN this step is described in detail in RAN specifications. In case the AN is a N3IWF this step is described in clause 4.12

Editor's note: It is FFs whether AMF collects all the PDU Session Event Report Notification or can issue a N2 UE Context Release Request immediately

6.	If the AN connection (e.g. RRC connection or NWu connection) with the UE is not already released (step 1), the (R)AN requests the UE to release the AN connection.. Upon receiving AN connection release confirmation from the UE, the (R)AN deletes the UE's context.

7.	The (R)AN confirms the N2 Release by returning an N2 UE Context Release Complete () message to the AMF. With this, the signalling connection between the AMF and the (R)AN for that UE is released.

Editor's note: During this procedure the AN may provide location information to the AMF. Details on the location information (e.g. Cell ID, TAI) provided by the AN are FFS.

* * * End of Changes * * *