SA WG2 Meeting #122bis	S2-175877

August 21 – 25, 2017, Sophia Antipolis, France	(was S2-17----)

Source:	CATT, China Mobile, CATR, China Unicom, China Telecom, Huawei, ZTE, OPPO, vivo

Title:	Discussion on EPS bearer ID allocation

Document for:	Discussion / Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to discuss the solutions for EPS bearer ID allocation and agree the way forway.

- Discussion

The Pros and Cons of UE based solution and AMF based solution has been analyzed by S2-174566. The analysis shows that:

The main disadvantage of UE based solution is that it cannot handle EBI exhaustion; and

The main disadvantage of AMF based solution is that it introduces additional AMF-SMF interaction.

Supporters of UE base solution think EBI exhaustion is a rare case especially when QoS mapping is only applied to SSC mode 1 PDU session, but others who support AMF based solution believe that, as long as the case of EBI exhaustion happens, it shall be carefully handled to promise high priority services not to be interrupted. Therefore, the final solution has not been decided.

As the argument on EBI exhaustion cannot convince SA2 to make a decision, this paper would like to further investigate UE based solution and AMF based solution, and then try to find a way forward.

UE based solution

There are two alternatives for UE based solution, one is from S2-171962, and another is from S2-173339.

For the UE based solution introduced in S2-171962, the ways to allocate EBI during the PDU session establishment procedure and the PDU session modification procedure are different. The UE has to allocate EBI before receiving mapped QoS parameters during the PDU session establishment procedure, but allocate EBI after receiving mapped QoS parameters during the PDU session modification procedure. This inconsistent operation makes the UE implementation more complex.

The UE based solution introduced in S2-173339 has an impact on current PDU session establishment for home routed roaming case. The solution suggests that the UE allocates EBI for QoS flow after receiving the EPS QoS parameters at the QoS flow setup, and then indicates the EBI to the SMF. However, in current PDU session establishment procedure for home routed roaming case, the V-SMF will not send any message to the H-SMF after the radio bearer(s) is established, thus a new message is also required to deliver the EBI to the H-SMF comparing with AMF based solution.

AMF based solution

There are also two alternatives for AMF based solution as introduced in S2-173322 or S2-174196:

Alternative 1: the SMF sends an SM request containing an NAS request to the AMF, if the SM request contains QFIs, the AMF allocates EBI, and send the NAS request and the binding of EBI and QFI to the UE. After receiving ACK or response message, the AMF includes the binding of EBI and QFI in the SM message sent to the SMF. This solution requires the AMF to build messages to send the binding of EBI and QFI to the UE, which complicates AMF behavior and disobeys the principle of MM/SM split.

Alternative 2: After determining the mapped EPS QoS parameters, the SMF requests AMF to allocate EBI(s). The AMF doesn’t need to know the binding between EBI and QFI, it just allocates EBI(s) based on ARP and returns the allocated EBI(s). The SMF assigns the EBI(s) to QoS flow(s) with mapped EPS QoS parameters, and includes the EPS QoS parameters in an SM NAS message.

Based on the above analysis, it is found that the AMF based solution alternative 2 has no impact on UE and less impact on AMF, we hence propose to adopt this solution as the way forward.

2.	Proposal

It is propose to agree the AMF based solution introduced in S2-175483.

Reference:

[1] S2-174566, NTT DOCOMO, Intel, Comparison of solutions for EBI allocation

[2] S2-171962, Intel, 23.502: QoS mapping for 5GC-EPC interworking

[3] S2-174196, Ericsson, 23.502: EPS bearer ID allocation in 5GS