SA WG2 Meeting #122Bis	S2-175905

21 - 25 August 2017, Sophia Antipolis, France

Source:	Motorola Mobility, Lenovo

Title:	PDU Session release during change of set of Network Slice(s)

Document for:	Discussion and Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution proposes to resolve the one Editor’s Notes and leave temporary the other in clause 5.15.5.2.2.

# Introduction

Observation 1

During the SA2#122 meeting the following new Editor’s Note was introduced in TS 23.501 clause 5.15.5.2.2 “Modification of the Set of Network Slice(s) for a UE”:

Editor's note:	The conditions under which the PDU Session(s) to the no longer available Network Slice instance are released via explicit signalling or released implicitly are FFS.

According to this Editor’s Note further study is needed for the scenarios of changing of the sets of network slices and a network slice instance becomes no longer available. The following observations are conducted:

- If a network slice instance becomes unavailable in a predictable way, e. g. subscription change pushed from the UDM to the AMF or network operator changes the network configuration, then PDU Session can be release in graceful way using N1 SM Information signalling exchange, i.e. the SMF can initiate PDU Session release command towards the UE.

- There are scenarios where the change of network slice instance occurs abruptly and there is no possibility to exchange N1 SM signalling. For example, if the UE in CM-IDLE state moves to a new registration area and the new selected AMF is not a part of the network slice instances used in the old registration area. In this case, the new AMF may not be able to setup N11 association with the SMF(s) from the no longer available network slice instance (i.e. no interact between AMF and the SMFs is possible). Thus, there are no means to exchange N1 SM signalling between the UE and the SMF(s).

In case a), the explicit PDU Session Release procedure can be performed as described in TS 23.502 [3], clause 4.3.4.

In case b), as there are no means to exchange N1 SM signalling between the UE and the SMF(s), the release of the PDU Session context in the UE and in the network is performed in implicitly. However, one issue is how are the SMF(s) from the no longer available network slice instance informed about the need to delete the US’s SM context. It is proposed that the (new) AMF indicates to the old AMF (e.g. during the UE context transfer procedure) which S-NSSAI or which PDU Session(s) cannot be longer served. The old AMF indicates to the corresponding SMF(s) that autonomous release of the UE’s SM context is needed.

Observation 2

There is one more Editor’s Note in clause 5.15.5.2.2.:

Editor's note:	The condition under which the UE is able to request the change of the Network Slices, and what it is able to request, are FFS.

It is proposed to leave this Editor’s Note unresolved until there is an agreement about the issues on (1) isolated network slices, (2) co-existing network slices and (3) default network slices. The agreements on the latter 3 issues would impact UE’s behaviour on requesting the change of Network Slices.

# Proposal

Based on the above discussion the following changes to TS 23.501 v1.2.0 are proposed:

* * * First Change * * *

##### 5.15.5.2.2	Modification of the Set of Network Slice(s) for a UE

The set of Network Slices for a UE can be changed at any time while the UE is registered with a network, and may be initiated by the network, or the UE under certain conditions as described below. In this release it is assumed that the registration area allocated by the AMF to the UE shall have homogeneous support for network slices.

The network, based on local policies, subscription changes and/or UE mobility, may change the set of permitted Network Slice(s) to which the UE is registered. The network may perform such change during a Registration procedure or trigger a notification towards the UE of the change of the supported Network Slices using an RM procedure (which may trigger a Registration procedure). The new Allowed NSSAI is determined as described in clause 5.15.5.2.1, and the AMF provides the UE with the new Allowed NSSAI and Tracking Area list.

NOTE:	The details of the RM procedure used to notify the UE of a change of the supported NSSAI are to be defined.

When a Network Slice instance used for a one or multiple PDU Sessions is no longer available, the 5GC determines whether N1 SM Information can be exchanged between the network (e.g. SMF) and the UE. The following applies:

- If N1 SM Information can be exchanged, the 5GC initiates a network-triggered PDU session release procedure with an appropriate cause value for the impacted PDU session(s), as defined in TS 23.502 [3], clause 4.3.4.

- If N1 SM Information cannot be exchanged, the serving AMF initiates autonomous release of the UE’s SM context in the SMF, UPF and PCF. In case of AMF change (e.g. a new AMF is assigned which is not a part of the Network Slice instance for the no longer available network slice), the new AMF indicates to the old AMF which S-NSSAI or which PDU Session(s) cannot be longer served. The old AMF informs the corresponding SMF(s) to autonomously release the UE’s SM context. The PDU session(s) context is  implicitly released in the UE after receiving the Allowed NSSAI in the Registration Accept message.

The UE uses UE Configuration (e.g. NSSP) to determine whether ongoing traffic can be routed over existing PDU sessions belonging to other Network Slices or establish new PDU session(s) associated with same/other Network Slice.

In order to change the set of S-NSSAIs being used, the UE shall initiate a Registration procedure as specified in clause 5.15.5.2.1.1.

Change of set of S-NSSAIs to which the UE is registered (whether UE or Network initiated) may lead to AMF change subject to operator policy, as described in clause 5.15.5.2.1.

Editor's note:	The condition under which the UE is able to request the change of the Network Slices, and what it is able to request, are FFS.

* * * End of Changes * * *