SA WG2 Meeting #122Bis	S2-175908

21 - 25 August 2017, Sophia Antipolis, France

Source:	Motorola Mobility, Lenovo

Title:	Impacts to 23.502 from modification of the Set of Network Slice(s)

Document for:	Discussion and Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution proposes to implement the concepts from TS 23.501 clause 5.15.5.2.2. in the Registration procedure and PDU Session release procedure from TS 23.502.

# Introduction

The contribution in Tdoc S2-175905 proposes the following changes to TS 23.501 clause 5.15.5.2.2:

When a Network Slice instance used for a one or multiple PDU Sessions is no longer available, the 5GC determines whether N1 SM Information can be exchanged between the network (e.g. SMF) and the UE. The following applies:

- If N1 SM Information can be exchanged, the 5GC initiates a network-triggered PDU session release procedure with an appropriate cause value for the impacted PDU session(s), as defined in TS 23.502 [3], clause 4.3.4.

- If N1 SM Information cannot be exchanged, the serving AMF initiates autonomous release of the UE’s SM context in the SMF, UPF and PCF. In case of AMF change (e.g. a new AMF is assigned which is not a part of the Network Slice instance for the no longer available network slice), the new AMF indicates to the old AMF which S-NSSAI or which PDU Session(s) cannot be longer served. The old AMF informs the corresponding SMF(s) to autonomously release the UE’s SM context. The PDU session(s) context is implicitly released in the UE after receiving the Allowed NSSAI in the Registration Accept message.

This contribution proposes to reflect those changes in TS 23.502 in the Registration procedure and PDU Session release procedure.

In addition some minor corrections are proposed.

# Proposal

Based on the above discussion the following changes to TS 23.502 v0.6.0 are proposed:

* * * 1st Change * * *

##### 4.2.2.2.2	General Registration

Figure 4.2.2.2.2-1: Registration procedure

1.	UE to (R)AN: AN message (AN parameters, RM-NAS Registration Request (Registration type, SUPI or 5G-GUTI, Security parameters, NSSAI, UE 5GCN Capability, PDU session status, PDU session(s) to be re-activated, Follow on request, and MICO mode preference)).

Editor's note:	The required AN parameters are FFS.

In case of 5G-RAN, the AN parameters include e.g. SUPI or the 5G-GUTI, the Selected Network and NSSAI.

In case of NG-RAN, the AN parameters also include Establishment cause. The Establishment cause provides the reason for requesting the establishment of an RRC connection.

The Registration type indicates if the UE wants to perform an "initial registration" (i.e. the UE is in RM-DEREGISTERED state), a "mobility registration update " (i.e. the UE is in registered state and initiates a Registration procedure due to mobility) or a "periodic registration update" (i.e. the UE is in registered state and initiates a Registration procedure due to the periodic update timer expiry, see clause 4.2.2.2.1). The UE performing an initial registration (i.e., the UE is in RM-DEREGISTERED state) to a PLMN for which the UE does not already have a 5G-GUTI, the UE shall include its SUPI in the registration attempt. In other cases, the 5G-GUTI is included and it indicates the last serving AMF. If the UE is already registered via a non-3GPP access in a PLMN different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the 3GPP access, the UE shall not provide over the 3GPP access the 5G-GUTI allocated by the AMF during the registration procedure over the non-3GPP access. Also, if the UE is already registered via a 3GPP access in a PLMN (i.e. the registered PLMN), different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the non-3GPP access, the UE shall not provide over the non-3GPP access the 5G-GUTI allocated by the AMF during the registration procedure over the 3GPP access.

The Security parameters are used for Authentication and integrity protection. NSSAI indicates the Network Slice Selection Assistance Information (as defined in clause 5.15 of TS 23.501 [2]. The PDU session status indicates the previously established PDU sessions in the UE. The PDU session(s) to be re-activated is included to indicate the PDU session(s) that the UE intends to activate. The Follow on request is included when the UE has pending uplink signalling and the UE doesn't include PDU session(s) to be re-activated.

Editor's note:	The content of the Security parameters is FFS.

Editor's note:	Changes required to support "stickiness" is FFS.

Editor's note:	It is FFS if HO attach indication is to be added.

Editor's note:	It is FFS how UE Radio capabilities are provided.

2.	If a SUPI is included or the 5G-GUTI does not indicate a valid AMF the (R)AN, based on (R)AT and NSSAI, if available, selects an AMF

The (R)AN selects an AMF as described in TS 23.501 [2], clause 6.3.5.

If the (R)AN cannot select an appropriate AMF, it forwards the Registration Request to an AMF which has been configured, in (R)AN, to perform AMF selection.

3.	(R)AN to new AMF: N2 message (N2 parameters, RM-NAS Registration Request (Registration type, Subscriber Permanent Identifier or 5G-GUTI, Security parameters, NSSAI and MICO mode preference)).

When 5G-RAN is used, the N2 parameters include the Location Information, Cell Identity and the RAT type related to the cell in which the UE is camping.

When NG-RAN is used, the N2 parameters also include the Establishment cause.

If the Registration type indicated by the UE is periodic registration update, then steps 4 to 17 may be omitted.

4.	[Conditional] new AMF to old AMF: Namf_Communication_UEContextTransfer  (complete Registration Request).

If the UE's 5G-GUTI was included in the Registration Request and the serving AMF has changed since last registration, the new AMF may invoke the Namf_Communication_UEContextTransfer service operation on the  old AMF including the complete Registration Request IE, which may be integrity protected, to request the UE's SUPI and MM Context. See clause 5.2.2.2.2 for details of this service operation. The old AMF uses the integrity protected complete registration request IE to verify if the context transfer service operation invocation corresponds to the UE requested.

The old AMF also transfers the event subscriptions information by each consumer NF, for the UE, to the new AMF.

NOTE: The consumer NFs does not need to subscribe for the events once again with the new AMF after the UE is successfully registered with the new AMF.

5.	[Conditional] old AMF to new AMF: Response to Namf_Communication_UEContextTransfer (SUPI, MM Context, SMF information).

Old AMF responds to the new AMF for the Namf_Communication_UEContextTransfer invocation by including the UE's SUPI and MM Context.

If old AMF holds information about established PDU Sessions, the old AMF sends SMF information including SMF identities and PDU session identities.

If old AMF holds information about active N2AP UE-TNLA bindings to N3IWF, the old AMF includes information about the N2AP UE-TNLA bindings.

6.	[Conditional] new AMF to UE: Identity Request ().

If the SUPI is not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE.

7.	[Conditional] UE to new AMF: Identity Response ().

The UE responds with an Identity Response message including the SUPI.

8.	The AMF may decide to invoke an AUSF. In that case, the AMF, shall be based on SUPI, select an AUSF as described in TS 23.501 [2], clause 6.3.4.

9a.	The AUSF shall initiate authentication of the UE.

The authentication is performed as described in clause X.

Editor's note:	It is FFS how UDM is selected in this step.

Editor's note:	It is FFS whether the AUSF or the AMF initiates the authentication.

If network slicing is used, the AMF decides if the Registration Request needs to be rerouted as described in clause 4.2.2.2.3, where the initial AMF refers to the AMF.

9b	The AMF shall initiate NAS security functions.

The NAS security is performed as described in clause X.

10.	[Conditional] new AMF to old AMF: Namf_Communication_RegistrationCompleteNotify ().

If the AMF has changed the new AMF notifies the old AMF that the registration of the UE in the new AMF is completed by invoking the Namf_Communication_RegistrationCompleteNotify service operation.

If the authentication/security procedure fails, then the Registration shall be rejected, and the new AMF invokes the Namf_Communication_RegistrationCompleteNotify service operation with a reject indication reason code towards the old AMF. The old AMF continues as if the UE context transfer service operation was never received.

The new AMF determines the Allowed NSSAI to be used by the UE as described in [2] clause 5.15. The new AMF may interrogate with the NSSF to determine the Allowed NSSAI and whether AMF relocation procedure as per clause 4.2.2.2.3 is needed.

If the new AMF determines that one or more of the S-NSSAIs used in the old registration area cannot be served in the target registration area, and the new AMF is not able to directly reach the corresponding SMF(s), then the new AMF includes an indication to the old AMF which particular PDU Session(s) cannot be used in the target registration area and the new AMF includes a cause value for autonomous UE SM context release in the network.

See clause 5.2.2.2.3 for details of Namf_Communication_RegistrationCompleteNotify service operation.

10a.	[Conditional] old AMF to SMF: Nsmf_PDUSession_Release SM Context ()

If the old AMF receives an indication from the new AMF that particular PDU Session(s) cannot be served in the target registration area and a cause value for autonomous SM context release, the old AMF invokes the Nsmf_PDUSession_Release SM Context service operation with the SMF to release the UE’s SM context and the user plane resources.

11.	[Conditional] new AMF to UE: Identity Request ().

If the PEI was not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE to retrieve the PEI.

12.	Optionally the new AMF initiates ME identity check by invoking the N5g-eir_MEIdentityCheck_Get service operation (see clause 5.2.4.2.2).

The PEI check is performed as described in clause 4.7.

13.	If step 14 is to be performed, the new AMF, based on the SUPI, selects a UDM.

The AMF selects an UDM as described in TS 23.501 [2], clause X.

14a. If the AMF has changed since the last registration, or if the UE provides a SUPI which doesn't refer to a valid context in the AMF, or if the UE registers to the same AMF it has already registered to a non-3GPP access (i.e. the UE is registered over a non-3GPP access and initiates this registration procedure to add a 3GPP access), the new AMF invokes the Nudm_UEContextManagement_Registration service operation towards the UDM. If there is no subscription context for the UE in the AMF, the "subscription data retrieval indication" is included. The new AMF provides the access type it serves for the UE to the UDM and the access type is set to "3GPP access". The UDM stores the associated access type together with the serving AMF.

See sub-clause 5.2.3.2.1 for details of Nudm_UEContextManagement_Registration service operation.

Editor's note:	Whether interactions with UDM and PCF includes mobility restriction related information is FFS.

Editor's note:	The PEI is to be provided to the UDM in the Update Location procedure.

14b. [Conditional] If "the subscription data retrieval" indication was included in step 14a, the UDM invokes the Nudm_SubscriptionData_UpdateNotification service operation to provide the subscription data from the UDM. See clause 5.2.3.3.2 for details of this service operation. The new AMF creates an MM context for the UE after getting the mobility related subscription data from the UDM.

14c. When the UDM stores the associated access type together with the serving AMF as indicated in step 14a, it will cause the UDM to initiate a Nudm_UEContextManagement_RemoveNotification (see clause 5.2.3.2.2) to the old AMF corresponding to 3GPP access, if one exists. The old AMF removes the MM context of the UE. If the serving NF removal reason indicated by the UDM is "Initial Registration", then the old AMF invokes the Namf_EventExposure_Notify service operation towards all the associated SMFs of the UE to notify that the UE is de-registered from old AMF. The SMF shall release the PDU session(s) on getting this notification.

15.	Conditionally the new AMF, based on the SUPI, selects a PCF.

The AMF selects a PCF as described in TS 23.501 [2], clause X.

Editor's note:	It is FFS whether the Permanent User ID is used for PCF selection.

Editor's note:	The conditions when the AMF selects a PCF is FFS.

16.	[Optional] new AMF to PCF: Npcf_PolicyControl_PolicyCreate (SUPI).

If the AMF has not yet obtained Access and Mobility policy for the UE or if the Access and Mobility policy in the AMF are no longer valid, the AMF requests the PCF to apply operator policies for the UE by creating a policy control session with the PCF through the Npcf_PolicyControl_PolicyCreate service operation (see clause 5.2.5.2.2). In roaming case, the interaction between H-PCF and V-PCF is required for the provision of Access and Mobility policy.

PCF to new AMF: Response to Npcf_PolicyControl_PolicyCreate (Access and Mobility policy data).

The PCF responds to the Npcf_PolicyControl_PolicyCreate service operation and provides the Access and Mobility policy data for the UE to the AMF.

17.	[Conditional] new AMF to SMF: Namf_EventExposure_Notify ().

The AMF invokes the Namf_EventExposure_Notify (see clause 5.2.2.3.4) in one or both of the following scenarios:

-	If the AMF is changed, the new AMF notifies each SMF of the new AMF serving the UE by informing the UE reachability status including the PDU session status from the UE relevant for each SMF. In case the AMF has changed, it is assumed that the old AMF provides the available SMF information. Based on the PDU session status provided by the new AMF checks the PDU session status and in the Namf_EventExposure_Notify service operation the SMF either re-activates the PDU sessions by executing the steps 4 to 12 specified in clause 4.2.3.2 to complete the User Plane(s) setup without sending MM NAS Service Accept from the AMF to (R)AN described in step 9 of clause 4.2.3.2 or  releases any network resources related to PDU sessions that the UE indicated as not established.

-	If the UE was in MICO mode and the AMF had notified an SMF of the UE being unreachable and that the SMF needs not to send DL data notifications to the AMF, the AMF informs the SMF that the UE is reachable.

-	If the AMF had notified an SMF of the UE being reachable only for regulatory prioritized service and the UE enters into Allowed area, the AMF informs the SMF that the UE is reachable.

NOTE 1:	the AMF will also notify any other NF that subscribed to UE reachability that the UE is reachable.

-	If an SMF has subscribed to UE location change notification via Namf_EventExposure_Subscribe service operation and if the AMF detects that the UE has moved out the area of interest subscribed by an SMF serving the UE, the AMF invokes Namf_EventExposure_Notify service operation to inform the SMF of the new location information of the UE.

NOTE 2: Whether notification Ack need a separate message or be realized in the transport layer will be determined in stage 3.

The SMF may decide to trigger e.g. new intermediate UPF insertion or UPF relocation as described in step 5 in clause 4.2.3.2.

If the Registration type indicated by the UE is periodic registration update, then steps 20 and 21 may be omitted.

Editor's note:	SMF relocation in relation to a Registration procedure is FFS.

18. New AMF to N3IWF: N2 Request().

The AMF may decide to modify the N2AP UE-TNLA-binding toward N3IWF as described in 4.2.7.1. This is done in case AMF is changed and old AMF have existing N2AP UE-TNLA-bindings toward a N3IWF for the UE.

19.	N3IWF to new AMF: N2 Response().

Editor's note:	It is FFS whether further description is needed on how the old AMF releases the existing N2AP UE-TNLA-bindings toward a N3IWF is FFS.

20.	[Conditional] old AMF to PCF: Npcf_PolicyControl_PolicyDelete ().

If the old AMF previously requested UE context to be established in the PCF, the old AMF terminates the UE context in the PCF by invoking the Npcf_PolicyControl_PolicyDelete service operation (see clause 5.2.5.2.4).PCF to old AMF: Response to Npcf_PolicyControl_PolicyDelete ().

21.	New AMF to UE: Registration Accept (5G-GUTI, Registration area. Mobility restrictions, PDU session status, NSSAI, Periodic registration update timer, LADN Information and accepted MICO mode).

The AMF sends a Registration Accept message to the UE indicating that the registration has been accepted. 5G-GUTI is included if the AMF allocates a new 5G-GUTI. Mobility restrictions is included in case mobility restrictions applies for the UE. The AMF indicates the established PDU sessions to the UE in the PDU session status. The UE removes locally any internal resources related to PDU sessions that are not marked as established in the received PDU session status and for which the UE has requested PDU session establishment and not received SMF response yet. If the PDU session status information was in the Registration Request, the AMF shall indicate the PDU session status to the UE. The NSSAI includes the Allowed S-NSSAIs. If the UE subscription data includes subscribed LADN identification information, the AMF shall include in the Registration Accept message the LADN Information for LADNs, defined in TS 23.501 [2] clause 5.6.5, that are available within the Registration area determined by the AMF for the UE. If the UE included MICO mode in the request, then AMF responds whether MICO mode should be used.

When the Follow on request is included, the AMF shall not release the signalling connection immediately after the completion of the registration procedure.

Editor's note:	The exact reference to steps of UE triggered Service Request procedure in clause 4.2.3.2 will be updated when the UE triggered Service Request procedure is stable.

Editor's note:	It is FFS whether NSSAI is provided to RAN by AMF.

Editor's note:	It is FFS whether LADN subscription information (i.e. information which indicates whether the UE subscribes the LADN service) is included in UE subscription data or is managed by policy information by PCF.

22.	[Conditional] UE to new AMF: Registration Complete ().

The UE sends a Registration Complete message to the AMF to acknowledge if a new 5G-GUTI was assigned.

When the "PDU session(s) to be re-activated" is not included in the Registration Request, the AMF releases the signalling connection with UE, according to clause 4.2.6. When the Follow on request is included in the Registration Request, the AMF shall not release the signalling connection immediately after the completion of the registration procedure.

Editor's note:	Details of N2 interaction is FFS.

* * * 2nd Change * * *

#### 4.3.4.2	UE or network requested PDU session release for Non-Roaming and Roaming with Local Breakout

Figure 4.3.4.2-1 captures both the UE requested PDU session release procedure and the network requested PDU session release procedure. The procedure allows the UE to request the release of one PDU session. The procedure also allows the SMF or PCF to initiate the release of a PDU session. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to know which UE is the target of the SMF request as well to determine which access towards the UE to use

Editor's note:	Alignment of message naming with the procedures for PDU session establishment is needed.

Editor's note:	The names of N11 Message are FFS.

Figure 4.3.4.2-1: UE or network requested PDU session release for non-roaming and roaming with local breakout

1.	The procedure is triggered by one of the following options:

1a.	The UE initiates the UE requested PDU session release procedure by the transmission of an NAS message (N1 SM information PDU Session Release Request), PDU Session ID) message. The N1 SM information message is forwarded by the (R)AN to the 5G Core with an indication of User Location Information. This message is relayed to the SMF corresponding to the PDU Session ID via N2 and the AMF. The AMF invokes the Nsmf_PDUSession_UpdateSMContext service operation and provides the N1 SM message to the SMF.

NOTE 1:	Depending on the access type, when the UE is in CM-IDLE state, the UE can trigger a Service Request procedure before being able to release the PDU session.

1b.	The PCF may invoke the Npcf_PolicyControl_PolicyUpdate service operation to request the release of the PDU session.

1c.	The SMF may decide to release a PDU session e.g. based on a request from the DN (cancelling the UE authorization to access to the DN) or based on a request from the UDM (subscription change) or from the OCS. The release procedure also may be triggered based on locally configured policy (e.g. the release procedure may be related with the UPF relocation for SSC mode 2 / mode 3).

1d.	The AMF may decide to initiate a PDU session release e.g. based on the change of the set of network slices for a UE where a network slice instance is no longer available as described in clause 5.15.5.2.2 in [2]. In such case the AMF invokes Nsmf_PDUSession_Release SM Context service operation, provides the PDU Session ID to the SMF and provides indication whether the release is autonomous in the network or explicit N1 SM Information can be sent.

If the SMF receives one of the triggers in step 1a ~ 1c, the SMF starts PDU session release procedure.

If the SMF receives trigger as per step 1d with autonomous release indication, the SMF initiates steps 2, 3a and 9, but the SMF shall not send N1 SM Information (e.g. PDU Session Release Command) to the UE and N2 Resource Release request (PDU Session ID).

2.	The SMF releases the IP address / Prefix(es) that were allocated to the PDU session and releases the corresponding User Plane resources:

2a.	The SMF sends an N4 Session Release Request (N4 Session ID) message to the UPF. The UPF shall drop any remaining packets of the PDU session and release all tunnel resource and contexts associated with the N4 Session.

2b.	The UPF acknowledges the N4 Session Release Request by the transmission of an N4 Session Release Response (N4 Session ID) message to the SMF.

NOTE 2:	If there are multiple UPFs associated with the PDU session, the Session Release Request procedure (steps 2a and 2b) is done for each UPF.

3a.The SMF responds to the AMF with the Nsmf_PDUSession_UpdateSMContext response (N2 SM Resource Release request, N1 SM Information (PDU Session Release Command))

The SMF creates an N1 SM Information including PDU Session Release Command message (PDU Session ID, Cause). The Cause may indicate a trigger to establish a new PDU session with the same characteristics (e.g. when procedures related with SSC mode 2 are invoked). The Cause may further indicate that the PDU Session is released due to the no longer available S-NSSAI meaning that the UE shall not initiate new PDU Session establishment procedure with this S-NSSAI.

NOTE 3:	SSC mode 2 is defined in TS 23.501 [2] clause 5.6.9.

3b.	If the PDU session release is initiated by the SMF, the SMF invokes the Namf_Communication_N1N2MessageTransfer service operation (N1 SM Information (PDU Session Release Command), skip indicator).	If the UP connection of the PDU session is active, the SMF shall also include the (N2 Resource Release request (PDU Session ID)) in the Namf_Communication_N1N2MessageTransfer, to release the (R)AN resources associated with the PDU session.

The "skip indicator" tells the AMF whether it may skip sending the N1 SM container to the UE (e.g. when the UE is in CM-IDLE mode).

NOTE 4:	If the procedure is triggered to relocate PDU session anchor of a PDU session with SSC mode 2, the SMF is assumed not to include the "N1 SM delivery can be skipped" Indication.

Editor's note:	It is FFS whether a Release Indication to explicitly delete the association of PDU Session ID and serving SMF ID within the AMF is added to the SMF request. This would allow skipping step 9 when the UE is in CM-IDLE state and "N1 SM delivery can be skipped" is included in the N11 message.

Editor's note:	It is FFS whether "N1 SM delivery can be skipped" Indication is required.

4.	If the UE is in CM-IDLE state and " skip indicator " is included in the Namf_Communication_N1N2MessageTransfer service operation, the AMF acknowledges the step 4 by sending an N11 Response message ("skip indicator " ) to SMF and steps 5 to 7 are skipped.

NOTE 5:	The UE and the 5GC will get synchronized about the status (released) of the PDU session at the next Service Request or Registration procedure.

Otherwise, if the UE is in CM-IDLE state and "N1 SM delivery can be skipped" is not indicated, the AMF initiates the network triggered Service Request procedure to transmit the NAS message (PDU session ID, N1 SM information) to the UE.

If the UE is in CM-CONNECTED state, then the AMF transfers the SM information received from the SMF in step 4 (N2 SM Resource Release request, N1 SM information) to the (R)AN.

5.	When the (R)AN has received an N2 SM request to release the AN resources associated with the PDU session it issues AN specific signalling exchange(s) with the UE to release the corresponding AN resources.

In case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE releasing the RAN resources related to the PDU session.

During the this procedure, the (R)AN sends any NAS message (N1 SM PDU Session Release Command) received from the AMF in step 5.

The UE acknowledges the PDU Session Release Command by sending a NAS message (PDU session ID, N1 SM information (PDU Session Release Ack)) message via N1 SM signalling sent over the (R)AN.

6.	[Conditional] If the (R)AN had received a N2 SM request to release the AN resources, the (R)AN acknowledges the N2 SM Resource Release Request by sending an N2 SM Resource Release Ack (N1 SM information (PDU Session Release Ack), User Location Information) Message to the AMF.

Otherwise, the (R)AN just forwards the NAS message (PDU session ID, N1 SM information (PDU Session Release Ack)) from the UE to the AMF.

7a.	The AMF invokes the Nsmf_PDUSession_UpdateSMContext (N1 SM Release Ack, N2 SM Resource Release Ack) to the SMF.

8.	The SMF responds to the AMF that the PDU session is released with an Nsmf_PDUSession_UpdateSMContext response. The AMF and SMF shall remove all contexts (including the PDU session ID) associated with the PDU session as well as any event subscriptions by the SMF on the AMF.

Editor's note:	It is FFS whether this takes the form of a Nsmf_ PDU Session Event Report Notification (AMF): The SMF sends (N11 Message) PDU Session Event Report Notification as defined in Clause 5.2.7.1 to the AMF; the event trigger is PDU session release.

9.	If Dynamic PCC applied to this session the SMF invokes the Npcf_PolicyControl_PolicyDelete service operation to delete the PDU CAN session.

SMF notifies any entity that has subscribed to User Location Information related with PDU session change

If it is the last PDU session it is handling for the UE the SMF releases the association with the UDM.

NOTE 6:	The order with which SMF releases the resources is implementation dependant.

* * * End of Changes * * *