SA WG2 Meeting #122bis	S2-176440 was 5509

August 21 – 25, 2017, Sophia Antipolis, France	(was S2-17----)

Source:	China Mobile, ZTE, Verizon

Title:	23.502: Service Request procedure using Mobile Termination service

Document for:	Discussion and Approval

Agenda Item:	6.5.2

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution updated the network triggered service request procedures by MT services and the AMF_MT service is further clarified.

- Introduction

In TS 23.502 section 5.2.2.4, Namf_ MT_EnableUEReachability service has been introduced but the Network triggered Service Request procedure has not been updated accordingly.

Approach:

When the network has downlink data or signaling to send, the other NF(s) first invokes AMF MT service, to ensure that the UE enter the connection state, and then sends the downlink signaling to the UE through AMF.

Cons: addition of two signaling and some latency This can be alleviated with synchronized way of service invoking.

Pros:

- NFs will know the UE status (IDLE/Connected), therefore, can decide whether to further send downlink signaling (or data in SMSF case). This can avoid AMF to buffer data (in case SMSF send SMS to AMF). This also simplifies the failure handling case, i.e., no need to roll back for AMF.

- As discussed in S2-175253, enable a separate Mobile Termination (MT) service will help decouple the functionality into more granular services and meet for the scalability requirements, in view that the resources consuming for paging is a big part of the current network element (MME).

- Proposal

It is proposed to update 23.502 cause 4 the Network triggered Service Request using the Namf_ MT_EnableUEReachability service.

The way to update 4.2.3.4 “Network Triggered Service Request”

There are three alternatives to leverage MT services:

Option 1) Consumer (e.g., SMF, SMSF) invokes AMF Communication service and Communication service invokes MT service if required. That is: Consumer Communication service MT service (do paging if required)

Option 2) Consumer invokes MT service and MT service further invokes Communication service: Consumer MT service (do paging if required) Communication service

Option 3) Consumer invokes MT service first then invokes Communication service: Step a) Consumer MT service (do paging if required), Step b) Consumer Communication service

The Option 1 and 2 contains the internal service invoke. The option 1) is actually can be considered as current mechanism and the MT service is defined but not exhibits service interface to NF services external of AMF. Therefore,

Proposal 1: it is suggested to adopt Option 3.

To do that, the current Step 3a (the N11 message) is replaced by invoking MT service to make UE reachable.

The N1N2 message in current Step 3a is sent when UE becomes connected (i.e., becomes Step 9).

* * * First Change * * *

The text below are all new. The changes are shown as compared with 4.2.3.4 for reference

4.2.3.x	Network initiated UE reachability procedure

This procedure is used when the network needs to achieve fast message deliver, i.e., make UE reachable ahead of network signal (e.g. N1 signalling to UE, Mobile-terminated SMS, User Plane resource establishment for PDU session(s) to deliver mobile terminating user data) with a UE. This procedure is also used when the network does not want to buffer downlink MT SMS or MT data over signaling in AMF.

If the UE is in CM-IDLE state or CM-CONNECTED state in 3GPP access, the network initiates a network triggered Service Request procedure. If the UE is in CM-IDLE state, and asynchronous type communication is not activated, the network sends a Paging Request to (R)AN/UE. The Paging Request triggers the UE triggered Service Request procedure in the UE. If asynchronous type communication is activated, the network stores the received message and forward the message to the (R)AN and/or the UE (i.e. synchronizes the context with the (R)AN and/or the UE) when the UE enters CM-CONNECTED state.

If the UE is in CM-CONNECTED state in non-3GPP access, the network shall initiate a Network triggered Service Request procedure when the network needs to signal with a UE. If the UE is in CM-IDLE state in non-3GPP access and if the UE is simultaneously registered over 3GPP and non-3GPP accesses in a PLMN, the network shall initiate a Network triggered Service Request procedure over 3GPP access.

Figure 4.2.3.x-1: Network Triggered Service Request

1. The requester invokes Namf_MT_EnableUEReachability (SUPI, Priority, Paging Policy Indication) service. If UE is in CM-CONNECTED state, steps 3 to 7 are skipped.

If the requester receives any additional Notification message for the same UE but with higher priority, the requester invokes a new Namf_MT_EnableUEReachability indicating the higher priority and PDU Session ID to the AMF.

If the SMF, while waiting for the User Plane to be activated, receives an N11 message response from an AMF other than the one to which the SMF sent an N11 message, the SMF sends the N11 message to this AMF.

When supporting Paging Policy Differentiation, the requester indicates in the Namf_MT_EnableUEReachability  the Paging Policy Indication as described in TS 23.501 [2] clause 5.4.3.

NOTE 1:	AMF may receive request message(s) from other network functions which leads to signalling towards UE/RAN, e.g. network initiated deregistration, SMF initiated PDU session modification. If the UE is in CM-CONNECTED state and the AMF only delivers N1 message towards UE, the flow continues in step 6 below.

2.	[conditional] The AMF responds to the requester.

If the UE is in CM-IDLE state, and the AMF determines that the UE is not reachable for paging, the AMF shall skip steps 3 to 7 and either send Namf_MT_EnableUEReachability response to network functions from which AMF received the request message in step 1, indicating the UE is not reachable, or the AMF performs asynchronous type communication and store the N11 message. If asynchronous type communication is invoked and the AMF stored Namf_MT_EnableUEReachability Request message, the AMF initiates communication with the UE and (R)AN when the UE is reachable e.g. when the UE enters CM-CONNECTED state.

If the AMF has determined the UE is unreachable for the requester (e.g., due to the UE in MICO mode or the UE is only registered over non-3GPP access and its state is CM-IDLE), then the AMF rejects the request. The AMF may include in the reject message an indication that the requester needs not send further notifications to the AMF, if the requester has not subscribed to the event of the UE reachability. The AMF stores an indication that the requester has been informed that the UE is unreachable.

If the UE is not in MICO mode and the AMF detects the UE is in Non-allowed area unless the request from the requester is regulatory prioritized service, the AMF reject the request and notify the requester that the UE is reachable only for regulatory prioritized service. The AMF stores an indication that the requester has been informed that the UE is reachable only for regulatory prioritized service.

If the Registration procedure with AMF change is in progress when the old AMF receives a Namf_MT_EnableUEReachability Request message, the old AMF may reject the message with an indication that the message has been temporarily rejected.

3, 4.	[Conditional] AMF sends Paging message to (R)AN node.

The steps follows the paging steps of clause 4.2.3.4 step 4 and 5.

5.	[Conditional] AMF to SMF: Namf_MT_EnableUEReachability response.

The AMF supervises the paging procedure with a timer. If the AMF receives no response from the UE to the Paging Request message, the AMF may apply further paging according to any applicable paging strategy described in step 2.

If the AMF receives no response from the UE, the AMF considers the UE as unreachable and the SM N2 message cannot be routed to the (R)AN, so the AMF shall return an Namf_MT_EnableUEReachability response with an appropriate "failure cause", e.g. UE unreachability, to the requester to indicate the failure of "message routing service", unless the AMF is aware of an ongoing MM procedure that prevents the UE from responding, i.e. the AMF receives an N14 Context Request message indicating that the UE performs Registration procedure with another AMF.

6.	When UE is in the CM-IDLE state, upon reception of paging request, the UE initiates the UE triggered Service Request procedure (clause 4.2.3.2), and step 1, 2, 3 of this procedure are executed.

6.	When UE is in the CM-IDLE state, upon reception of paging request, the UE initiates the UE triggered Service Request procedure (clause 4.2.3.2) , and step 1, 2, 3 of that procedure are executed.

7. AMF to SMF: Namf_ MT_EnableUEReachability response to requester indicating UE is reachable.

* * * Second Change * * *

5.2.2	AMF Services

5.2.2.1	General

The following table shows the AMF Services and AMF Service Operations.

Table 5.2.2.1-1: List of AMF Services

| Service Name | Service Operations | Known Consumer(s) | Operation Semantic | Reference |
| --- | --- | --- | --- | --- |
| Namf_Communication | UEContextTransfer | Peer AMF | Request/ Response | Step 4 and 5 of clause 4.2.2.2.2. |
|  | UERegistrationCompleteNotify | Peer AMF | Subscribe / Notify | Step 10 of clause 4.2.2.2.2. |
|  | N1MessageNotify | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 3 clause 4.3.2.2.1. Step 5a of clause 4.2.2.2.3. |
|  | N1MessageSubscribe | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 5a of clause 4.13.3.6. |
|  | N1MessageUnSubscribe | SMF, SMSF, PCF, NEF | Subscribe / Notify | Step 7 of clause 4.13.3.3. Step 6c of clause 4.13.3.6. |
|  | N1N2MessageTransfer | SMF, SMSF, PCF, NEF | Request/ Response | Step 10 of clause 4.3.2.2.1. Step 14 of lause 4.3.2.2.2. Step 5a of clause 4.13.3.6. |
|  | N2InfoSubscribe | SMF | Subscribe / Notify | Step 10 of clause 4.3.2.2.1. Step 14 of clause 4.3.2.2.2. |
|  | N2InfoUnSubscribe | SMF | Subscribe / Notify |  |
|  | N2InfoNotify | SMF | Subscribe / Notify | Step 14 of clause 4.3.2.2.1. Step 18 of clause 4.3.2.2.2. Step 9 of clause 4.2.3.2. |
| Namf_EventExposure | Subscribe | NEF, SMF, PCF, UDM | Subscribe / Notify | Step 10 of clause 4.3.2.2.1. Step 14 of clause 4.3.2.2.2. |
|  | UnSubscribe | NEF, SMF, PCF, UDM | Subscribe / Notify |  |
|  | Notify | NEF, SMF, PCF, UDM | Subscribe / Notify | Step 4a of clause 4.2.3.2. Step 3a of clause 4.2.3.3. |
| Namf_MT | EnableUEReachability | NEF, SMF, PCF, UDM | Request/ Response | EnableUEReachability |

Editor's note:	Whether additional NF services can be identified or be decomposed from existing NF services are FFS.

5.2.2.4	Namf_MT service

5.2.2.4.1	General

Service description: It provides a NF the service that make sure UE is reachable to send MT signaling or data. The following are the key functionalities of this NF service

-   paging UE if UE is in IDLE state and respond other NF after the UE enters CM-CONNECTED state.

-  response to the requester NF if UE is in CONNECTED state.

5.2.2.4.2	Namf_MT_ EnableUEReachability service operation

Service operation name: Namf_MT_EnableUEReachability.

Description: The consumer NF uses this service operation to make UE reachable which may lead to page UE and make UE enter connected state if required.

Known NF Consumers: NEF, SMF, PCF, UDM

Inputs, Required: NF ID, UE ID, PPI, 5QI, DNN.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

See step 3 of clause 4.2.3.4 for details on the usage of this service operation, and 4.2.2.3.3 for UDM initiated de-registration.

The consumer NF does not need to know UE state. The AMF accepts the request and respond the consumer NF immediately if UE is in CM-CONNECTED state. If the UE is in CM-IDLE state, the AMF may page UE and respond to the consumer NF after the UE enters CM-CONNECTED state. Detailed procedures of the service operation are described in clause 4.2.3.x.

*************** end of change *********************