SA WG2 Meeting #S2-122bis	S2-176504

August 21 – 25, 2017, Sophia Antipolis, France	(was S2-175956)

Source:	ITRI

Title:	Update to the information returned by NSSF

Document for:	Approval

Agenda Item:	6.5.1

Work Item / Release:	5GS_Ph1/ Rel15

Abstract of the contribution: This paper proposes to update the network slicing in Sec. 5.13 to update the information returned by NSSF after processing the information provided by current AMF.

1	Discussion

This contribution proposes to remove the following Editor’s note

“Whether more information is returned by the NSSF (e.g. mapping of S-NSSAI to specific network slice instances) is FFS.”

In case of AMF behavior in (B), the AMF queries NSSF with Requested NSSAI, the subscribed NSSAIs, location information, and other required information used by UE. After receiving the query from AMF, NSSF will process the provided information and return the information as Allowed NSSAI, target AMF Set, the list of candidate AMF(s). At the same time, NSSF also determines the set of network slices instances to serve the UE according to the Allowed NSSAI. Instead of the mapping S-NSSAI to a specific network slice instance by AMF, NSSF can also return a target NRF to the AMF. When the AMF receive the S-NSSAIS information during PDU session establishment procedure from UE, the AMF can query the S-NSSAIS with the target NRF to know which SMF shall be selected for this particular S-NSSAI.

For this reason, this contribution adds the target NRF information to the set of network slice instance(s) as one of returned information from NSSF.

2	Proposal

It is proposed to adopt the following content in TS 23.501.

* * * * (First Change) * * * *

5.15.5.2.1 Registration to a Set of Network Slices

When a UE registers with a PLMN, if the UE for this PLMN has a Configured NSSAI or an Allowed NSSAI, the UE shall provide to the network in RRC and NAS layer a Requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wishes to register, in addition to the Temporary User ID if one was assigned to the UE.

The Requested NSSAI may be either:

-	the Configured-NSSAI, or a subset thereof as described below, if the UE has no Allowed NSSAI for the current PLMN; or

-	the Allowed-NSSAI, or a subset thereof as described below, if the UE has an Allowed NSSAI for the current PLMN; or

-	the Allowed-NSSAI, or a subset thereof as described below, plus one or more S-NSSAIs from the Configured-NSSAI for which no corresponding S-NSSAI is present in the Allowed NSSAI and that were not previously permanently rejected (as defined below) by the network for the present tracking area.

The subset of Configured-NSSAI provided in the Requested NSSAI consists of one or more S-NSSAI(s) in the Configured NSSAI applicable to this PLMN, if the S-NSSAI was not previously permanently rejected (as defined below) by the network for the present registration area, or was not previously added by the UE in a Requested NSSAI.

The subset of Allowed NSSAI provided in the Requested NSSAI consists of one or more S-NSSAI(s) in the last Allowed NSSAI for this PLMN.

The UE may provide in the Requested NSSAI an S-NSSAI from the Configured NSSAI that the UE previously provided to the serving PLMN in the present registration area if the S-NSSAI was not previously permanently rejected (as defined below) by the network for the present registration area.

The UE shall include the Requested NSSAI at RRC Connection Establishment and in NAS messages. The RAN shall route the NAS signalling between this UE and an AMF selected using the Requested NSSAI obtained during RRC Connection Establishment. If the RAN is unable to select an AMF based on the Requested NSSAI, it routes the NAS signalling to an AMF from a set of default AMFs.

Editor's note: Whether NSSAI in RRC and NAS are exactly the same, is to be determined.

When a UE registers with a PLMN, if for this PLMN the UE has no Configured NSSAI or Allowed NSSAI, the RAN shall route all NAS signalling from/to this UE to/from a default AMF. The UE shall not indicate any NSSAI in RRC Connection Establishment or Initial NAS message unless it has a Configured NSSAI or Allowed NSSAI for the corresponding PLMN. When receiving from the UE a Requested NSSAI and a 5G-S-TMSI in RRC, if the RAN can reach an AMF corresponding to the 5G-S-TMSI, then RAN forwards the request to this AMF. Otherwise, the RAN selects a suitable AMF based on the Requested NSSAI provided by the UE and forwards the request to the selected AMF. If the RAN is not able to select an AMF based on the Requested NSSAI, then the request is sent to a default AMF.

When the AMF selected by the AN receives the UE Initial Registration request:

-	As part of the registration procedure described in 3GPP TS 23.502 [3], clause 4.2.2.2.2, the AMF may query the UDM to retrieve UE subscription information including the Subscribed S-NSSAIs.

-	The AMF verifies whether the S-NSSAI(s) in the Requested NSSAI are permitted based on the Subscribed S-NSSAIs.

-	When the UE context in the AMF does not yet include an Allowed NSSAI, the AMF queries the NSSF (see (B) below for subsequent handling), except in the case when, based on configuration criteria in this AMF, the AMF is allowed to determine whether it can serve the UE (see (A) below for subsequent handling).

Editor's note: Whether to define the configuration criteria, and if so, which criteria, is FFS.

-	When the UE context in the AMF already includes an Allowed NSSAI, based on configuration criteria for this AMF, the AMF may be allowed to determine whether it can serve the UE (see (A) below for subsequent handling).

Editor's note: Whether to define the configuration criteria, and if so, which criteria, is FFS.

(A) Depending on fulfilling the configuration criteria as described above, the AMF may be allowed to determine whether it can serve the UE, and the following is performed:

-	AMF checks whether it can serve all the S-NSSAI(s) from the Requested NSSAI present in the Subscribed S-NSSAIs, or all the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs in case no Requested NSSAI was provided (see clause 5.15.3).

-	If this is the case, the AMF remains the serving AMF for the UE. The Allowed NSSAI is then composed of the list of S-NSSAI(s) in the Requested NSSAI permitted based on the Subscribed S-NSSAIs, or, if no Requested NSSAI was provided, all the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs (see (C) below for subsequent handling).

-	If this is not the case, the AMF queries the NSSF (see (B) below).

(B) When required as described above, the AMF needs to query the NSSF, and the following is performed:

-	The AMF queries the NSSF, with Requested NSSAI, the Subscribed S-NSSAIs, location information, and possibly access technology being used by the UE.-	Based on this information, local configuration, and other locally available information including RAN capabilities in the registration area, the NSSF does the following:

Editor's note: Whether more information can be sent to the NSSF is FFS.

-	It selects the set of network slice instances to serve the UE.

-	It determines the target AMF Set to be used to serve the UE, or, based on configuration, the list of candidate AMF(s), possibly after querying the NRF.

-	It determines the Allowed NSSAI.

-	Additional processing to determine the Allowed NSSAI in roaming scenarios, as described in clause 5.15.6.

Editor's note: the roaming scenario for network slice instance selection is FFS.

The NSSF returns to the current AMF the Allowed NSSAI and the target AMF Set, serving NRF(s)  corresponding to the Allowed S-NSSAI(s) or, based on configuration, the list of candidate AMF(s). It may return also information regarding rejection causes for S-NSSAI(s) not included in the Allowed NSSAI which were part of the Requested NSSAI.

Editor's note:  Whether more information is returned by the NSSF (e.g. mapping of S-NSSAI to specific network slice instances) is FFS.

(C) Depending on the available information and based on configuration, the AMF may query the NRF with the target AMF Set. The NRF returns a list of candidate AMFs.

If rerouting to a target serving AMF is necessary, the current AMF reroutes the Registration Request to a target serving AMF as described in clause 5.15.5.2.3.

The serving AMF shall return to the UE the Allowed NSSAI. It may also indicate to the UE for Requested S-NSSAI(s) not included in the Allowed NSSAI, whether the rejection is permanent (e.g. the S-NSSAI is not supported in the PLMN) or temporary (e.g. the S-NSSAI is not currently available in the registration area).

Upon successful Registration, the UE is provided with a 5G-S-TMSI by the serving AMF. The UE shall include this 5G-S-TMSI in any RRC Connection Establishment during subsequent initial accesses to enable the RAN to route the NAS signalling between the UE and the appropriate AMF.

Editor's note: Aspects of uniqueness of the 5G-S-TMSI are addressed in the discussion on registration management.

If the UE receives an Allowed NSSAI from the serving AMF, it shall store this new Allowed NSSAI and override any previously stored Allowed NSSAI for this PLMN, as described in clause 5.15.4.

* * * * End of First Change * * * *