SA WG2 Meeting #122bis	S2- 176556

21 - 25 August 2017, Sophia Antipolis, France	(revision of S2-176487)

Source:	LG Electronics

Title:	TS 23.501: Clarification on QoS parameters sent to the UE

Document for:	Approval

Agenda Item:	6.5.5 QoS concept and functionality

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution is proposed to clarify which QoS parameters are sent to the UE.

Discussion

According to TS 23.501 5.7.1, some QoS parameters are included in the QoS rule and sent to the UE as follows:

The UE performs the classification and marking of UL User plane traffic, i.e. the association of uplink traffic to QoS flows, based on QoS rules. These rules may be explicitly provided to the UE (at PDU Session establishment or QoS flow establishment), pre-configured in the UE or implicitly derived by UE by applying reflective QoS. A QoS rule contains a QoS rule identifier which is unique within the PDU session, the QFI of the associated QoS flow, one or more packet filters and a precedence value. Additionally, for an assigned QFI, the QoS rule contains the QoS parameters relevant to the UE. There can be more than one QoS rule associated with the same QoS Flow (i.e. with the same QFI).

However, it is not clear which QoS parameters are relevant to the UE.

In EPS, following parameters are sent to the UE.

-	QCI

-	Maximum bit rate for uplink / downlink

-	Guaranteed bit rate for uplink / downlink

-	APN-AMBR

It is proposed to send QoS parameters to the UE in 5GS as follows which is similar with EPS.

| QoS parameters | UE | AN | UPF |
| --- | --- | --- | --- |
| 5QI | O | O | O |
| ARP | X | O | O |
| GFBR for UL/DL | O | O | O |
| MFBR for UL/DL | O | O | O |
| Notification control | X | O | X |
| UE-AMBR | X | O | X |
| Session-AMBR | O | O | O |
| RQA | X | O | X |

Also wrong clause number which is referencing a removed clause is updated.

Proposal

* * * * Start of 1st Change * * * *

5.7.2	5G QoS Parameters

A 5QI is a scalar that is used as a reference to 5G QoS characteristics defined in clause 5.7.4, i.e. access node-specific parameters that control QoS forwarding treatment for the QoS flow (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

The 5QI in the standardized value range have one-to-one mapping to a standardized combination of 5G QoS characteristics as specified in Table 5.7.4-1.

For non-standardized combinations of 5G QoS characteristics, a 5QI value from the non-standardized value range together with the 5G QoS characteristics are signalled over N2, N11 and N7 at the time of PDU session or QoS flow establishment and in addition, over N2 every time the User Plane of the PDU session is activated.

NOTE 1:	On N3, each PDU (i.e. in the tunnel used for the PDU session) is associated with one 5QI via the QFI carried in the encapsulation header.

The QoS parameter ARP contains information about the priority level, the pre-emption capability and the pre-emption vulnerability. The priority level defines the relative importance of a resource request. This allows deciding whether a new QoS flow may be accepted or needs to be rejected in case of resource limitations (typically used for admission control of GBR traffic). It may also be used to decide which existing QoS flow to pre-empt during resource limitations.

The range of the ARP priority level is 1 to 15 with 1 as the highest level of priority. The pre-emption capability information defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level. The pre-emption vulnerability information defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level. The pre-emption capability and the pre-emption vulnerability shall be either set to 'yes' or 'no'.

The ARP priority levels 1-8 should only be assigned to resources for services that are authorized to receive prioritized treatment within an operator domain (i.e. that are authorized by the serving network). The ARP priority levels 9-15 may be assigned to resources that are authorized by the home network and thus applicable when a UE is roaming.

NOTE 2:	This ensures that future releases may use ARP priority level 1-8 to indicate e.g. emergency and other priority services within an operator domain in a backward compatible manner. This does not prevent the use of ARP priority level 1-8 in roaming situation in case appropriate roaming agreements exist that ensure a compatible use of these priority levels.

The Reflective QoS Attribute (RQA) is an optional parameter that, in the case of NG-RAN, may be signalled to RAN via N2 when Reflective QoS control is used, as described in clause 5.7.5.4 The RQA indicates that certain traffic on this QoS flow may be subject to Reflective QoS.

In addition, the QoS flow may be associated with the parameter:

-	Notification control.

The Notification control may be provided for GBR QoS flows. The Notification control indicates whether notification should be made by the RAN if the QoS targets cannot be fulfilled for a QoS flow during the lifetime of the QoS flow. If it is set and QoS targets cannot be fulfilled, RAN sends a notification towards SMF.

Editor's note:	It is FFS whether the Notification control is needed for non GBR QoS flows.

For GBR QoS flows, the 5G QoS profile additionally include the following QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL;

-	Maximum Flow Bit Rate (MFBR) -- UL and DL.

The GFBR denotes the bit rate that may be expected to be provided by a GBR QoS flow. The MFBR limits the bit rate that may be expected to be provided by a GBR QoS flow (e.g. excess traffic may get discarded by a rate shaping function).

GFBR and MFBR are signalled on N2 and N11 for each of the GBR QoS Flows for setting up the 5G QoS profile.

The MBR (and GBR, if applicable) per SDF, based on the information received from PCF, is signalled on N7 and N4.

Each PDU Session of a UE is associated with the following aggregate rate limit QoS parameter:

-	per Session Aggregate Maximum Bit Rate (Session-AMBR).

The subscribed Session-AMBR is a subscription parameter which is retrieved by the SMF from UDM. SMF may use the subscribed Session-AMBR or modify it based on local policy or use the authorized Session-AMBR received from PCF to get the Session-AMBR , which is signalled to the appropriate UPF entity/ies (and to the (R)AN to enable the calculation of the UE-AMBR). The Session-AMBR limits the aggregate bit rate that can be expected to be provided across all Non-GBR QoS flows for a specific PDU session.

Each UE is associated with the following aggregate rate limit QoS parameter:

-	per UE Aggregate Maximum Bit Rate (UE-AMBR).

The UE-AMBR limits the aggregate bit rate that can be expected to be provided across all Non-GBR QoS flows of a UE. Each (R)AN shall set its UE-AMBR to the sum of the Session-AMBR of all PDU Sessions with active user plane to this (R)AN up to the value of the subscribed UE-AMBR. The subscribed UE-AMBR is a subscription parameter which is retrieved from UDM and provided to the (R)AN by the AMF.

For each PDU Session, SMF retrieves the default 5QI/ARP from UDM. The default 5QI/ARP is used by SMF to set QoS parameters for the QoS flow corresponding to the default QoS rule. The SMF may change the default 5QI/ARP based on local configuration or interaction with the PCF (to retrieve the authorized default 5QI/ARP which overrides the default 5QI/ARP).

The SMF sends some of QoS parameters to the UE. During the PDU session establishment procedure, the SMF provides Session-AMBR. Other QoS parameters required by the UE (e.g. 5QI, GFBR and MFBR) is provided to the UE when relevant QoS flow is established or modified.

NOTE 3:	In case of non-standardized 5QI, the SMF also provides related QoS characteristics defined by the operator.