SA WG2 Meeting #123	S2-177306

October 23 – 27, 2017, Ljubljana, Slovenia	(revision of S2-17xxxx)

Source:	ETRI

Title:	Updating a PDU session handover procedure between 3GPP and non-3GPP access

Document for:	Approval

Agenda Item:	6.5.10

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes to update a PDU session handover procedure between 3GPP and untrusted non-3GPP access in TS 23.502.

Discussion

It is proposed to update the section 4.9.2 Handover of a PDU session procedure between 3GPP and untrusted non-3GPP access in TS 23.502 to consider the case of the UE in CM-IDLE state over the target access during the handover.

Proposal

It is proposed to update the following texts into TS 23.502.

* * * Start of changes* * * *

4.9.2	Handover of a PDU Session procedure between 3GPP and untrusted non-3GPP access

4.9.2.1	Handover of a PDU Session procedure from untrusted non-3GPP to 3GPP access (non-roaming and roaming with local breakout)

This clause specifies how a UE can handover a PDU Session from untrusted non-3GPP access to 3GPP access. It is based on the PDU Session establishment procedure for 3GPP access as specified in clause 4.3.2.

Figure 4.9.2.1-1: Handover of a PDU Session procedure from untrusted non-3GPP access to 3GPP access (non-roaming and roaming with local breakout)

NOTE:	Step 2 includes release procedure to release the user-plane over the untrusted non-3GPP access.

1.	If the UE is not registered via 3GPP access, the UE shall initiate registration procedure as defined in clause 4.2.2.2.2.

2.	The UE performs a PDU Session Establishment procedure as specified clause 4.3.2.2.1 in the case of UE in CM-CONNECTED state over 3GPP access. The UE may perform a UE triggered Service Request procedure as specified clause 4.2.3.2 in the case of UE in CM-IDLE state over 3GPP access within the same PLMN.

4.9.2.2	Handover of a PDU Session procedure from 3GPP to untrusted non-3GPP access (non-roaming and roaming with local breakout)

This clause specifies how a UE can handover a PDU Session from 3GPP access to untrusted non-3GPP access. It is based on the PDU Session establishment procedure for non-3GPP access as specified in clause 4.12.5.

Figure 4.9.2.2-1: Handover of a PDU Session from 3GPP access to untrusted non-3GPP access (non-roaming and roaming with local breakout)

NOTE:	Step 2 includes release procedure to release the user-plane over the 3GPP access.

1.	If the UE is not registered via untrusted non-3GPP access, the UE shall initiate registration procedure as defined in clause 4.12.2.

2.	The UE performs PDU Session Establishment procedure as specified in clause 4.12.5 in the case of UE in CM-CONNECTED state over non-3GPP access. If the UE is in CM-IDLE state over non-3GPP access, this step is not performed.

Editor's note:	The clause 4.12.5 should be updated to describe handover procedure.

* * * End of Changes * * *