SA WG2 Meeting #123 meeting	S2-177475

23 – 27 November 2017, Ljubljana	(was S2-17----)

Source:	CATT

Title:	The Editorial Change to Emergency Services

Document for:	Approval

Agenda Item:	6.5.8

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to change the text of TS 23.501 to avoid misunderstanding the content of Emergency Services.

- Discussion

In clause 5.16.4.10, 3GPP TS 23.501 v1.4.0, “Mobility Management” should be changed into “Registration Management” in alignment with the context and terminology in 5GC.

- Proposal

The following changes are proposed to TS 23.501 v1.4.0.

*************** Start of change*********************

5.16.4.10	Support of eCall Only Mode

For service requirements for eCall only mode, refer to TS 22.101 [33].

A UE configured for eCall Only Mode shall remain in RM-DEREGISTERED state, shall camp on a network cell when available but shall refrain from any Registration Management, Connection Management or other signalling with the network. The UE may instigate Registration Management and Connection Management procedures in order to establish, maintain and release an eCall Over IMS session or a session to any non-emergency MSISDN(s) or URI(s) configured in the UICC for test and/or terminal reconfiguration services. Following the release of either session, the UE starts a timer whose value depends on the type of session (i.e. whether eCall or a session to a non-emergency MSISDN or URI for test/reconfiguration). While the timer is running, the UE shall perform normal RM/CM procedures and is permitted to respond to paging to accept and establish an incoming session (e.g. from an emergency centre, PSAP or HPLMN operator). When the timer expires, the UE shall perform a UE-initiated deregistration procedure if still registered and enter RM-DEREGISTERED state.

NOTE 1:	An HPLMN operator can change the eCall Only Mode configuration state of a UE in the UICC. An HPLMN operator can also instead add, modify or remove a non-emergency MSISDN or URI in the UICC for test and/or terminal reconfiguration services. This can occur following a UE call to a non-emergency MSISDN or URI configured for reconfiguration. When the eCall Only Mode configuration is removed, the UE operates as a normal UE that can support eCall over IMS.

NOTE 2:	A test call and a reconfiguration call can be seen as normal (non-emergency) call by a serving PLMN and normal charging rules can apply depending on operator policy.

NOTE 3:	An MSISDN configured in the UICC for test and/or terminal reconfiguration services for eCall Over IMS can differ from an MSISDN configured in the UICC for test services for eCall over the CS domain.

*************** End of change*********************