SA WG2 Meeting #123	S2-177811

October 23 – 27, 2017, Ljubljana, Slovenia	(was S2-17----)

Source:	China Mobile, AT&T, Deutsche Telecom, KT, SK Telecom, Sprint, Telecom Italia, T-Mobile USA, Verizon

Title:	23.501 update the NRF service table

Document for:	Discussion and Approval

Agenda Item:	6.5.11

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: This contribution update the NRF service table in 23.501 to align with NRF services identified in 23.502.

- Introduction

In SA2#122BIS two more NRF services are agreed and documented in 23.502, i.e., Nnrf_NFStatus and Nnrf_NFDiscovery. However these two services has not been documented in 23.501 yet, making the two TS not aligned.

CT already start TS work where services exhibits through each SBI are defined. Therefore, a clear 23.501/23.502 text are important for CT to start their work.

In SA2#122, SA2#122bis and SA2#122E, there is argument that NF Discovery not a separate service but an operation of NF_Management. While, the NF Management provides consumers with registration/deregistration service. That may modify the NF/NF service information in 23.501. However, NF Discovery is only a READ operation thus can be separated.

- Proposal

This proposal updates the NRF service table of 23.501 v1.4.0.

* * * First Change * * *

7.2.6	NRF Services

The following NF services are specified for NRF:

Table 7.2.6-1: NF Services provided by NRF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nnrf_NFManagement | Provides support for register, deregister and update service to NF/NF services. Provide consumers with notifications of newly registered NF along with its NF services. | 5.2.7.2 |
| Nnrf_NFDiscovery | Enables one NF service consumer to discover a set of NF instances with specific NF service or a target NF type. Also enables one NF service to discover a specific NF service. | 5.2.7.3 |

* * * End of Changes * * *