SA WG2 Meeting #123	S2-177819

October 23 – 27, 2017, Ljubljana, Slovenia	(was S2-177325)

Source:	Cisco

Title:	23.501: Time zone change reporting from SMF to PCF

Document for:	Discussion

Agenda Item:	6.5.3

Work Item / Release:	5G_ph1 / Rel-15

Abstract of the contribution: Clarifies that SMF provides info to PCF. Remove possible mis-interpretation that SMF may expose ULI and Time-zone as events, which should only be done as an event by AMF.

## 4. Proposed changes to specification

TS 23.501

************************* 23.501 START CHANGE **********************

### 5.6.2	Interaction between AMF and SMF

The AMF and SMF are separate Network Functions.

N1 related interaction with SMF is as follows:

-	The single N1 termination point is located in AMF. The AMF forwards SM related NAS information to the SMF based on the PDU session ID in the NAS message. Further SM NAS exchanges (e.g. SM NAS message responses) for N1 NAS signalling received by the AMF over an access (e.g. 3GPP access or non-3GPP access) are transported over the same access.

-	The serving PLMN ensures that for N1 NAS signalling received by the AMF over an access (e.g. 3GPP access or non-3GPP access) further SM NAS exchanges (e.g. SM NAS message responses) are transported over the same access.

-	SMF handles the Session management part of NAS signalling exchanged with the UE.

-	The UE may request registration and PDU session establishment at the same time. Otherwise, the UE shall only initiate PDU session establishment in RM-REGISTERED state.

-	When a SMF has been selected to serve a specific PDU session, AMF has to ensure that all NAS signalling related with this PDU session is handled by the same SMF instance.

-	Upon successful PDU session establishment, the SMF stores the Access Type that the PDU session is associated.

N11 related interaction with SMF is as follows:

-	The AMF reports the reachability of the UE based on a subscription from the SMF, including:

-	the UE location with respect to the area of interest indicated by the SMF (e.g. the LADN service area).

-	The SMF indicates to AMF when a PDU session has been released.

-	Upon successful PDU session establishment, AMF stores the identification of serving SMF of UE and SMF stores the identification of serving AMF of UE including the AMF set. When trying to reach the AMF serving the UE, the SMF may need to apply the behaviour described for "the other CP NFs" in clause 5.21.

N2 related interaction with SMF is as follows:

-	Some N2 signalling (such as Handover related signalling) may require the action of both AMF and SMF. In such case, the AMF is responsible to ensure the coordination between AMF and SMF. The AMF may forward the SM N2 signalling towards the corresponding SMF based on the PDU session ID in N2 signalling.

N3 related interaction with SMF is as follows:

-	In case of UE having multiple established PDU sessions using multiple UPFs, the SMF supports the independent activation of UE-CN user plane connection per PDU session.

N4 related interaction with SMF is as follows:

-	The SMF(s) supports the end-to-end control functions on PDU sessions (including any N4 interface to control the UPF(s)).

-	When it is made aware by the UPF that some DL data has arrived for a UE without downlink N3 tunnel information, the SMF interacts with the AMF to initiate Network Triggered Service Request procedure. In this case, if the SMF is aware that the UE is unreachable or if the UE is reachable only for regulatory prioritized service and the PDU session is not for regulatory prioritized service, then the SMF shall not inform DL data notification to the AMF

In order to support charging data collection and to fulfill regulatory requirement (in order to provide NPLI - Network Provided Location Information- as defined in TS 23.228 [15]) related with with the set-up, modification and release of IMS Voice calls or with SMS transfer  the following applies

-	At the time of the establishment of a PDU session, the AMF provides the SMF with the PEI of the UE if the PEI is available at the AMF.

-	When it forwards UL NAS or N2 signalling to a peer NF (e.g. to SMF or to SMSF) or during the activation of  the user Plane of a PDU session, the AMF provides any User Location Information it has received from the 5G AN as well as the Access Type (3GPP – Non 3GPP) of the AN over which it has received the UL NAS or N2 signalling. The AMF provides also the corresponding UE Time Zone.

The User Location Information, the the access type  and the UE Time Zone may be further provided by SMF to PCF. The PCF  may get this information  in order to provide NPLI to applications (such as IMS) that have requested it.

-	The  User Location Information may correspond to

-	in case of a NG-RAN: a Cell-Id.

-	in case of a N3IWF: an UE local IP address (used to reach the N3IWF) and optionally UDP or TCP source port number (if NAT is detected).

The AMF is responsible of selecting the SMF per procedures described in clause 6.3.2. For this purpose, it gets subscription data from the UDM that are defined in that clause. Furthermore, it retrieves the subscribed UE-AMBR from the UDM to send it to the (R)AN as defined in clause 5.7.2

AMF-SMF interactions to support LADN are defined in clause 5.6.5.

************************* END CHANGE **********************