SA WG2 Meeting #123	S2-176961

October 23 – 27, 2017, Ljubljana, Slovenia

Source:	OPPO

Title:	CN Assisted Information to RAN Paging Occasion Calculation

Document for:	Approval

Agenda Item:	6.5.7.5

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution involves that UE identifier should be provided to RAN for purpose of RAN paging occasion calculation.

- Discussion

In Inactive state, when downlink data arrives, RAN will send RAN paging. According to RAN2 discussion, the paging occasion for CN and RAN overlaps and same mechanism is used, RAN will calculate RAN paging occasion based on UE identifier.

“ The UE in RRC_IDLE and RRC_INACTIVE states may use DRX in order to reduce power consumption. While in RRC_IDLE the UE monitors 5GC-initiated paging, in RRC_INACTIVE the UE is reachable via RAN-initiated paging and 5GC-initiated paging. RAN and 5GC paging occasions overlap and same paging mechanism is used. The UE monitors one paging occasion per DRX cycle for the reception of paging as follows:

-	Paging DRX cycle length is configurable:

-	A default DRX cycle for CN paging is configurable via system information;

-	A UE specific DRX cycle for CN paging is configurable via UE dedicated signalling;

-	NG-RAN can configure a UE with a DRX cycle for RAN paging. This configuration can be UE specific.

-	The number of paging occasions in a DRX cycle is configurable via system information:

-	A network may distribute UEs to the paging occasions based on UE id when multiple paging occasions are configured in the DRX cycle.

-	Paging occasion can consist of multiple time slots (e.g. subframe or OFDM symbol). The number of time slots in a paging occasion is configurable via system information:

-     A network may transmit a paging using a different set of DL Tx beam(s) or repetitions in each time slot.”

For CN paging, when CN sends paging request to RAN, the UE identifier, e.g. IMSI will be included in the paging request message for RAN to calculate the paging occasions.

For RAN paging, no UE identifier will be provided to RAN when downlink data arrives to RAN, therefore, CN needs to provide the UE identifier to the RAN in the "RRC Inactive assistance information" during N2 activation with the (new) serving NG-RAN node.

Proposal: Include UE identifier in "RRC Inactive assistance information" to assist RAN calculating paging occasions.

2	Proposal

It is proposed to agree on the proposed changes to TS 23.501.

*************** Start of changes *********************

##### 5.3.3.2.5	CM-CONNECTED with RRC Inactive state

RRC Inactive state applies to NG-RAN.

The AMF, based on network configuration may provide assistance information to the NG-RAN, to assist the NG-RAN's decision whether the UE can be sent to RRC Inactive state.

Editor's note: It is FFS if the UE provides indication of support for RRC inactive state on NAS or AS layer.

The "RRC Inactive assistance information" includes:

-	UE specific DRX values.

-	the Registration Area provided to the UE;

-	Periodic Registration Update timer

-	If the AMF has enabled MICO mode for the UE, an indication that the UE is in MICO mode.

-	UE identifier (e.g. SUPI) for RAN to calculate RAN paging occasions.

The RRC Inactive assistance information mentioned above is provided by the AMF during N2 activation with the (new) serving NG-RAN node (i.e. during Registration, service request, handover) to assist the NG RAN's decision whether the UE can be sent to RRC Inactive state. RRC Inactive state is part of RRC state machine, and it is up to the RAN to determine the conditions to enter RRC Inactive state. If any of the parameters included in the RRC Inactive Assistance Information changes as the result of NAS procedure, the AMF shall update the RRC Inactive Assistance Information to the NG-RAN node.

When the UE is in CM-CONNECTED state, if the AMF has provided RRC Inactive assistance information, the RAN node may decide to move a UE to CM-CONNECTED with RRC Inactive state.

The state of the N2 and N3 reference points are not changed by the UE entering CM-CONNECTED with RRC Inactive state. A UE in RRC inactive state is aware of the RAN Notification area.

The 5GC network is not aware of the UE transitions between CM-CONNECTED with RRC Connected and CM-CONNECTED with RRC Inactive state, unless the 5GC network is notified via N2 notification procedure in TS 23.502 [3] clause 4.8.3.

At transition into CM-CONNECTED with RRC Inactive state, the UE applies with a periodic RAN Notification Area Update and the timer is restarted in the UE and in the RAN as defined in TS 38.300 [27].

When assigning the Periodic RAN Notification Area Update timer, the NG-RAN shall take into account the value of the Periodic Registration Update timer value indicated in the RRC Inactive Assistance Information.

If the periodic RAN Notification Area Update timer expires in RAN, the RAN shall start an RRC release timer. If the RRC release timer expires, the RAN shall initiate UE context release in the AN procedure as specified in TS 23.502 [3].

When the UE is in CM- CONNECTED with RRC inactive state, the UE performs PLMN selection procedures as defined in TS 23.122 [17] for CM-IDLE.

When the UE is CM-CONNECTED with RRC Inactive state, the UE may resume the RRC connection due to:

-	Uplink data pending;

-	Mobile initiated NAS signalling procedure;

-	As a response to RAN paging;

-	Notifying the network that it has left the RAN Notification area;

-	Upon periodic RAN update timer expiration.

If the UE resumes the connection in a different NG-RAN node within the same PLMN, the UE AS context is retrieved from the old NG-RAN node and a procedure is triggered towards the CN (see TS 23.502 [3]).

If the RAN paging procedure, as defined in TS 38.300 [27], is not successful in establishing contact with the UE the procedure shall be handled by the network as follows:

-	If NG-RAN has a pending NAS PDU for transmission, the RAN node shall initiate the NAS signaling connection release procedure (see TS 23.502 [3]) to move the UE CM state in the AMF to CM-IDLE state and indicate to the AMF the NAS non-delivery.

-	If NG RAN has only pending user plane data for transmission, the NG-RAN node may keep the N2 connection active or initiate the NAS signaling connection release procedure (see TS 23.502 [3]) based on local configuration in NG-RAN.

Editor's note: Whether and how access control/barring applies for UE in CM-CONNECTED with RRC Inactive state is FFS.

If a UE in CM-CONNECTED with RRC Inactive state performs cell selection to GERAN/UTRAN/EPS, it shall follow idle mode procedures.

In addition, a UE in CM-CONNECTED state with RRC Inactive state shall enter CM-IDLE mode and follow relevant NAS procedure in the following cases:

-	If RRC resume procedure fails,

If the UE receives Core Network paging,

-	If the periodic RAN Notification Area Update timer expires and the UE cannot successfully resume the RRC connection.

-	in any other failure scenario that cannot be resolved in RRC Inactive mode and requires the UE to move to CM-IDLE mode.

When UE is in CM-CONNECTED with RRC Inactive state, if RAN has received Location Reporting Control message from AMF with the reporting type indicating single stand-alone report, the RAN shall perform RAN paging before reporting the location to AMF.

When UE is in CM-CONNECTED with RRC Inactive state, if RAN has received Location Reporting Control message from AMF with the reporting type indicating continuously reporting whenever the UE changes cell, the RAN shall send a Location Report message to AMF including UE's last known location with time stamp.

*************** End of changes *********************