SA WG2 Meeting #123	S2-177311

October 23 – 27, 2017, Ljubljana, Slovenia	(was S2-17----)

Source:	CATT

Title:	TS 23.501: Correcting the Note3 in clause 5.6.1

Document for:	Discussion / Approval

Agenda Item:	6.5.7.1

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to correct the NOTE3 in the clause 5.6.1.

- Discussion

It is agreed that, if a UE moves from a normal service area to a mobility restricted area, the AMF only notifies the SMF that the UE is only reachable for regulatory priority service. Therefore the SMF can not know whether a UE moves into a non-allowed area, a forbidden area, or else.

With this consideration, it is proposed to correct the NOTE 3 in the clause 5.6.1.

2.	Proposal

It is propose to include the following changes into the TS 23.501.

* * * * First Change * * * *

5.6.1	Overview

The 5GC supports a PDU Connectivity Service i.e. a service that provides exchange of PDUs between a UE and a data network identified by a DNN. The PDU Connectivity Service is supported via PDU sessions that are established upon request from the UE.

Subscription Information may include multiple DNNs and may contain a Default DNN. The UE is assigned to a default DNN if it does not provide a valid DNN in a PDU Session Establishment Request sent to the network.

Each PDU session supports a single PDU session type i.e. supports the exchange of a single type of PDU requested by the UE at the establishment of the PDU session. The following PDU session types are defined: IPv4, IPv6, Ethernet, Unstructured.

NOTE 1:	In this release the 5GC does not support dual stack PDU Session (PDU Session type IPv4v6): The 5GC supports dual Stack UEs by using separate PDU sessions for IPv4 and IPv6.

PDU sessions are established (upon UE request), modified (upon UE and 5GC request) and released (upon UE and 5GC request) using NAS SM signalling exchanged over N1 between the UE and the SMF. Upon request from an Application Server, the 5GC is able to trigger a specific application in the UE. When receiving that trigger message, the UE shall pass it to the identified application in the UE. The identified application in the UE may establish a PDU Session to a specific DNN, see clause 4.4.5.

SMF may support PDU sessions for LADN where the access to a DN is only available in a specific LADN service area. This is further defined in clause 5.6.5.

The SMF is responsible of checking whether the UE requests are compliant with the user subscription. For this purpose, it retrieves SMF level subscription data from the UDM. Such data may indicate per DNN and, if applicable, per S-NSSAI:

-	The allowed PDU session Types and the default PDU session Type.

-	The allowed SSC modes and the default SSC mode.

-	QoS Information (refer to clause 5.7):  the subscribed Session-AMBR, Default 5QI and Default ARP

An UE that is registered over multiple accesses chooses over which access to establish a PDU session. As defined in TS 23.503 [45], the HPLMN may send policies to the UE to guide the UE selection of the access over which to establish a PDU session.

NOTE 2:	In this release, at a given time, a PDU session is routed over only a single access network.

An UE may request to move a PDU session between 3GPP and Non 3GPP accesses. The decision to move PDU sessions between 3GPP access and Non 3GPP access is made on a per PDU session basis, i.e. the UE may, at a given time, have some PDU sessions using 3GPP access while other PDU sessions are using Non 3GPP access.

In a PDU session establishment request sent to the network, the UE shall provide a PDU Session Identifier. PDU session ID is unique per UE and is the identifier used to uniquely identify one of an UE's PDU sessions. PDU session ID shall be stored in the UDM to support handover between 3GPP and non-3GPP access when different PLMNs are used for the two accesses. The UE may also provide:

-	A PDU session Type.

-	S-NSSAI.

-	The DNN (Data Network Name).

-	The SSC mode (Service and Session Continuity mode defined in clause 5.6.9.2).

# Table 5.6.1-1: Attributes of a PDU session

| PDU session attribute | May be modified later during the lifetime of the PDU session | Notes |
| --- | --- | --- |
| S-NSSAI | No | (Note 1)(Note 2) |
| DNN (Data Network Name) | No | (Note 1)(Note 2) |
| PDU session Type | No | (Note 1) |
| SSC mode | No | (Note 1) The semantics of Service and Session Continuity mode is defined in clause 5.6.9.2 |
| PDU session Id | No |  |
| NOTE 1: If it is not provided by the UE, the network determines the parameter based on default information received in user subscription. Subscription to different DNN(s) may correspond to different default SSC modes and different default PDU session Types NOTE 2: S-NSSAI and DNN are used by AMF to select a SMF to handle a new session. Refer to clause 6.3.2. |  |  |

An UE may establish multiple PDU sessions, to the same data network or to different data networks, via 3GPP and via and Non-3GPP access networks at the same time.

An UE may establish multiple PDU sessions to the same Data Network and served by different UPF terminating N6.

A UE with multiple established PDU sessions may be served by different SMF.

The user plane paths of different PDU Sessions (to the same or to different DNN) belonging to the same UE may be completely disjoint between the AN and the UPF interfacing with the DN.

NOTE 3:	User Plane resources for PDU sessions of a UE, except for regulatory prioritized service like Emergency Services and MPS, can be deactivated by the SMF if the UE is only reachable for regulatory prioritized services.

NOTE 4:	The handling if the UE goes out of the SMF service area is not specified in this release.

* * * * End of Changes * * * *