SA WG2 Meeting #123	S2-177367

23 - 27 October 2017, Ljubljana, Slovenia	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	TS 23.502: Providing PDU session type to the RAN during PDU session establishment procedure

Document for:	Approval

Agenda Item:	6.5.3 Session management and continuity functions and flows

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution is proposed to provide PDU session type information to the RAN during PDU session establishment procedure. This contribution can be handled during email approval.

Discussion

When an Ethernet or Unstructured PDU session is established, the RAN should not perform IP header compression. This is already captured in RAN3 specification TS 38.413 as follows:

8.2.1	PDU Session Resource Setup

8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU Session Resources and the corresponding QoS flows, and to setup corresponding Data Radio Bearers for a given UE. The procedure uses UE-associated signalling.

︙

For each PDU session for which the PDU Session Type IE is included in the PDU SESSION RESOURCE SETUP REQUEST message and is not set to “IPv4” nor “IPv6” [values FFS], the NG-RAN node shall not perform header compression for the concerned PDU session.

︙

In order to support this behaviour PDU session type needs to be sent to the RAN during the PDU session establishment procedure.

Proposal

* * * * Start of 1st Change * * * *

4.3.2.2.1	Non-roaming and Roaming with Local Breakout

This clause specifies PDU Session establishment in the non-roaming and roaming with local breakout cases. The procedure is used to establish a new PDU Session as well as to hand over an existing PDU Session between 3GPP access and non-3GPP access.

In case of roaming, the AMF determines if a PDU Session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the AMF, the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI, DNN, PDU Session ID, Request type, N1 SM container (PDU Session Establishment Request)).

In order to establish a new PDU Session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM container. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The Request Type indicates "Initial request" if the PDU Session Establishment is a request to establish a new PDU Session and indicates "Existing PDU Session" if the request refers to an existing PDU Session between 3GPP access and non-3GPP access. The Request Type indicates "Emergency Request" if the PDU Session Establishment is a request to establish a PDU Session for emergency services. The Request Type indicates "Existing Emergency PDU Session" if the request refers to an existing PDU Session for emergency services between 3GPP access and non-3GPP access.

The NAS message sent by the UE is encapsulated by the AN in a N2 message towards the AMF that should include User location information and Access Technology Type Information.

The PDU Session Establishment Request message may contain SM PDU DN Request Container containing information for the PDU Session authorization by the external DN.

The AMF receives from the AN the NAS SM message (built in step 1) together with User Location Information (e.g. Cell Id in case of the RAN).

The UE shall not trigger a PDU Session establishment for a PDU Session corresponding to a LADN when the UE is outside the area of availability of the LADN.

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on that Request Type indicates "initial request" and that the PDU Session ID is not used for any existing PDU Session(s) of the UE. If the NAS message does not contain an S-NSSAI, the AMF may determine a default S-NSSAI for the requested PDU Session either according to the UE subscription, if it contains only one default S-NSSAI, or based on operator policy. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. The AMF stores an association of the PDU Session ID and the SMF ID.

The case where the Request Type indicates "Existing PDU Session", and either the AMF does not recognize the PDU Session ID or the subscription context from UDM does not contain an SMF ID corresponding to the DNN constitutes an error case.

If the Request Type indicates "Emergency Request" or "Existing Emergency PDU Session", the AMF selects the SMF as described in TS 23.501 [2], clause 5.16.4.

3.	From AMF to SMF: Nsmf_PDUSession_CreateSMRequest (SUPI, DNN, S-NSSAI, PDU Session ID, AMF ID, Request Type, N1 SM container (PDU Session Establishment Request), User location information, Access Type, PEI).

The AMF ID uniquely identifies the AMF serving the UE. The AMF forwards the PDU Session ID together with the N1 SM container containing the PDU Session Establishment Request received from the UE. The PEI is defined in TS 23.501 [2] clause 5.9.3.

4.	If the SMF has not yet registered and subscription data is not available, then the SMF registers with the UDM, retrieves subscription data and subscribes to be notified when subscription data is modified.

Editor's note: Whether registration and subscription data retrieval happen in 1 or 2 steps and the use of corresponding service operations is FFS. Whether indicating "subscriber data retrieval" in the Nudm_UEContext_Management service is possible is FFS.

If the Request Type in step in step 3 indicates "Existing PDU Session" the SMF determines that the request is due to handover between 3GPP access and non-3GPP access. The SMF identifies the existing PDU Session based on the PDU Session ID.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), default 5QI/ARP, subscribed Session-AMBR.

The SMF checks the validity of the UE request: it checks

-	whether the UE request is compliant with the user subscription and with local policies;

-	(If the DNN corresponds to an LADN), whether the UE is located within the LADN service area based on the UE location reporting from the AMF.

If the UE request is considered as not valid, the SMF decides to not accept to establish the PDU Session.

When the SMF decides to not accept to establish a PDU Session, the SMF rejects the UE request via NAS SM signalling including a relevant SM rejection cause by responding to the AMF with Nsmf_PDUSession_CreateSMResponse. The SMF also indicates to the AMF that the PDU Session ID is to be considered as released, deregisters from UDM for this PDU Session and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs perform secondary authorization/authentication during the establishment of the PDU Session by a DN-AAA server as described in TS 23.501 [2] clause 5.6.6, the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU Session establishment authentication/authorization as described in clause 4.3.2.3.

If the PDU Session establishment authentication/authorization fails, the SMF behaves as described in step 4 when the SMF decides to not establish a PDU Session.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection. If dynamic PCC is not deployed, the SMF may apply local policy.

6b.	The SMF may invoke the Npcf_SMPolicyControl_Get operation (see clause 5.2.5.4.2) to establish a PDU Session with the PCF and get the default PCC Rules for the PDU Session. If the Request Type in step 3 indicates "Existing PDU Session", the SMF may notify an event previously subscribed by the PCF by invoking the Nsmf_EventExposure_Notify operation and the PCF may update policy information in the SMF by invoking Npcf_SMPolicyControl_UpdateNotify operation (see clause 5.2.5.5.3 for details). The PCF may provide authorized Session-AMBR and the authorized 5QI/ARP to SMF.

6c.	The PCF subscribes to the IP allocation/release event in the SMF (and may subscribe other events) by invoking the Nsmf_EventExposure_Subscribe operation.

NOTE 1:	The purpose of step 6 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 6 can be skipped.

7.	If the Request Type in step 3 indicates "Initial request", the SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1. For Unstructured PDU Type the SMF may allocate an IPv6 prefix for the PDU Session and N6 point-to-point tunnelling (based on UDP/IPv6) as described in TS 23.501 [2] clause 5.6.10.3.

If the Request Type indicates "Emergency Request" or "Existing Emergency PDU Session", the SMF selects the UPF as described in TS 23.501 [2], clause 5.16.4.

8a.	SMF may invoke the Nsmf_EventExposure_Notify service operation to report some event to the PCF that has previously subscribed. If Request Type is "initial request" and dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF notifies the PCF (that has previously subscribed) with the allocated UE IP address/prefix.

8b.	PCF may provide updated policies to the SMF by invoking the Npcf_SMPolicyControl_UpdateNotify service operation (see clause 5.2.5.5.3). The PCF may provide authorized Session-AMBR and the authorized 5QI/ARP to SMF.

9.	If Request Type indicates "initial request" and step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If CN Tunnel Info is allocated by the UPF, the CN Tunnel Info is provided to SMF in this step.

10.	SMF to AMF: Nsmf_PDUSession_CreateSM Response (Cause, N2 SM information (PDU Session ID, QoS Profile(s), CN Tunnel Info, S-NSSAI, Session-AMBR, PDU session type), N1 SM container (PDU Session Establishment Accept (QoS Rule, SSC mode, S-NSSAI, allocated IPv4 address, Session-AMBR, PDU session type))).

The N2 SM information carries information that the AMF shall forward to the (R)AN which includes:

-	The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU Session.

-	The QoS Profile provides the (R)AN with the mapping between QoS parameters and QoS Flow Identifiers. Multiple QoS profiles can be provided to the (R)AN. This is further described in TS 23.501 [2] clause 5.7.

-	The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU Session for the UE.

S-NSSAI is corresponding to the PDU Session.

The N1 SM container contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM information and in the N2 SM container.

The SM Response further contains PDU Session ID and information allowing the AMF to know which the target UE as well to determine which access towards the UE to use.

NOTE 2:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

The SMF shall subscribe to the UE location reporting in case of successful PDU Session establishment providing reporting criteria (e.g. UE location with respect to the LADN service area).

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, NAS message (PDU Session ID, N1 SM container (PDU Session Establishment Accept))).

The AMF sends the NAS message containing PDU Session ID and PDU Session Establishment Accept targeted to the UE and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN N3 tunnel information for the PDU Session. In case of Dual Connectivity, the Master RAN node may assign some (zero or more) QFIs to be setup to a Master RAN node and others to Secondary RAN node. The RAN tunnel information includes a tunnel endpoint for each involved RAN node, and the QFIs assigned to each tunnel endpoint. A QFI can be assigned to either the Master RAN node or the Secondary RAN node and not to both.

(R)AN forwards the NAS message (PDU Session ID, N1 SM container (PDU Session Establishment Accept)) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Response (PDU Session ID, Cause, N2 SM information (PDU Session ID, (R)AN Tunnel Info, List of accepted/rejected QoS profile(s))).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU Session.

14.	AMF to SMF: Nsmf_PDUSession_UpdateSMContext Request (N2 SM information).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

15a.	If the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and CN Tunnel Info. The CN Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

NOTE 3:	If the PDU Session Establishment Request was due to mobility between 3GPP and non-3GPP access, the downlink data path is switched towards the target access in this step.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

16.	SMF to AMF: Nsmf_PDUSession_UpdateSMContext Response (Cause).

After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

18.	If the PDU Session Establishment request is due to a handover between 3GPP access and non-3GPP access, i.e. the Request type is set to "existing PDU Session" the SMF executes the steps 2a-9 specified in figure 4.3.4.2-1 in order to release the user-plane over the source access (3GPP or non-3GPP access).

19.	If the SMF identity is not included in the DNN subscription context, either stored already in SMF if it was previously registered or provided in steps 4a-4b by the UDM, the SMF invokes the Nudm_UEContextManagement_Update service operation including the SMF address, the DNN and the PDU Session Id. The UDM stores the SMF identity, SMF address and the associated DNN and PDU Session Id.

If during the procedure the PDU Session establishment is not successful, the SMF informs the AMF. Once the AMF associates an SMF with a PDU Session ID, the SMF is automatically subscribed to notifications of incoming N1 signalling associated with this PDU Session ID. These notifications automatically provide any User Location information and Access Type that the AMF has received from the (R)AN in association with N1 signalling.

* * * * Start of 2nd Change * * * *

4.3.2.2.2	Home-routed Roaming

This procedure is used in case of home-routed roaming scenarios.

Figure 4.3.2.2.2-1: UE-requested PDU Session Establishment for home-routed roaming scenarios

1.	This step is the same as step 1 in clause 4.3.2.2.1.

2.	As in step 2 of clause 4.3.2.2.1 with the addition that the AMF also selects a SMF in HPLMN. The AMF stores the association of the PDU Session ID and the SMF ID in VPLMN.

3.	As in step 3 of clause 4.3.2.2.1 with the addition that the AMF also provides the identity of the SMF in HPLMN it has selected in step 2 The AMF ID uniquely identifies the AMF serving the UE. The H-SMF is provided when the PDU Session is home-routed. The N1 SM container contains the PDU Session Establishment Request received from the UE.

Editor's note:	The parameter S-NSSAI may need to be updated to reflect agreements regarding slice selection parameters. This is FFS.

4.	The V-SMF selects a UPF in VPLMN as described in TS 23.501 [2], clause 6.3.3.

5.	The V-SMF initiates an N4 Session Establishment procedure with the selected V-UPF:

5a.	The V-SMF sends an N4 Session Establishment Request to the V-UPF. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to V-UPF in this step.

5b.	The V-UPF acknowledges by sending an N4 Session Establishment Response. If CN Tunnel Info is allocated by the V-UPF, the CN Tunnel Info is provided to V-SMF in this step.

6.	V-SMF to H-SMF: Create PDU Session Request (SUPI, DNN, S-NSSAI, PDU Session ID, V-SMF ID, V-CN-Tunnel-Info, PDU type, Protocol Configuration Options, User location information, SM PDU DN Request Container).

7-12.	These steps are the same as steps 4-9 in clause 4.3.2.2.1 with the following differences:

-	These steps are executed in Home PLMN;

-	The H-SMF stores an association of the PDU Session and V-SMF ID for this PDU Session for this UE.

13.	H-SMF to V-SMF: Create PDU Session Response (QoS Rule, SSC mode, PDU session type, H-CN Tunnel Info, QoS profile(s), Session-AMBR)

The H-CN Tunnel Info contains the tunnel information for uplink traffic towards H-UPF.

Multiple QoS Rules may be included in the Create PDU Session Response.

14-18.	These steps are the same as steps 10-14 in clause 4.3.2.2.1 with the following differences:

-	These steps are executed in Visited PLMN;

-	The V-SMF stores an association of the PDU Session and H-SMF ID for this PDU Session for this UE.

19a.	The V-SMF initiates an N4 Session Modification procedure with the V-UPF. The V-SMF provides Packet detection, enforcement and reporting rules to be installed on the V-UPF for this PDU Session, including AN Tunnel Info, H-CN Tunnel Info and V-CN Tunnel Info.

19b.	The V-UPF provides a N4 Session Establishment/Modification Response to the V-SMF.

After this step, the V-UPF delivers any down-link packets to the UE that may have been buffered for this PDU Session.

20.	This step is the same as step 18 in clause 4.3.2.2.1. with the following differences:

-	The SMF is a V-SMF

21.	H-SMF to UE, via H-UPF and V-UPF in VPLMN: In case of PDU Type IPv6, the H-SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the H-UPF and V-UPF.

22.	H-SMF may register the address to UDM as Step 19 of 4.3.2.2.1. The UDM stores the H-SMF identity and the associated DNN.

NOTE:	The SMF in HPLMN may initiate step 21 already after step 13.

* * * End of Changes * * * *