SA WG2 Meeting #124	S2-178703

27 November - 1 December, 2017, Reno, USA	(revision of S2-17xxxx)

Source:	ETRI

Title:	Updating 5.3.2.4 Support of a UE registered over both 3GPP and Non3GPP access

Document for:	Approval

Agenda Item:	6.5.10

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes updates in 5.3.2.4 Support of a UE registered over both 3GPP and Non3GPP access to consider the initial registration and the Deregistration.

Discussion

When a UE 5G-GUTI assigned during a Registration procedure over 3GPP (e.g. the UE registers first over a 3GPP access) is location-specific, e.g. refers to a geographical Group Id, the same UE 5G-GUTI can be re-used over the non-3GPP access when the selected N3IWF function is in the same PLMN as the 3GPP access. When an UE 5G-GUTI is assigned during a Registration procedure performed over a Non 3GPP access (e.g. the UE registers first over a non-3GPP access) refers to a non-geographical Group Id, the 5G-GUTI may not be valid for NAS procedures over the 3GPP access and, in this case, a new AMF is allocated during the Registration procedure over the 3GPP access.

In the initial Registration procedure, a UE shall not initiate the registration over the other access until the registration procedure over first access is completed. Therefore, the registration procedure is performed over the access one by one although the UE wants to initially register over both 3GPP and non-3GPP access simultaneously. In this case, we propose that the UE registers over 3GPP access first because a new AMF may be allocated during the registration procedure over 3GPP access when the UE has already completed the registration procedure over non-3GPP access as following texts in TS 23.501.

As part of deregistration for a particular access (3GPP or non-3GPP), the AMF shall request the UE's related SMF to release the PDU Sessions established on that access.

In the Deregistration procedure, the AMF shall request the SMF to release the PDU Sessions established on either 3GPP or non-3GPP access. (Refer to following texts in clause 5.4.1.1 of TS 23.501.)

Even though the Deregistration on the non-3GPP access, the PDU sessions shall be released. Therefore, we propose updates to clause 5.3.2.4 of TS 23.501.

In addition, the Deregistration Request message indicates ‘Access Type’ to apply to the 3GPP access or non-3GPP access, or both over an access. (Refer to clause 4.2.2.3.2 and 4.2.2.3.3 of TS 23.501.) However, it is only mentioned the case that Deregistration procedure over the 3GPP access to deregister only on the non-3GPP access in TS 23.501. Therefore, we propose updates to support the case that Deregistration procedure over the non-3GPP access to deregister only on the 3GPP access in TS 23.501 to align with the Deregistration procedures in TS 23.502.

Proposal

It is proposed to update the following texts into TS 23.501.

* * * Start of changes* * * *

5.3.2.4	Support of a UE registered over both 3GPP and Non3GPP access

For a given serving PLMN there is one RM context for a UE for each access, e.g. when the UE is consecutively or simultaneously served by a 3GPP access and by a non-3GPP access (via an N3IWF) of the same PLMN. UDM manages separate/independent UE Registration procedures for each access.

An AMF associates multiple access-specific RM contexts for an UE with:

-	a 5G-GUTI that is common to both 3GPP and non-3GPP accesses. This 5G-GUTI is globally unique.

-	a Registration state per access type (3GPP / Non-3GPP)

-	a Registration Area per access type: one Registration Area for 3GPP access and another Registration Area for non 3GPP access. Registration Areas for the 3GPP access and the non-3GPP access are independent.

-	a Periodic Registration timer for 3GPP access.

-	a Non-3GPP Implicit Deregistration timer.

The AMF shall not provide a Periodic Registration Timer for the UE over a non-3GPP access. Consequently, the UE need not perform Periodic Registration Update procedure over non-3GPP access. Instead, during the Initial Registration procedure and Re-registration, the UE is provided by the network with a UE non-3GPP Deregistration timer that starts when the UE enters non-3GPP CM-IDLE state.

The 5G-GUTI may be assigned or re-assigned over any of the 3GPP and Non-3GPP accesses. The AMF assigns to the UE a single 5G-GUTI that is used over 3GPP and Non-3GPP access of the same PLMN or equivalent PLMN (which presumes that there is control and user plane connectivity between nodes of the EPLMN and the 3GPP access PLMN). The 5G-GUTI is assigned upon a successful registration of the UE, and is valid over both 3GPP and Non-3GPP access to the same PLMN or equivalent PLMN for the UE. Upon performing any initial access over the non-3GPP access or over the 3GPP access, the UE provides the 5G-GUTI it has received in an earlier successful registration over any access of the same PLMN or equivalent PLMN. This enables the AN to select an AMF that maintains the UE context created at the previous Registration procedure, and enables the AMF to correlate the UE request to the existing UE context.

If the UE is performing registration over one access and intends to perform registration over the other access in the same PLMN (e.g. the 3GPP access and the selected N3IWF are located in the same PLMN), the UE shall not initiate the registration over the other access until the registration procedure over first access is completed. If the UE wants to simultaneously register over both 3GPP and non-3GPP access in the initial Registration procedure, the UE initiates the Registration procedure over 3GPP first to possibly avoid allocating a new AMF.

NOTE:	To which access the UE performs registration first is up to UE implementation.

When the UE is successfully registered to an access (3GPP access or non-3GPP access respectively) and the UE registers via the other access:

-	if the second access is located in the same PLMN or equivalent PLMN (e.g. the UE is registered via a 3GPP access and selects a N3IWF located in the same PLMN), the UE shall use for the registration to the PLMN associated with the new access the 5G-GUTI that the UE has been provided at the previous registration for the first access in the same PLMN or equivalent PLMN.

-	if the second access is located in a PLMN different from the registered PLMN of the first access (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN), (e.g. the UE is registered to a 3GPP access and selects a N3IWF located in a PLMN different from the PLMN of the 3GPP access, or the UE is registered over non-3GPP and registers to a 3GPP access in a PLMN different from the PLMN of the N3IWF), the UE shall use for the registration to the PLMN associated with the new access a 5G-GUTI only if it has got one previously received from the same PLMN. However, if the UE does not already have a 5G-GUTI from the PLMN to which it is attempting to register or from an equivalent PLMN, the SUCI shall be used for the new registration.

When a UE 5G-GUTI assigned during a Registration procedure over 3GPP (e.g. the UE registers first over a 3GPP access) is location-specific, e.g. refers to a geographical Group Id, the same UE 5G-GUTI can be re-used over the non-3GPP access when the selected N3IWF function is in the same PLMN as the 3GPP access. When an UE 5G-GUTI is assigned during a Registration procedure performed over a Non 3GPP access (e.g. the UE registers first over a non-3GPP access) refers to a non-geographical Group Id, the 5G-GUTI may not be valid for NAS procedures over the 3GPP access and, in this case, a new AMF is allocated during the Registration procedure over the 3GPP access.

The deregistration request indicates whether it applies to the 3GPP access the non-3GPP access, or both.

If the UE is registered on both 3GPP and non-3GPP accesses and it is in CM-IDLE over non-3GPP access, then the UE or AMF may initiate a Deregistration procedure over the 3GPP access to deregister the UE only on the non-3GPP access, in which case all the PDU Sessions which are associated with the non-3GPP access shall be released.

If the UE is registered on both 3GPP and non-3GPP accesses and it is in CM-IDLE over 3GPP access and in CM-CONNECTED over non-3GPP access, then the UE or AMF may initiate a Deregistration procedure over the non-3GPP access to deregister the UE only on the 3GPP access, in which case all the PDU Sessions which are associated with the 3GPP access shall be released.

Registration Management over non-3GPP access is further defined in clause 5.5.2.

* * * End of Changes * * *