SA WG2 Meeting #124	S2-178811

27 Nov–1 Dec 2017, Reno, Nevada

Source:	Huawei, HiSilicon

Title:	TS 23.502: Clarification of Xn based HO

Document for:	Approval

Agenda Item:	6.5.7.3

Work Item / Release:	5G_ph1/Rel-15

Abstract of the contribution: This contribution proposes to add Registration procedure after Xn based HO in TS 23.502.

# Introduction

### Registration procedure after Xn based HO

It is agreed in 4.9.1.3.3 of TS 23.502 that UE may initiate mobility registration procedure after the N2 based handover:

12. The UE may initiate mobility Registration procedure as described in clause 4.2.2.2.2. The target AMF knows that it is a Handover procedure and therefore the target AMF performs only a subset of the Registration procedure, specifically the step 5,6,10 for the context transfer between source AMF and target AMF are skipped.

Similar, UE may also initiate Mobility Registration Update procedure after Xn based handover. For example, the UE moves out of the UE’s current Registration Area.

Proposal 1: Add optional Registration procedure at the end of Xn based handover.

### Editorial changes in clause 4.9.1.3.3

- Change ‘mobility Registration procedure’ in clause 4.9.1.3.3 to ‘Mobility Registration Update procedure’:

- Change the ‘the step 5,6,10’ in clause 4.9.1.3.3 to ‘the steps 4,5,10’ to align with the latest TS23.502.

### AMF selection during Registration procedure

For UE in CM-CONNECTED state, the AMF selection is not needed, since the RAN can forward the message to the AMF based on the existing N2 connection for the UE.

Proposal 2: If UE is in CM-CONNECTED state, the (R)AN can forward the Registration Request message to the AMF based on the existing N2 connection

# Proposal

It is proposed to update TS 23.502 as following:

* * * 1st Change * * *

#### 4.9.1.2	Xn based inter NG-RAN handover

##### 4.9.1.2.1	General

Clause 4.9.1.2 includes details regarding the Xn based inter NG-RAN handover with and without UPF relocation.

The handover preparation and execution phases are performed as specified in TS 38.300 [9], in case of handover to a shared network, source NG-RAN determines a PLMN to be used in the target network based on current PLMN in use. If the serving PLMN changes during Xn-based handover, the source NG-RAN node shall indicate to the target NG-RAN node (in the Handover Restriction List) the selected PLMN ID to be used in the target network.

If the AMF receives a rejection to a N2 interface procedure (e.g. Location reporting control; NAS message transfer; QoS Flow establishment/modification/release; etc.) from the NG-RAN with an indication that a Xn based handover procedure is in progress, the AMF shall reattempt the same N2 interface procedure either when the handover is complete or the handover is deemed to have failed. The failure is known by expiry of the timer guarding the N2 interface procedure.

##### 4.9.1.2.2	Xn based inter NG-RAN handover without User Plane function relocation

This procedure is used to hand over a UE from a source RAN to target RAN using Xn when the AMF is unchanged and the SMF decides to keep the existing UPF. The UPF referred in this clause 4.9.1.2.2 is the UPF which terminates N3 interface in the 5GC. The presence of IP connectivity between the Source UPF and Target UPF is assumed.

The call flow is shown in figure 4.9.1.2.2-1.

Figure 4.9.1.2.2-1 - Xn based inter NG-RAN handover without UPF relocation

1.	Target NG-RAN to AMF: N2 Path Switch Request (PDU Sessions To Be Switched, PDU Sessions Rejected, N2 SM Information)

The Target RAN sends an N2 Path Switch Request message to an AMF to inform that the UE has moved to a new target cell and provides a List Of PDU Sessions To Be Switched. The selected PLMN ID is included in the message. If none of the QoS Flows of a PDU Session are accepted by the Target RAN, the target RAN shall include the PDU Session in the list of rejected PDU Sessions. Depending on the type of target cell, the Target RAN includes appropriate information in this message.

For the PDU Sessions to be switched to the Target RAN, the N2 Path Switch Request message shall include the list of accepted QoS Flows.

2.	AMF to SMF: Nsmf_PDUSession_UpdateSMContext Request (PDU Sessions To Be Switched, PDU Sessions Rejected, UE Location Information, N2 SM Information)

The AMF sends N2 SM information by invoking the Nsmf_PDUSession_UpdateSMContext request service operation to each SMF associated with the lists of PDU Sessions received in the N2 Path Switch Request. The UE location information parameter (see clause 5.2.8.2.6) sent by the AMF is set to the target RAN Identifier.

For the PDU Sessions to be switched to the Target RAN, upon receipt of the Nsmf_PDUSession_UpdateSMContext request, each of these SMFs determines whether the existing UPF can continue to serve the UE. If the existing UPF cannot continue to serve the UE and it is not a PDU Session Anchor, steps 3-11 of clause 4.9.1.2.3 and 4.9.1.2.4 are performed. Otherwise, the following steps 3 to 6 are performed by each of these SMFs if their existing UPFs can continue to serve the UE.

If a PDU Session is included in the list of rejected PDU Session(s) by the Target RAN, the SMF deactivates the UP connections of those PDU Session(s).

If only partial QoS Flows of the PDU Session(s) are accepted by the target RAN, the SMF may initiate the PDU Session modification procedure to remove the non-accepted QoS Flows from the PDU Session(s) after the HO procedure.

For the PDU Session(s) that do not have active UP connections before HO procedure, the SMF(s) keep the inactive status after HO procedure. For the PDU Session(s) which are neither in the list of accepted PDU Sessions and nor in the list of rejected PDU Sessions received in the N2 Path Switch Request, the AMF sends the UE's location information, set to the target RAN identifier, by invoking the Namf_EventExposure_Notify service operation to the corresponding SMF if the mobility event is subscribed. SMF may reselect a UPF according to the current UE's location for the PDU Session. If a new UPF is selected to serve the PDU Session(s), steps 3-11 of clause 4.9.1.2.3 and 4.9.1.2.4 are performed. Otherwise, the following steps 3 to 6 are performed by each of these SMFs if their existing UPFs can continue to serve the UE.

If the UE moves into a non-Allowed Area, the AMF also notifies each SMF corresponding to the list of PDU Sessions received from the N2 Path Switch Request that the UE is only reachable for regulatory prioritized services.

3.	SMF to UPF: N4 Session Modification Request (AN Tunnel Info, CN Tunnel Info)

For PDU Sessions that are accepted by the Target RAN, the SMF sends an N4 Session Modification Request message to the UPF. For the PDU Sessions that are rejected by the Target RAN, the SMF sends a N4 Session Modification Request to the UPF to release the N3 tunnel. For PDU Session(s) corresponding to LADN(s) when the UE is outside the area of availability of the LADN, the SMF updates the N4 session of the UPF(s) corresponding to the PDU Session to deactivate the corresponding UP connection. The SMF may notify the UPF that originated the Data Notification to discard downlink data for the PDU Sessions and/or to not provide further Data Notification messages. If the CN Tunnel Info is derived by the SMF, the SMF provides the CN Tunnel Info to the UPF.

4.	UPF to SMF: N4 Session Modification Response (CN Tunnel Info)

For the PDU Sessions that are switched, the UPF returns an N4 Session Modification Response message to the SMF after requested PDU Sessions are switched. Tunnel identifiers for UL traffic are included only for PDU Sessions whose user plane resources are not being released and only if the UPF allocates CN Tunnel Info. For the PDU Sessions that are deactivated, the UPF returns an N4 Session Modification Response message to the SMF after the N3 (R)AN tunnel information is released.

5.	In order to assist the reordering function in the Target RAN, the UPF sends one or more "end marker" packets for each QoS Flow that require reordering on the old path immediately after switching the path. The UPF starts sending downlink packets to the Target RAN.

6.	SMF to AMF: Nsmf_PDUSession_UpdateSMContext Response (CN Tunnel Info)

The SMF sends an Nsmf_PDUSession_UpdateSMContext response (CN Tunnel Info) to the AMF for PDU Sessions which have been switched successfully. The SMF sends an Nsmf_PDUSession_UpdateSMContext response without including the CN Tunnel Info to the AMF for the PDU Sessions for which user plane resources are deactivated or released, and then the SMF releases the PDU Session(s) which is to be released using a separate procedure as defined in clause 4.3.4.

NOTE:	Step 6 can occur any time after receipt of N4 Session Modification Response at the SMF.

7.	AMF to RAN: N2 Path Switch Request Ack (N2 SM Information, List Of Failed PDU Sessions)

Once the Nsmf_PDUSession_UpdateSMContext response is received from all the SMFs, the AMF aggregates received CN Tunnel Info and sends this aggregated information as a part of N2 SM Information along with the List Of Failed PDU Sessions in N2 Path Switch Request Ack to the Target RAN. If none of the requested PDU Sessions have been switched successfully, the AMF shall send an N2 Path Switch Request Failure message to the Target RAN.

8.	By sending a Release Resources message to the Source RAN, the Target RAN confirms success of the handover. It then triggers the release of resources with the Source RAN.

9. [Conditional] The UE may initiate mobility registration update procedure if one of the triggers of registration procedure applies as described in clause 4.2.2.2.2. In this case, only steps 1, 2, 3, 17 and 21 in clause 4.2.2.2.2 are performed.

##### 4.9.1.2.3	Xn based inter NG-RAN handover with insertion of intermediate UPF

This procedure is used to hand over a UE from a Source RAN to a Target RAN using Xn when the AMF is unchanged and the SMF decides that insertion of a new additional intermediate UPF is needed. In case of using UL CL, the I-UPF can be regarded as UL CL and additional PSA providing local access to a DN. In case of using Branching Point, the I-UPF can be regarded as BP.

It is assumed that the PDU Session for the UE comprises of only one UPF that acts as a PDU Session Anchor at the time of this Handover procedure for non-roaming and local breakout roaming scenario. In case of home routed roaming scenario, the PDU Session of the UE comprises of at least one UPF in the VPLMN and one UPF in the HPLMN at the time of this handover procedure. In this case, additional insertion of an N3 terminating intermediate UPF will not have impact on the connectivity between the UPF in VPLMN and UPF in HPLMN. The presence of IP connectivity between the UPF (PDU Session Anchor) and Source RAN, and between the new intermediate UPF (I-UPF) and Target RAN, is assumed.

The call flow is shown in figure 4.9.1.2.3-1.

Figure 4.9.1.2.3-1 - Xn based inter NG-RAN handover with insertion of intermediate UPF

Steps 1-2 are the same as described in clause 4.9.1.2.2.

3.	SMF to Target UPF (intermediate): N4 Session Establishment Request (Target RAN address, UL and DL tunnel identifiers)

For PDU Sessions to be updated, the SMF then selects a new Target UPF (intermediate) based on UPF Selection Criteria according to clause 6.3.3 of TS 23.501 [2]. If the UE has moved out of the service area of UPF connecting to the serving NG-RAN node, SMF selects a Target UPF (intermediate). Target UPF IP address assignment and allocation of downlink and uplink tunnel identifiers are performed by the SMF. An N4 Session Establishment Request message is sent to the Target UPF (intermediate).

4.	Target UPF (intermediate) to SMF: N4 Session Establishment Response (Target UPF address, Tunnel Identifiers for DL User Plane)

The Target UPF (intermediate) sends an N4 Session Establishment Response message to the SMF.

5.	SMF to PDU Session Anchor: N4 Session Modification Request

The SMF sends N4 Session Modification message to the PDU Session Anchor.

6.	PDU Session Anchor to SMF: N4 Session Modification Response

The PDU Session Anchor responds with the N4 Session Modification Response message after requested PDU Sessions are switched. At this point, PDU Session Anchor starts sending downlink packets to the Target RAN using the address and tunnel identifiers of the Target RAN via Target UPF.

7.	In order to assist the reordering function in the Target RAN, the PDU Session Anchor sends one or more "end marker" packets for each QoS Flow that require reordering on the old path immediately after switching the path, the source UPF shall forward the "end marker" packets to the source RAN.

8.	SMF to AMF: Nsmf_PDUSession_UpdateSMContext Response (CN Tunnel Info)

The SMF sends a Nsmf_PDUSession_UpdateSMContext response to the AMF. CN Tunnel Info includes UL tunnel identifiers and address of the new intermediate UPF.

Steps 9-11 are same as steps 7-9 defined in clause 4.9.1.2.2.

##### 4.9.1.2.4	Xn based inter NG-RAN handover with relocation of intermediate UPF

This procedure is used to hand over a UE from a Source RAN to a Target RAN using Xn when the AMF is unchanged and the SMF decides that the intermediate UPF (I-UPF) is to be relocated. In case of using UL CL, the I-UPF can be regarded as UL CL and additional PSA provides local access to a DN, the simultaneous relocation of UL-CL and the additional PSA is described in clause 4.3.5.7. In case of using Branching Point, the I-UPF can be regarded as BP.

It is assumed that the PDU Session for the UE comprises of an UPF that acts as a PDU Session Anchor and an intermediate UPF at the time of this Handover procedure for non-roaming and local breakout roaming scenario. In case of home routed roaming scenario, the PDU Session of the UE comprises of at least one UPF in the VPLMN and UPF in the HPLMN which acts as a PDU Session Anchor at the time of this handover procedure. The Source UPF referred in this clause 4.9.1.2.4 is the UPF which terminates N3 interface in the 5GC and it serves as the PDU mobility anchor for the given PDU Session. The presence of IP connectivity between the Source UPF and Source RAN, and between the Target UPF and Target RAN, is assumed.

The call flow is shown in figure 4.9.1.2.4-1.

Figure 4.9.1.2.4-1: Xn based inter NG-RAN handover with intermediate UPF relocation

Steps 1-4 are same as steps 1-4 described in clause 4.9.1.2.3.

5.	[Conditional] The SMF sends N4 Session Modification message to the PDU Session Anchor. In case of home routed roaming, if the N9 terminating V-UPF is relocated, the V-SMF invokes an Nsmf_PDUSession_Update Request service operation toward the H-SMF.

6.	[Conditional] The SMF associated with the PDU Session Anchor responds with the N4 Session Modification Response message. In case of home routed roaming, the H-SMF responds with the Nsmf_PDUSession_Update Response service operation toward the V-SMF once H-UPF is updated with the UL tunnel information of the T-UPF. At this point, PDU Session Anchor starts sending downlink packets to the Target RAN using the address and tunnel identifiers of the Target RAN via Target UPF.

Steps 7-11 are same as steps 7-11 described in clause 4.9.1.2.3.

11.	The timer is started in step 4 if the source UPF is not the PSA UPF. When this timer is expired, the SMF initiates Source UPF Release procedure by sending an N4 Session Release Request (Release Cause).

12.	The Source UPF acknowledges with an N4 Session Release Response message to confirm the release of resources.

* * * 2nd Change * * *

##### 4.9.1.3.3	Execution phase

Figure 4.9.1.3.3-1: inter NG-RAN node N2 based handover, execution phase

NOTE:	Registration of serving AMF with the UDM is not shown in the figure for brevity.

1.	S-AMF to S-RAN:  Handover Command (Target to Source transparent container, PDU Sessions failed to be setup list, SM forwarding info list).

Target to Source transparent container is forwarded as received from S-AMF.

The SM forwarding info list includes T-RAN SM N3 forwarding info list for direct forwarding or S-UPF SM N3 forwarding info list for indirect data forwarding

S-RAN uses the PDU Sessions failed to be setup list and the indicated reason for failure to decide whether to proceed with the N2 Handover procedure.

2.	S-RAN to UE:  Handover Command (UE container).

UE container is a UE part of the Target to Source transparent container which is sent transparently from T-RAN via AMF to S-RAN and is provided to the UE by the S-RAN.

3.	Uplink packets are sent from T-RAN to T-UPF and UPF (PSA). Downlink packets are sent from UPF (PSA) to S-RAN via S-UPF. The S-RAN should start forwarding of downlink data from the S-RAN towards the T-RAN for QoS Flows subject to data forwarding. This may be either direct (step 3a) or indirect forwarding (step 3b).

4.	UE to T-RAN: Handover Confirm.

After the UE has successfully synchronized to the target cell, it sends a Handover Confirm message to the T-RAN. Handover is by this message considered as successful by the UE.

5.	T-RAN to T-AMF: Handover Notify.

Handover is by this message considered as successful in T-RAN.

6a.	[Conditional]T-AMF to S-AMF: Namf_Communication_N2InfoNotify.

The T-AMF notifies to the S-AMF about the N2 handover notify received from the T-RAN by invoking the Namf_Communication_N2InfoNotify.

A timer in S-AMF is started to supervise when resources in S-RAN shall be release.

6b.	[Conditional]S-AMF to T-AMF: Namf_Communication_N2InfoNotify ACK.

The S-AMF acknowledges by sending the Namf_Communication_N2InfoNotify ACK to the T-AMF.

7.	T-AMF to SMF: Nsmf_PDUSession_UpdateSMContext Request (Handover Complete indication for PDU Session ID).

Handover Complete is sent per each PDU Session to the corresponding SMF to indicate the success of the N2 Handover.

T-AMF also registers itself as the serving AMF with the UDM by using Nudm_UECM_Registration.

8a.	[Conditional] SMF to T-UPF (intermediate): N4 Session Modification Request.

If new T-UPF is inserted or an existing intermediate S-UPF is relocated, the SMF shall send N4 Session Modification Request indicating DL AN Tunnel Info of T-RAN to the T-UPF.

8b.	[Conditional] T-UPF to SMF: N4 Session Modification Response.

The T-UPF acknowledges by sending N4 Session Modification Response message to SMF.

9a.	[Conditional] SMF to S-UPF (intermediate): N4 Session Modification Request.

If UPF is not relocated, the SMF shall send N4 Session Modification Request indicating DL AN Tunnel Info of T-RAN to the S-UPF.

9b.	[Conditional] S-UPF to SMF: N4 Session Modification Response.

The S-UPF acknowledges by sending N4 Session Modification Response message to SMF.

10a.	[Conditional] SMF to UPF (PSA): N4 Session Modification Request.

For non-roaming or local breakout roaming scenario, the SMF sends N4 Session Modification Request message to PDU Session Anchor UPF, UPF (PSA), providing N3 AN Tunnel Info of T-RAN or the DL CN Tunnel Info of T-UPF if a new T-UPF is inserted or an existing intermediate S-UPF is relocated. If the existing intermediate S-UPF terminating to N9 toward the H-UPF (PDU Session Anchor) is relocated for the home routed roaming scenario, the V-SMF invokes an Nsmf_PDUSession_Update Request service operation toward the H-SMF.

If T-UPF is not inserted or an existing intermediate S-UPF is not relocated, steps 10a and step 10b are skipped.

10b.	[Conditional] UPF (PSA) to SMF: N4 Session Modification Response.

The UPF (PSA) sends N4 Session Modification Response message to SMF. In order to assist the reordering function in the T-RAN, the UPF (PSA) sends one or more "end marker" packets each QoS Flow that require reordering on the old path immediately after switching the path, the source UPF shall forward the "end marker" packets to the source RAN. At this point, UPF (PSA) starts sending downlink packets to the T-RAN, via T-UPF if a new T-UPF is inserted or an existing intermediate S-UPF is relocated. In case of home routed roaming scenario, the H-SMF responds with the Nsmf_PDUSession_Update Response service operation once the H-UPF (PDU Session Anchor) is updated with the UL tunnel information of the T-UPF.

When there are multiple UPFs(PSA), step 10a and step 10b are performed for each UPFs(PSA).

11.	SMF to T-AMF: Nsmf_PDUSession_UpdateSMContext Response (PDU Session ID).

SMF confirms reception of Handover Complete.

12.	The UE may initiate Mobility Registration Update procedure as described in clause 4.2.2.2.2.

The target AMF knows that it is a Handover procedure and therefore the target AMF performs only a subset of the Registration procedure, specifically the step 4,5,10 for the context transfer between source AMF and target AMF are skipped.

13a.	[Conditional] SMF to S-UPF (intermediate): N4 Session Release Request.

If there is a source intermediate UPF, the SMF initiates resource release, after timer in step 7 or indirect data forwarding timer expires, by sending an N4 Session Release Request (Release Cause) to source UPF. This message is also used to release the indirect data forwarding resource in S-UPF.

13b.	S-UPF to SMF: N4 Session Release Response.

The S-UPF acknowledges with an N4 Session Release Response message to confirm the release of resources.

In case of indirect data forwarding, the resource of indirect data forwarding is also released.

14a.	AMF to S-RAN: UE Context Release Command ().

After the timer in step 6a expires, the AMF sends UE Context Release Command.

14b.	S-RAN to AMF: UE Context Release Complete ().

The source RAN releases its resources related to the UE and responds with a UE Context Release Complete () message.

15a.	[Conditional] SMF to T-UPF: N4 Session Modification Request.

If indirect forwarding applies and UPF is relocated, after timer of indirect data forwarding expires, the SMF sends N4 Session Modification Request to T-UPF to release the indirect data forwarding resource.

15b.	[Conditional] T-UPF to SMF: N4 Session Modification Response.

The T-UPF acknowledges with an N4 Session Modification Response message to confirm the release of indirect data forwarding resources.

* * * 3rd Change * * *

#### 4.2.2.2	Registration procedures

##### 4.2.2.2.1	General

A UE needs to register with the network to get authorized to receive services, to enable mobility tracking and to enable reachability. The Registration procedure is used when the UE needs to perform Initial Registration to the 5GS, Mobility Registration Update upon changing to a new Tracking Area (TA) outside the UE's Registration Area in both CM_CONNECTED and CM_IDLE mode, when the UE performs a Periodic Registration Update (due to a predefined time period of inactivity), and additionally when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure.

The General Registration call flow in clause 4.2.2.2.2 applies on all these Registration procedures, but the periodic registration need not include all parameters that are used in other registration cases.

Editor's note:	Aspects related to dual registration in 3GPP and non-3GPP access is FFS.

The general Registration call flow in clause 4.2.2.2.2 is also used for emergency registration by UEs requiring to perform emergency services but cannot gain normal services from the network. These UEs are in limited service state as defined in TS 23.122 [22].

During the initial registration the PEI is obtained from the UE. The AMF operator may check the PEI with an EIR. The AMF passes the PEI (IMEISV) to the UDM, to the SMF and the PCF.

##### 4.2.2.2.2	General Registration

Figure 4.2.2.2.2-1: Registration procedure

1.	UE to (R)AN: AN message (AN parameters, RM-NAS Registration Request (Registration type, SUPI or 5G-GUTI, last visited TAI (if available), Security parameters, Requested NSSAI, UE 5GC Capability, PDU Session status, PDU Session(s) to be re-activated, Follow on request, and MICO mode preference)).

In case of NG-RAN, the AN parameters include e.g. SUPI or the 5G-GUTI, the Selected PLMN ID and Requested NSSAI, the AN parameters also include Establishment cause. The Establishment cause provides the reason for requesting the establishment of an RRC connection.

The Registration type indicates if the UE wants to perform an "initial registration" (i.e. the UE is in RM-DEREGISTERED state), a "Mobility Registration Update" (i.e. the UE is in RM-REGISTERED state and initiates a Registration procedure due to mobility), a "Periodic Registration Update" (i.e. the UE is in RM-REGISTERED state and initiates a Registration procedure due to the Periodic Registration Update timer expiry, see clause 4.2.2.2.1) or an "Emergency Registration" (i.e. the UE is in limited service state). The UE performing an Initial Registration (i.e., the UE is in RM-DEREGISTERED state) to a PLMN for which the UE does not already have a 5G-GUTI, the UE shall include its SUPI in the Registration Request. For an Emergency Registration, the SUPI shall be included if the UE does not have a valid 5G-GUTI available; the PEI shall be included when the UE has no SUPI and no valid 5G-GUTI. In other cases, the 5G-GUTI is included and it indicates the last serving AMF. If the UE is already registered via a non-3GPP access in a PLMN different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the 3GPP access, the UE shall not provide over the 3GPP access the 5G-GUTI allocated by the AMF during the Registration procedure over the non-3GPP access. Also, if the UE is already registered via a 3GPP access in a PLMN (i.e. the registered PLMN), different from the new PLMN (i.e. not the registered PLMN or an equivalent PLMN of the registered PLMN) of the non-3GPP access, the UE shall not provide over the non-3GPP access the 5G-GUTI allocated by the AMF during the Registration procedure over the 3GPP access. The UE may provide the UE's usage setting based on its configuration as defined in TS 23.501 [2] clause 5.16.3.7.

If available, the last visited TAI shall be included in order to help the AMF produce Registration Area for the UE.

The Security parameters are used for Authentication and integrity protection. Requested NSSAI indicates the Network Slice Selection Assistance Information (as defined in clause 5.15 of TS 23.501 [2]). The PDU Session status indicates the previously established PDU Sessions in the UE. The PDU Session(s) to be re-activated is included to indicate the PDU Session(s) for which the UE intends to activate UP connections. A PDU Session corresponding to a LADN shall not be included in the PDU Session(s) to be re-activated when the UE is outside the area of availability of the LADN. The Follow on request is included when the UE has pending uplink signalling and the UE doesn't include PDU Session(s) to be re-activated.

Editor's note:	The content of the Security parameters is FFS.

Editor's note:	It is FFS if HO attach indication is to be added.

Editor's note:	It is FFS how UE Radio capabilities are provided.

2.	If a SUPI is included or the 5G-GUTI does not indicate a valid AMF the (R)AN, based on (R)AT and Requested NSSAI, if available, selects an AMF.

The (R)AN selects an AMF as described in TS 23.501 [2], clause 6.3.5. If UE is in CM-CONNECTED state, the (R)AN can forward the Registration Request message to the AMF based on the existing N2 connection.

If the (R)AN cannot select an appropriate AMF, it forwards the Registration Request to an AMF which has been configured, in (R)AN, to perform AMF selection.

3.	(R)AN to new AMF: N2 message (N2 parameters, RM-NAS Registration Request (Registration type, SUPI or 5G-GUTI, last visited TAI (if available), Security parameters, Requested NSSAI, UE 5GC Capability, PDU Session status, PDU Session(s) to be re-activated, Follow on request, and MICO mode preference)).

When NG-RAN is used, the N2 parameters include the Selected PLMN ID, Location Information, Cell Identity and the RAT type related to the cell in which the UE is camping.

When NG-RAN is used, the N2 parameters also include the Establishment cause.

If the Registration type indicated by the UE is Periodic Registration Update, then steps 4 to 17 may be omitted.

4.	[Conditional] new AMF to old AMF: Namf_Communication_UEContextTransfer (complete Registration Request).

If the UE's 5G-GUTI was included in the Registration Request and the serving AMF has changed since last Registration procedure, the new AMF may invoke the Namf_Communication_UEContextTransfer service operation on the old AMF including the complete Registration Request IE, which may be integrity protected, to request the UE's SUPI and MM Context. See clause 5.2.2.2.2 for details of this service operation. The old AMF uses the integrity protected complete Registration request IE to verify if the context transfer service operation invocation corresponds to the UE requested.

The old AMF also transfers the event subscriptions information by each consumer NF, for the UE, to the new AMF.

NOTE 1:	The consumer NFs does not need to subscribe for the events once again with the new AMF after the UE is successfully registered with the new AMF.

If the new AMF has already received UE contexts from the old AMF during handover procedure, then step 4,5 and 10 shall be skipped.

For an Emergency Registration, if the UE identifies itself with a 5G-GUTI that is not known to the AMF, steps 4 and 5 are skipped and the AMF immediately requests the SUPI from the UE. If the UE identifies itself with PEI, the SUPI request shall be skipped. Allowing Emergency Registration without a user identity is dependent on local regulations.

5.	[Conditional] old AMF to new AMF: Response to Namf_Communication_UEContextTransfer (SUPI, MM Context, SMF information, PCF ID).

Old AMF responds to the new AMF for the Namf_Communication_UEContextTransfer invocation by including the UE's SUPI and MM Context.

If old AMF holds information about established PDU Sessions, the old AMF includes SMF information including S-NSSAI, SMF identities and PDU Session ID.

If old AMF holds information about active NGAP UE-TNLA bindings to N3IWF, the old AMF includes information about the NGAP UE-TNLA bindings.

6.	[Conditional] new AMF to UE: Identity Request ().

If the SUPI is not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE.

7.	[Conditional] UE to new AMF: Identity Response ().

The UE responds with an Identity Response message including the SUPI.

8.	The AMF may decide to initiate UE authentication by invoking an AUSF. In that case, the AMF, shall be based on SUPI, select an AUSF as described in TS 23.501 [2], clause 6.3.4.

If the AMF is configured to support Emergency Registration for unauthenticated SUPIs and the UE indicated Registration type "Emergency Registration" the AMF skips the authentication and security setup or the AMF accepts that the authentication may fail and continues the Registration procedure.

9a.	The AUSF shall execute authentication of the UE.

The authentication is performed as described in clause X, it requires UDM. The AUSF discovers a UDM as described in TS 23.501 [2], clause 6.3.8.

If network slicing is used, the AMF decides if the Registration Request needs to be rerouted as described in clause 4.2.2.2.3, where the initial AMF refers to the AMF.

9b	The AMF shall initiate NAS security functions.

The NAS security is performed as described in clause X.

10.	[Conditional] new AMF to old AMF: Namf_Communication_RegistrationCompleteNotify ().

If the AMF has changed the new AMF notifies the old AMF that the registration of the UE in the new AMF is completed by invoking the Namf_Communication_RegistrationCompleteNotify service operation.

If the authentication/security procedure fails, then the Registration shall be rejected, and the new AMF invokes the Namf_Communication_RegistrationCompleteNotify service operation with a reject indication reason code towards the old AMF. The old AMF continues as if the UE context transfer service operation was never received.

If one or more of the S-NSSAIs used in the old Registration Area cannot be served in the target Registration Area, the new AMF determines which PDU Session cannot be supported in the new Registration Area. The new AMF invokes the Namf_Communication_RegistrationCompleteNotify service operation including the rejected PDU Session ID and a reject cause (e.g. the S-NSSAI becomes no longer available) towards the old AMF. Then the new AMF modifies the PDU Session Status correspondingly. The old AMF informs the corresponding SMF(s) to locally release the UE's SM context by invoking the Nsmf_PDUSession_ReleaseSMContext service operation.

See clause 5.2.2.2.3 for details of Namf_Communication_RegistrationCompleteNotify service operation.

11.	[Conditional] new AMF to UE: Identity Request/Response (PEI).

If the PEI was not provided by the UE nor retrieved from the old AMF the Identity Request procedure is initiated by AMF sending an Identity Request message to the UE to retrieve the PEI. The PEI shall be transferred encrypted unless the UE performs Emergency Registration and cannot be authenticated.

For an Emergency Registration, the UE may have included the PEI in the Registration Request. If so, the PEI retrieval is skipped.

12.	Optionally the new AMF initiates ME identity check by invoking the N5g-eir_MEquipmentIdentityCheck_Get service operation (see clause 5.2.4.2.2).

The PEI check is performed as described in clause 4.7.

For an Emergency Registration, if the PEI is blocked, operator policies determine whether the Emergency Registration procedure continues or is stopped.

13.	If step 14 is to be performed, the new AMF, based on the SUPI, selects a UDM.

The AMF selects a UDM as described in TS 23.501 [2], clause 6.3.8.

14a-b. If the AMF has changed since the last Registration procedure, or if the UE provides a SUPI which doesn't refer to a valid context in the AMF, or if the UE registers to the same AMF it has already registered to a non-3GPP access (i.e. the UE is registered over a non-3GPP access and initiates this Registration procedure to add a 3GPP access), the new AMF registers with the UDM using Nudm_UECM_Registration and subscribes to be notified when the UDM deregisters this AMF. AMF retrieves the mobility subscription data using Nudm_SDM_Get and subscribes to be notified using Nudm_SDM_Subscribe when the mobility subscription data is modified. The GPSI is provided to the AMF in the subscription data from the UDM if the GPSI is available in the UE subscription data.

The new AMF provides the Access Type it serves for the UE to the UDM and the Access Type is set to "3GPP access". The UDM stores the associated Access Type together with the serving AMF.

Editor's note:	Whether interactions with UDM and PCF includes mobility restriction related information is FFS.

Editor's note:	The PEI is to be provided to the UDM in the Update Location procedure.

The new AMF creates an MM context for the UE after getting the mobility subscription data from the UDM.

For an Emergency Registration in which the UE was not successfully authenticated, the AMF shall not register with the UDM.

For an Emergency Registration, the AMF shall not check for access restrictions, regional restrictions or subscription restrictions. For an Emergency Registration, the AMF shall ignore any unsuccessful registration response from UDM and continue with the Registration procedure.

14c.	When the UDM stores the associated Access Type together with the serving AMF as indicated in step 14a, it will cause the UDM to initiate a Nudm_UECM_DeregistrationNotification (see clause 5.2.3.2.2) to the old AMF corresponding to 3GPP access, if one exists. The old AMF removes the MM context of the UE. If the serving NF removal reason indicated by the UDM is "Initial Registration", then the old AMF invokes the Namf_EventExposure_Notify service operation towards all the associated SMFs of the UE to notify that the UE is deregistered from old AMF. The SMF shall release the PDU Session(s) on getting this notification.

14d.	The Old AMF unsubscribes with the UDM for subscription data using Nudm_SDM_unsubscribe.

15.	In case the new AMF receives a PCF ID from the old AMF in step 5 and successfully contacts the PCF identified by the PCF ID, the new AMF retrieves the dynamic policy from the PCF.

If the PCF identified by the PCF ID cannot be used (e.g. no response from the PCF) or there is no the PCF ID received from the old AMF in step 5, t he AMF selects a PCF as described in TS 23.501 [2], clause 6.3.7.1.

16.	[Optional] new AMF to PCF: Npcf_AMPolicyControl_Get (SUPI, GPSI).

If the AMF has not yet obtained Access and Mobility policy for the UE or if the Access and Mobility policy in the AMF are no longer valid, the AMF requests the PCF to apply operator policies for the UE from the PCF through the Npcf_AMPolicyControl_Get service operation (see clause 5.2.5.2.2). The GPSI is included if available at AMF. In roaming case, the interaction between H-PCF and V-PCF is required for the provision of Access and Mobility policy. For an Emergency Registration, this step is skipped.

PCF to new AMF: Response to Npcf_AMPolicyControl_Get (Access and Mobility policy data).

The PCF responds to the Npcf_AMPolicyControl_Get service operation and provides the Access and Mobility policy data for the UE to the AMF.

17.	[Conditional] AMF to SMF: Namf_EventExposure_Notify () or Nsmf_PDUSession_UpdateSMContext ().

For an Emergency Registered UE, this step is applied when the Registration Type is "Mobility Registration Update".

The AMF invokes the Namf_EventExposure_Notify (see clause 5.2.2.3.4) or Nsmf_PDUSession_UpdateSMContext (see clause 5.2.8.2.6) in one or more of the following scenarios:

-	If the AMF is changed, the new AMF notifies each SMF of the new AMF serving the UE by informing the UE reachability status. In case the AMF has changed, it is assumed that the old AMF provides the available SMF information. If any PDU Session status indicates that it is released at the UE, the AMF invokes the Nsmf_PDUSession_ReleaseSMContext service operation towards the SMF in order to release any network resources related to the PDU Session.

-	If the "PDU Session(s) to be re-activated" is included in the Registration Request in step 1, the AMF sends Nsmf_PDUSession_UpdateSMContext Request to SMF(s) associated with the PDU Session(s) to activate User Plane connections of the PDU Session(s). From step 4 onwards described in clause 4.2.3.3 are executed to complete the User Plane connection activation without sending MM NAS Service Accept from the AMF to (R)AN described in step 8 of clause 4.2.3.3.

Editor's note:	It is FFS how the AMF handles if N1 SM information is included in Nsmf_PDUSession_UpdateSMContext Response by the SMF in step 7 of clause 4.2.3.3 due to relocation of PDU Session Anchor UPF.

-	If the UE was in MICO mode and the AMF had notified an SMF of the UE being unreachable and that the SMF needs not to send DL data notifications to the AMF, the AMF informs the SMF that the UE is reachable.

-	If the AMF had notified an SMF of the UE being reachable only for regulatory prioritized service and the UE enters into Allowed Area, the AMF informs the SMF that the UE is reachable.

-	If the UE is in Non-Allowed Area, and the PDU Session(s) to be re-activated is included in the Registration Request, then the AMF informs all the associated SMF(s) of the UE is reachable only for regulatory prioritized service.

NOTE 2:	the AMF will also notify any NF(s) that subscribed to UE reachability of the change of UE reachability.

-	If an SMF has subscribed to UE location change notification via Namf_EventExposure_Subscribe service operation and if the AMF detects that the UE has moved out the area of interest subscribed by an SMF serving the UE, the AMF invokes Namf_EventExposure_Notify service operation to inform the SMF of the new location information of the UE.

NOTE 3: Whether notification Ack needs a separate message or be realized in the transport layer will be determined in TS 29.518 [18].

The SMF may decide to trigger e.g. new intermediate UPF insertion, removal or relocation or UPF relocation as described in step 5 in clause 4.2.3.2.

If the Registration type indicated by the UE is Periodic Registration Update, then steps 20 may be omitted.

If the serving AMF is changed, the new AMF shall wait until step 17 is finished with all the SMFs associated with the UE. Otherwise, steps 18 to 22 can continue in parallel to this step.

18. New AMF to N3IWF: N2 Request ().

The AMF may decide to modify the NGAP UE-TNLA-binding toward N3IWF as described in 4.2.7.2. This is done in case AMF is changed and old AMF have existing NGAP UE-TNLA-bindings toward a N3IWF for the UE.

19.	N3IWF to new AMF: N2 Response ().

Editor's note:	It is FFS whether further description is needed on how the old AMF releases the existing NGAP UE-TNLA-bindings toward a N3IWF is FFS.

20.	[Conditional] old AMF to PCF: Npcf_AMPolicyControl_Delete ().

If the old AMF previously requested UE context to be established in the PCF, the old AMF terminates the UE context in the PCF by invoking the Npcf_AMPolicyControl_Delete service operation (see clause 5.2.5.2.4).PCF to old AMF: Response to Npcf_AMPolicyControl_Delete ().

21.	New AMF to UE: Registration Accept (5G-GUTI, Registration Area, Mobility restrictions, PDU Session status, Allowed NSSAI, Periodic Registration Update timer, LADN Information and accepted MICO mode, IMS Voice over PS session supported Indication, Emergency Service Support indicator).

The AMF sends a Registration Accept message to the UE indicating that the Registration Request has been accepted. 5G-GUTI is included if the AMF allocates a new 5G-GUTI. If the AMF allocates a new Registration area, it shall send the Registration area to the UE via Registration Accept message. If there is no Registration area included in the Registration Accept message, the UE shall consider the old Registration Area as valid. Mobility restrictions is included in case mobility restrictions applies for the UE and Registration Type is not "Emergency Registration". The AMF indicates the established PDU Sessions to the UE in the PDU Session status. The UE removes locally any internal resources related to PDU Sessions that are not marked as established in the received PDU Session status and for which the UE has requested PDU Session establishment and not received SMF response yet. If the PDU Session status information was in the Registration Request, the AMF shall indicate the PDU Session status to the UE. The NSSAI includes the Allowed S-NSSAIs. If the UE subscription data includes subscribed LADN identification information, the AMF shall include in the Registration Accept message the LADN Information for LADNs, defined in TS 23.501 [2] clause 5.6.5, that are available within the Registration area determined by the AMF for the UE. If the UE included MICO mode in the request, then AMF responds whether MICO mode should be used. The AMF sets the IMS Voice over PS session supported Indication as described in clause 5.16.3.2 of TS 23.501 [2]. In order to set the IMS Voice over PS session supported Indication the AMF may need to perform the UE/RAN Radio information and Compatibility Request procedure in clause 4.2.8 to check the compatibility of the UE and RAN radio capabilities related to IMS Voice over PS. If the AMF hasn't received Voice Support Match Indicator from the NG-RAN on time then, based on implementation, AMF may set IMS Voice over PS session supported Indication and update it at a later stage. The Emergency Service Support indicator informs the UE that emergency services are supported, i.e. the UE is allowed to request PDU Session for emergency services.

The Handover Restriction List is provided to NG-RAN by AMF in this step.

When the Follow on request is included, the AMF shall not release the signalling connection immediately after the completion of the Registration procedure.

For an Emergency Registered UE, there is no AS security context information included in the N2 control messages and there is no NAS level security when the UE cannot be authenticated.

Editor's note:	It is FFS whether NSSAI is provided to RAN by AMF.

Editor's note:	It is FFS whether LADN subscription information (i.e. information which indicates whether the UE subscribes the LADN service) is included in UE subscription data or is managed by policy information by PCF.

22.	[Conditional] UE to new AMF: Registration Complete ().

The UE sends a Registration Complete message to the AMF to acknowledge if a new 5G-GUTI was assigned.

When the "PDU Session(s) to be re-activated" is not included in the Registration Request, the AMF releases the signalling connection with UE, according to clause 4.2.6. When the Follow on request is included in the Registration Request, the AMF shall not release the signalling connection immediately after the completion of the Registration procedure.

Editor's note:	Details of N2 interaction is FFS.

* * * End of Changes * * *