# SA WG2 Meeting #S2-124 SA WG2 Meeting # SA WG2 Meeting #124	S2-179640

# 27 November - 01 December, 2017,Reno, Nevada, USA	(e-mail revision 6 of S2-179006)

Source:	OPPO

Title:	Handover Cancel Procedure between 5GS and EPS

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_ph1 / Rel-15

Abstract of the contribution: This contribution adds the handover cancel procedures between 5GS and EPS.

# Discussion

This contribution provides the handover cancel procedure within 5GS and between 5GS and EPS.

# 2	Proposal

It is proposed to agree on the proposed changes to TS 23.502.

*************** Start of changes *********************

#### 4.9.1.4	Inter NG-RAN node N2 based handover, Cancel

Instead of completing the handover procedure, the source NG-RAN node may at any time during the handover procedure, up to the time when a handover command message is sent to the UE cancel the handover. The reason for cancelling may be e.g. due to a timer expiration or due to other events within the source NG-RAN node and is initiated by sending a handover cancel request to the source AMF. This is done in order to release the resources reserved for the handover in the target system.

The AMF shall cancel the handover resources as defined in clause 4.11.1.2.x for case the source RAN is NG-RAN.

*************** 2nd of changes *********************

##### 4.11.1.2.x	Handover Cancel

Instead of completing the handover procedure, the source RAN node (NG-RAN, E-UTRAN) may at any time, during the handover procedure, up to the time when a handover command message is sent to the UE, cancel the handover. The reason for cancelling may be e.g. due to a timer expiration or due to other events within the source RAN node and is initiated by sending a handover cancel message to the source CN node (AMF or MME).

A handover cancel message shall also be sent by the source RAN node after a handover command message is sent to the UE for the case where the handover fails and the UE returns to the old cell or radio contact with the UE is lost. This is done in order to release the resources reserved for the handover in the target system.

Figure 4.11.1.2.x-y: Handover Cancel procedure

1. When the source RAN (NG-RAN, E-UTRAN) decides to cancel the handover to the target system, the source RAN initiates handover cancel message to the source CN node (AMF or MME).

2. After receiving the handover cancel message from the source RAN, the source CN node (AMF or MME) sends a "Relocation Cancel Request" message to the target CN node (MME or AMF).

3. The target CN node (MME or AMF) triggers release of resources towards target RAN node. The target RAN node releases the AN resources allocated for the handover.

4. The target CN node deletes the session established during handover preparation phase to SGW(PGW-C)/SMF.

4a. [Conditional] The SGW(PGW-C)/SMF releases the corresponding resource in the T-UPF/S-UPF if allocated during the handover preparation.

5. The target CN node (MME or AMF) sends "Relocation Cancel Response" towards the source CN node (AMF orMME).

6. The source CN node (AMF or MME) responds with handover cancel ACK towards the source RAN.

7. If indirect forwarding tunnel is setup during handover preparation phase then cancellation of handover triggers the source CN node to release the temporary resources used for indirect forwarding.

8. If indirect forwarding tunnel is setup during handover preparation phase then cancellation of handover triggers the target CN node to release the temporary resources used for indirect forwarding.

*************** End of changes *********************