SA WG2 Meeting #124	S2-178815

November 27 – December 1, 2017, Reno, USA

Source:	Huawei, HiSilicon

Title:	TS 23.502: Update the indirect data forwarding during HO procedure with N26

Document for:	Approval

Agenda Item:	6.5.9

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to clarify the indirect data forwarding procedure during UE handover from 5GS to EPS.

# Introduction

Clarify the following issues for indirect data forwarding during the UE HO between 5GS and EPS with N26 interface.

For HO from 5GS to EPS:

- Clarify the AMF shall decide the PGW-C+SMF to be contact in step 10a based on the EPS bearer ID to be forwarded.

- The EPS bearers and QoS flow(s) mapping should be performed before the PGW-C+SMF or UPF assigns CN tunnel information in step 10b, not in step 10c.

- Clarify that the CN tunnel information assignment for indirect data forwarding should be in per PDU Session level.

- Replace “Create Indirect Data Forwarding Tunnel Request/Response” with SBI message “Nsmf_PDUSession_UpdateSMContext Request/Response” and adding “Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding” to input of Nsmf_PDUSession_UpdateSMContext Request.

For HO from EPS to 5GS:

-   If multiple QoS flows are mapped to an EPS bearer, the UPF+PGW-U shall determine the QFI of the downlink data packet(s) forwarded from S-GW based on the IP header of the downlink data packet(s).

-   Reorganize the final steps of the call flow of preparation phase to align with the figure.

# Proposal

It is proposed to update TS 23.502 as follows:

*************** Start of the changes ***************

#### 4.11.1.2	Handover procedures

##### 4.11.1.2.1	5GS to EPS handover using N26 interface

Figure 4.11.1.2.1-1 describes the handover procedure from 5GS to EPS when N26 is supported.

In case of handover to a shared network, the source NG-RAN determines a PLMN to be used in the target network based on current PLMN in use. The source NG-RAN shall indicate the selected PLMN ID to be used in the target network to the AMF as part of the Tracking Area sent in the HO Required message.

During the handover procedure, as specified in clause 4.9.1.2.1, the source AMF shall reject any SMF+PGW-C initiated N2 request received since handover procedure started and shall include an indication that the request has been temporarily rejected due to handover procedure in progress.

Upon reception of a rejection for a SMF+PGW-C initiated N2 request(s) with an indication that the request has been temporarily rejected due to handover procedure in progress, the SMF+PGW-C behaves as specified in clause 4.9.1.2.1.

Figure 4.11.1.2.1-1: 5GS to EPS handover for single-registration mode with N26 interface

The procedure involves a handover to EPC and setup of default EPS bearer and dedicated bearers for GBR QoS Flows in EPC in steps 1-16 and re-activation, if required, of dedicated EPS bearers for non-GBR QoS Flows in step 17. This procedure can be triggered, for example, due to new radio conditions, load balancing or due to specific service e.g. in the presence of QoS Flow for voice, the source NG-RAN node may trigger handover to EPC.

For Ethernet and Unstructured PDU Session Types, the PDN Type non-IP is used, when supported, in EPS. The SMF shall thus set the PDN Type of the EPS Bearer Context to non-IP in these cases. After the handover to EPS, the PDN Connection will have PDN Type non-IP, but it shall be locally associated in UE and SMF to PDU Session Type Ethernet or Unstructured respectively.

In the roaming home routed case, the PGW-C+SMF always provides the EPS Bearer ID and the mapped QoS parameters to UE. This also applies in the case that the HPLMN operates the interworking procedure without N26.

NOTE 1:	The IP address continuity can't be supported, if PGW-C+SMF in the HPLMN doesn't provide the mapped QoS parameters.

1.	NG-RAN decides that the UE should be handed over to the E-UTRAN. The NG-RAN sends a Handover Required (Target eNB ID, Source to Target Transparent Container) message to the AMF. NG-RAN indicates bearers corresponding to the 5G QoS Flows for data forwarding in Source to Target Transparent Container.

2.	The AMF determines from the 'Target eNB Identifier' IE that the type of handover is Handover to E-UTRAN. The AMF selects an MME as described in TS 23.401 [13] clause 4.3.8.3.

The AMF requests the PGW-C+SMF to provide SM Context that also includes the mapped EPS Bearer Contexts. For PDU Sessions with PDU Session Type Ethernet or Unstructured, the SMF provides SM Context for non-IP PDN Type. The AMF provides the target MME capability to SMF in the request to allow the SMF to determine whether to included EPS Bearer context for non-IP PDN Type or not. This step is performed with all PGW-C+SMFs allocated to the UE for each PDU Session of the UE.

NOTE 2:	The AMF knows the MME capability to support non-IP PDN type or not through local configuration.

Editor's note:	This step should be aligned with intra 5GC inter-AMF handover.

NOTE 3:	In roaming scenario, the UE's SM EPS Contexts are obtained from the V-SMF.

3.	The AMF sends a Forward Relocation Request (Target E-UTRAN Node ID, Source to Target Transparent Container, mapped MM and SM EPS UE Context (default and dedicated GBR bearers)) message. The SGW address and TEID for both the control-plane or EPS bearers in the message are such that target MME selects a new SGW.

NOTE 4:	The mapped SM EPS UE Contexts are included for PDU Sessions with and without active UP connections.

4.	The MME selects the Serving GW and sends a Create Session Request message for each PDN connection to the Serving GW.

5.	The Serving GW allocates its local resources and returns them in a Create Session Response message to the MME.

6.	The MME requests the target eNodeB to establish the bearer(s) by sending the message Handover Request message. This message also contains a list of EPS Bearer IDs that need to be setup.

7.	The target eNB allocates the requested resources and returns the applicable parameters to the target MME in the message Handover Request Acknowledge (Target to Source Transparent Container, EPS Bearers setup list, EPS Bearers failed to setup list). The EPS Bearer Setup list includes a list of addresses and TEIDs allocated at the target eNB for downlink traffic on S1-U reference point (one TEID per bearer) and addresses and TEIDs for receiving forwarded data if necessary.

8.	If indirect forwarding applies, the MME sends a Create Indirect Data Forwarding Tunnel Request message (Target eNB Address, TEID(s) for DL data forwarding) to the Serving GW. The Serving GW returns a Create Indirect Data Forwarding Tunnel Response (Cause, Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding) message to the target MME.

9.	The MME sends the message Forward Relocation Response (Cause, List of Set Up RABs, EPS Bearers setup list, MME Tunnel Endpoint Identifier for Control Plane, RAN Cause, MME Address for Control Plane, Target to Source Transparent Container, Address(es) and TEID(s) for Data Forwarding).

10a.	If indirect data forwarding applies, the AMF sends the Nsmf_PDUSession_UpdateSMContext  Request (Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding) to the PGW-C+SMF. If multiple PGW-C+SMFs serves the UE, the AMF maps the EPS bearers for Data forwarding to the PGW-C+SMF address(es) based on the association between the EPS bearer ID(s) and PDU Session ID(s).

10b. The PGW-C+SMF maps the EPS bearers for Data forwarding to the 5G QoS flows based on the association between the EPS bearer ID(s) and QFI(s) for the QoS flow(s) in the PGW-C+SMF, and then sends the QFIs, Serving GW Address(es) and TEID(s) for data forwarding to the PGW-U+UPF. If CN Tunnel Info for Data Forwarding is allocated by the PGW-C+SMF, the CN Tunnel Info for Data Forwarding is provided to PGW-U+UPF in this step. The PGW-U+UPF acknowledges by sending a response. If CN Tunnel Info is allocated by the PGW-U+UPF, the CN Tunnel Info is provided to PGW-C+SMF in this response.

10c.	The PGW-C+SMF returns a Nsmf_PDUSession_UpdateSMContext Response (Cause, CN tunnel Info for Data Forwarding, QoS flows for Data Forwarding).

11.	The AMF sends the Handover Command to the source NG-RAN. The source NG-RAN commands the UE to handover to the target access network by sending the HO Command (Transparent container (radio aspect parameters that the target eNB has set-up in the preparation phase), CN tunnel info for data forwarding per PDU Session, QoS flows for Data Forwarding). The UE correlates the ongoing QoS Flows with the indicated EPS Bearer IDs to be setup in the HO command. The UE locally deletes the PDU Session if the QoS flow of the default QoS rule in the PDU Session does not have an EPS Bearer ID assigned. For the remaining PDU Sessions, the UE locally deletes the QoS Flows that do not have an EPS bearer ID assigned. The UE deletes any UE derived QoS rules. The EPS Bearer ID that was assigned for the QoS flow of the default QoS rule in the PDU Session becomes the EPS Bearer ID of the default bearer in the corresponding PDN connection.

For the QoS Flows indicated in the "QoS Flows for Data Forwarding", NG-RAN initiate data forwarding via to the PGW-U+UPF based on the CN Tunnel Info for Data Forwarding per PDU Session. Then the PGW-U+UPF maps the data received from the CN Tunnel(s) for data forwarding in the 5GS to the Serving GW tunnel(s) for data forwarding in EPS, and sends the data to the target eNodeB via the Serving GW.

12.	When the UE has successfully accessed the target eNodeB, the target eNodeB informs the target MME by sending the message Handover Notify.

12c.	Then the MME knows that the UE has arrived to the target side and MME informs the AMF by sending the Relocation Complete Notification message.

12d.	The AMF acknowledges MME with Relocation Complete Ack message. A timer in AMF is started to supervise when resources in the in NG-RAN and PGW-C+SMF shall be released.

Upon receipt of the Relocation Complete Ack message, the MME starts a timer if the MME applies indirect forwarding.

13.	The target MME informs the Serving GW that the MME is responsible for all the bearers the UE have established by sending the Modify Bearer Request message for each PDN connection.

The target MME releases the non-accepted EPS Bearer contexts by triggering the Bearer Context deactivation procedure. If the Serving GW receives a DL packet for a non-accepted bearer, the Serving GW drops the DL packet and does not send a Downlink Data Notification to the SGSN.

14.	The Serving GW informs the PGW-C+SMF of the relocation by sending the Modify Bearer Request message for each PDN connection. The PGW-C+SMF locally deletes the PDU Session if the QoS flow of the default QoS rule in the PDU Session does not have an EPS Bearer ID assigned. For the remaining PDU Sessions, the PGW-C+SMF locally deletes the QoS Flows that do not have an EPS bearer ID assigned.

NOTE 5:	If the QoS flow is deleted and the default QoS rule contains no packet filters, the IP flows of the deleted QoS rules are mapped to the default EPS bearer. If the default QoS rule contains packet filter(s), the IP flows in the deleted QoS Flow may be interrupted until Step 17.

15.	The PGW-C+SMF acknowledges the Modify Bearer Request. At this stage the User Plane path is established for the default bearer and the dedicated GBR bearers between the UE, target eNodeB, Serving GW and the PGW+SMF. The PGW-C+SMF uses the EPS QoS parameters as assigned for the dedicated EPS GBR bearers during the QoS flow establishment. PGW-C+SMF maps all the other IP flows to the default EPS bearer.

16.	The Serving GW acknowledges the User Plane switch to the MME via the message Modify Bearer Response.

17. The UE initiates a Tracking Area Update procedure as specified in TS 23.401 [13], clause 5.3.3.0.

The target MME knows that a Handover has been performed for this UE as it received the EPS bearer context(s) in Step 3 and therefore the target MME performs only a subset of the TA update procedure, specifically it excludes the context transfer procedures between source AMF and target MME.

The MME shall indicate the EPS bearer status to the UE in the TAU Accept and the UE shall locally release any non-transferred PDU Session.

18.	The PGW-C+SMF initiates dedicated bearer activation procedure for non-GBR QoS Flows by mapping the parameters of the non-GBR Flows to EPC QoS parameters. This setup may be triggered by the PCRF+PCF which may also provide the mapped QoS parameters, if PCC is deployed. This procedure is specified in TS 23.401 [13], clause 5.4.1. For Ethernet PDU Session Type, using non-IP PDN Type in EPS, this step is not applicable.

19.	If indirect forwarding was used, then the expiry of the timer at MME started at step 12d triggers the MME to send a Delete Indirect Data Forwarding Tunnel Request message to the SGW, in order to release the temporary resources used for indirect forwarding that were allocated at step 8. The SGW returns Delete Indirect Data Forwarding Tunnel Response message.

20.	If indirect forwarding was used, then the expiry of the timer at AMF started at step 12c triggers the AMF to send a Delete Indirect Data Forwarding Tunnel Request message to the PGW-C+SMF, in order to release temporary resources used for indirect forwarding that were allocated at step 10. The PGW-C+SMF returns Delete Indirect Data Forwarding Tunnel Response message.

*************** End of the 1st changes****************

##### 4.11.1.2.2	EPS to 5GS handover using N26 interface

4.11.1.2.2.1	General

N26 interface is used to provide seamless session continuity for single registration mode. Figure 4.11.1.2.1-1 describes and Figure 4.11.1.x.3-1 the handover procedure from EPS to 5GS when N26 is supported.

The procedure involves a handover to 5GS and setup of QoS Flows in 5GS.

In the home routed roaming case, the PGW-C+ SMF in the HPLMN always receives the PDU Session ID from UE and provides other 5G QoS parameters to UE. This also applies in the case that the HPLMN operates the interworking procedure without N26.

In case the PDN Type of a PDN Connection in EPS is non-IP, and is locally associated in UE and SMF to PDU Session Type Ethernet or Unstructured, the PDU Session Type in 5GS shall be set to Ethernet or Unstructured respectively.

NOTE 1:	Interworking between EPS and 5GS is supported with IP address preservation by assuming SSC mode 1 in this release of the specification.

NOTE 2:	The IP address continuity can't be supported, if PGW-C+SMF in the HPLMN doesn't provide the mapped QoS parameters.

Editor's note:	It is FFS how the SSC mode parameters is determined in the UE and network when UE is in EPS.

Editor's note:	Indirect forwarding for home-routed roaming is FFS.

4.11.1.2.2.2	Preparation phase

Figure 4.11.1.2.2.2-1 shows the preparation phase of the Single Registration-based Interworking from EPS to 5GS procedure.

Figure 4.11.1.2.2.2-1: EPS to 5GS handover using N26 interface, preparation phase

This procedure applies to the Non-Roaming (TS 23.501 [2] Figure 4.3.1-1), Home-routed roaming (TS 23.501 [2] Figure 4.3.2-1) and Local Breakout roaming Local Breakout (TS 23.501 [2] Figure 4.3.2-2) cases.

-	For non-roaming scenario, v-SMF, v-UPF and v-PCF+v-PCRF are not present

-	For home-routed roaming scenario, the SMF+PGW-C and UPF+PGW-U are in the HPLMN. v-PCF+v-PCRF are not present

-	For local breakout roaming scenario, v-SMF and v-UPF are not present. SMF+PGW-C and UPF+PGW-U are in the VPLMN.

In local-breakout roaming case, the v-PCF+v-PCRF forwards messages between the SMF+PGW-C and the h-PCF+h-PCRF.

1.	The source E-UTRAN decides that the UE should be handed over to the NG-RAN.

2.	The E-UTRAN sends a Handover Required (Target NG-RAN Node ID, Source to Target Transparent Container) message to the MME.

3.	The MME selects the target AMF and sends a Forward Relocation Request (Target NG-RAN Node ID, Source to Target Transparent Container, EPS MM Context, EPS Bearer Context(s)) message to the selected AMF.

The AMF converts the received EPS MM Context into the 5GS MM Context. The MME UE context includes IMSI, ME Identity, UE security context, UE Network Capability, and EPS Bearer context(s). An EPS Bearer context includes the common SMF + PGW-C address and V-CN Tunnel Info at the UPF + PGW-U for uplink traffic, and APN.

Editor's note:	Whether 5GS Security Context can be derived from the EPS Security Context should be clarified by SA3 study.

4.	The AMF invokes the Nsmf_PDUSession_UpdateSMContext service operation (PDN Connection ID, EPS Bearer Context(s), AMF ID) on the SMF identified by the common SMF + PGW-C address received in step 3.

For home-routed roaming scenario, the v-SMF selects the SMF + PGW-C using the PDN Connection. The AMF ID is the UE's GUAMI which uniquely identifies the AMF serving the UE.

Editor's note:	It is FFS whether and how to provide the SM context from MME to SMF via AMF during handover.

5.	If dynamic PCC is deployed, the SMF may initiate PDU Session Modification towards the h-PCF + h-PCRF to obtain the 5GS PCC Rules for the PDU Session. The h-PCF + h-PCRF does not apply the 5GS PCC Rules for the PDU Session.

6.  The SMF+PGW-C modifies the PGW-U+UPF.

Editor's note:	The details of SMF+PGW-C and UPF interaction is FFS.

7.	The SMF + PGW-C sends a Nsmf_PDUSession_UpdateSMContext Response (N2 SM Information (PDU Session ID, QoS Rules, EPS Bearer Setup List, SSC Mode, H-CN Tunnel-Info)) to the AMF. AMF stores an association of the PDU Session ID and the SMF ID.

In case the PDN Type of a PDN Connection in EPS is non-IP, and is locally associated in SMF to PDU Session Type Ethernet, the PDU Session Type in 5GS shall be set to Ethernet . In case the PDN type of a PDN Connection in EPS is non-IP, and is locally associated in UE and SMF to PDU Session Type Unstructured, the PDU Session Type in 5GS shall be set to Unstructured.

In case of PDU Session Type Ethernet, that was using PDN type non-IP in EPS, the SMF creates QoS rules based on the 5GS PCC Rules received from PCF.

8.	For home-routed roaming scenario only: The v-SMF selects a v-UPF and initiates an N4 Session Establishment procedure with the selected v-UPF. The v-SMF provides the v-UPF with packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session, including H-CN Tunnel Info. If CN Tunnel Info is allocated by the SMF, the V-CN Tunnel Info is provided to the v-UPF in this step.

The v-UPF acknowledges by sending an N4 Session Establishment Response message. If CN Tunnel Info is allocated by the UPF, the V-CN Tunnel info is provided to the v-SMF in this step.

9.	The AMF sends a Handover Request (Source to Target Transparent Container, N2 SM Information (PDU Session ID, QoS Profile(s), V-CN Tunnel Info) , Handover Restriction List) message to the NG-RAN.

10.	The NG-RAN sends a Handover Request Acknowledge (Target to Source Transparent Container, N2 SM response (PDU Session ID, list of accepted QoS Flows and AN Tunnel Info), N2 SM Information for PDU Forwarding (PDU Session ID, N3 Tunnel Info for PDU Forwarding)) message to the AMF.

11.	The AMF sends an Nsmf_PDUSession_UpdateSMContext Request (PDU Session ID, N2 SM response (list of accepted QoS Flows and AN Tunnel Info), (N2 SM Information for PDU Forwarding (PDU Session ID, N3 Tunnel Info for PDU Forwarding)) message to the SMF for updating N3 tunnel information.

12. SMF+PGW-C to AMF: Nsmf_PDUSession_UpdateSMContext Response (PDU Session ID, EPS Bearer Setup List).

This message is sent for each received Modify PDU Request message.

13.SMF+PGW-C performs preparations for N2 Handover by indicating N3 UP address and Tunnel ID of NG-RAN to the UPF if N2 Handover is accepted by NG-RAN. If multiple QoS flows are mapped to an EPS bearer, the UPF+PGW-U determines the QFI of the downlink data packet(s) of the EPS bearer based on the IP header of the downlink data packet(s). If N2 Handover is not accepted by NG-RAN, SMF+PGW-C deallocates N3 UP address and Tunnel ID of the selected UPF.

The EPS Bearer Setup list is a list of EPS bearer Identifiers successfully handover to 5GC, which is generated based on the list of accepted QoS Flows.

14. The AMF sends the message Forward Relocation Response (Cause, Target to Source Transparent Container, Serving GW change indication, EPS Bearer Setup List, AMF Tunnel Endpoint Identifier for Control Plane, Addresses and TEIDs). The EPS Bearer Setup list is the combination of EPS Bearer Setup list from different SMF+PGW-C(s).

15.	If indirect data forwarding applies, the MME sends the Create Indirect Data Forwarding Tunnel Request/Response.

14. The AMF sends the message Forward Relocation Response (Cause, Target to Source Transparent Container, Serving GW change indication, EPS Bearer Setup List, AMF Tunnel Endpoint Identifier for Control Plane, Addresses and TEIDs). The EPS Bearer Setup list is the combination of EPS Bearer Setup list from different SMF+PGW-C(s).

****************** 2nd changes ******************

##### 5.2.8.2.6	Nsmf_PDUSession_Update SM Context service operation

Service operation name: Nsmf_PDUSession_Update SM Context.

Description: It allows to update the AMF-SMF association to support a PDU Session and/or to provide SMF with N1/N2 SM information received from the UE or from the AN.

Input, Required: SUPI, PDU Session ID.

Input, Optional: N1 SM container received from the UE, N2 SM information received from the AN (e.g. N3 addressing information, notification indicating that the QoS targets cannot be fulfilled for a QFI), Operation type (e.g. activate, deactivate), Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding during HO from 5GS to EPS, UE location information, AN type, UE Time Zone.

Output, Required: Result Indication.

Output, Optional: Cause, EBI list, N2 SM information (e.g. QFI, UE location information, notification indication indicating that the QoS targets cannot be fulfilled), N1 SM container to be transferred to the AN/UE.

See clause 4.3.3.2 and clause 4.3.3.3 for an example usage of this service operation.

If the consumer NF is AMF and the SMF determines that some EBIs are not needed, the SMF will put the EBIs back.

*************** End of the changes****************