SA WG2 Meeting #125	S2-181245

22 - 26 January 2018, Gothenburg, Sweden	(revision of S2-180161,0827,0923)

Source:	Ericsson, InterDigital Inc.

Title:	23.724: Adding small data frequent communication key issue

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	FS_5G_CIoT / Rel-16

Abstract of the contribution: Small data frequent communication key issue

# Discussion

The frequent small data key issue and its basic architectural requirement and baseline have been added. The Key issue description and Architecture requirements are based on the Rel-14 CIoT study documented in TR 23.720 clauses 5.3.2 & 5.2.2.

# Proposal

It is proposed to add the below key issue for small data frequent communication to the 5G CIoT TR 23.724.

***** First Change *****

# 5	Key Issues

## 5.X	Key Issue X: Frequent small data communication

### 5.X.1	Description

This key issue aims at providing a solution to support efficient frequent small data transmissions for CIoT, e.g. tracking devices for both Mobile Originated (MO and Mobile Terminated (MT) use cases. It is expected that the number of such devices can increase exponentially, but the data size per device will remain small. Traffic characteristics for UEs used for CIoT using frequent small data transmissions may lead to inefficient use of resources in the 3GPP system and high UE power consumption without use of appropriate optimization.

Frequent small data communication targets optimizations that can meet both architecture requirements on UE power consumption and resource efficient system signalling in a balanced way. A traffic pattern is assumed where small data transmissions may occur from a few small data transmissions per hour to multiple small data transmissions per minute.

The objective of this key issue is to ensure optimized transmission with IP based and non-IP based messaging protocols. Examples are LWM2M, CoAP, MQTT-SN, NIDD, HTTP 1.1, HTTP 2. Failure to do so may lead to sub-optimal transmission characteristics with increased signalling and UE power consumption.

### 5.X.2	Architectural requirements

In addition to the common requirements in clause 4, the following architecture requirements shall be supported in 5GS:

-	It shall be possible to transmit frequent small data with consideration of resource-efficiency and UE power efficiency;

-	Minimal Access Stratum changes required for NB-IoT UEs to connect to 5GS

-	Support for delivery of IP data and Unstructured (Non-IP) data;

-	Support for the following small data transmissions:

-	single packet transmission (UL or DL);

-	dual packet transmission (UL with subsequent DL, or DL with subsequent UL);

-	multiple packet transmission (one or a few UL and/or one or a few DL (in any combination or order));

-	An at least equivalent level of security mechanisms for CIoT in 5G system as in EPS;

-	Equivalent or reduced level of power consumption for UEs used for CIoT in 5GS system as in EPS

-	Support for charging, roaming, policy control

### 5.X.3	Architectural baseline

The following are considered architecture baselines for this key issue:

-	The 5GS network function terminating the T8 NIDD API is assumed to support message based charging.

-	The AF (i.e. SCS/AS) shall be able to access frequent small data service via an API equivalent to T8 NIDD API.  The AF (i.e. SCS/AS) shall be able to access frequent small data communication over N6.

### 5.X.4	Open issues

The following open issues for supporting key issue Small data frequent communication:

-	How to enable frequent small data transmissions (see clause 5.X.1) in a resource-efficient and UE power efficient way in 5GS;

***** End of Changes *****