SA WG2 Meeting #125	S2-181375

22-26th January 2018, Gothenburg, Sweden
	(revision of 1048, was 0954, was S2-180060+0413+0551+0351)

Source:	Nokia, Nokia Shanghai Bell, Ericsson, CATT, Huawei

Title:	FS_ETSUN / 23.726 Key Issue: Support of the preservation of a PDU Session while the UE moves out of an SMF service area

Document for:	Approval

Agenda Item:	6.12

Work Item / Release:	FS_ETSUN / Rel-16

Abstract of the contribution: adding Key Issue: Support of the preservation of a PDU Session while the UE moves out of an SMF service area

# Introduction

This KI addresses item 1 of following 2 objectives of the SID

- investigate mechanisms to enable the 3GPP system to support deployments where a SMF is not able / allowed to control UPF(s) throughout the same PLMN.

- Study whether it is needed to enhance the capabilities for a UPF to be controlled by multiple SMF’s (and many UPF’s to be controlled by many SMFs) and, if yes, define such enhancements

# Proposal

It is proposed to update TS 23.726 as follows:

* * * Change * * * * all text new

* * * Change * * * *

# 5	Use cases and Key Issues

## 5.1	 Use Cases

## 5.1.x	 Use Case #X: deployments where a SMF is not able / allowed to control UPF(s) throughout the whole PLMN

In large countries the network may be divided into “regions” that each are locally (“regionally)” managed and where it is required that UPF deployed in a “region” are only “controlled” (via N4) by SMF(s) in the same “region”. This is called a “regional control” of UPF.

The SMF may be not able to control UPFs throughout the whole PLMN. When the UE moves to a new area, there will be no UPF under the SMF control can connect the target NG-RAN.

Figure 5.X-1: Use case 1: Mobility across 2 regions within a PLMN

## 5.1.Y	 Use Case #Y: inter PLMN mobility

Figure 5.Y-2: Use case 2: inter PLMN mobility

## 5.1.Z	 Use Case #Z: Third party (corporate)

For some DNN(s) the SMF controlling the PSA is dedicated to a Third party (corporate) for some DNN(s) where for such DNN(s):

- The dedicated SMF controls UPF deployed within the corporate premises (e.g. UPF that serve UE(s) using a non 3GPP access to reach the 5GC)

- The UPF(s) that serve UE using 5G-AN not operated by the Third party are operated by the MNO and should be controlled by another SMF (of the MNO)

Editor's Note: Whether the SMF that controls the PDU Session and the PDU Session Anchor (PSA) is under control of the Third party is FFS.

Figure 5.Z-3: Use case 3: Corporate case

## 5.2	 Key Issues

## 5.2.X	 Key Issue #A: Control of UPFs in use cases X, Y, Z

(IP type) PDU sessions in SSC mode 1 need IP address preservation even though the UE moves between regions or between different 5G-AN corresponding to different domains. Hence, after UE mobility, while the (I-)UPF connected to the 5G-AN serving the UE is now in a different region / administrative domain, the PSA needs to remain in its initial location and controlled by an SMF in the same region or administrative domain. Thus, the SMF controlling the PSA needs to remain despite the UE mobility.

The KI is to investigate mechanisms to enable the 3GPP system to support deployments where each UPF is only controlled by a single SMF and the SMF that controls the main PSA cannot control all UPF serving the PDU Session. Depending on the use case, the mechanisms studied may be based on solutions to objective 1 and/or objective 2.

The solutions shall take into account and/or define:

- UE’s should not be aware of the split of SMF & UPF in different regional / administrative areas.

- The solutions shall define SMF area and their Granularity (TAI?, Others?)

- Impact of the UE mobility within 5GS (mobility between different SMF areas) but also mobility with EPS, e.g. the case where an UE moves from 5GS, to EPS and then back to 5GS but in a different region / administrative area

-	Especially the solutions shall describe how to detect mobility between different SMF areas

- If in a solution (within HPLMN or within VPLMN), a PDU Session may be simultaneously controlled by multiple SMF, the solution shall describe:

-	Which NF is responsible for SMF selection or reselection, and how is it enforced

-	How to take into account slicing

-	Is there any impact to NRF? Are dedicated NRF needed or assumed?

-	How to insert an UPF supporting UL-CL/BP (IPv6 multi-homing) which is not controlled by the SMF that controls the main PSA of the PDU Session. The UL-CL/BP to be inserted may be at the same or different regional / administrative areas.

NOTE:	Mobility aspects are described in Key Issue #[ref KI of HW tdoc]

- How to ensure proper data collection to support charging

- Home Routed roaming deployments where the VPLMN may support regional control of UPF

-	Is the H-SMF aware / involved when multiple SMF are used (possibly not simultaneously) in the VPLMN?

-	Impacts to Rel-15 procedures, including MM and SM procedure, e.g. Mobility procedures and PDU Session Establishment / Modification / Release procedures, the support of LADN, the support of the different SSC modes, the activation / deactivation of the UP of the PDU Session (e.g. because the UE moves between CM-IDLE and CM-CONNECTED)

-	Potentials relationship with the different SSC modes and with AF influence on traffic routing (23.501 § 5.6.7)

-	Any impacts to UPF selection

-	Potential security impacts