SA WG2 Meeting #126	S2-181967

26 February -02 March, 2018, Montreal, Canada

Source:	Huawei, HiSilicon

Title:	Use Case: NWDA-assisted detection of situations in E2E QoS Profiles

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes a new use case in TR23.791 focus on the use of NWDAF to assist the detection of situations in E2E QoS profile of 5G scenarios.

# 1 Discussion

Latest 3GPP technical specifications (TS 23.501 and TS23.503) define the NWDAF (Network Data Analytics Function). In Rel.15 the focus of NWDAF was to support PCF and NSSF operation with data analytics based on load level information from network slices.

Extracted from TS 23.503, Section 6.2.9:

### Network Data Analytics Function (NWDAF)

NWDAF represents operator managed network analytics logical function. NWDAF provides slice specific network data analytics to the PCF and NSSF. NWDAF provides network data analytics (i.e., load level information) to PCF and NSSF on a network slice level and the NWDAF is not required to be aware of the current subscribers using the slice. NWDAF notifies / publishes slice specific network status analytic information to the PCF(s) and NSSF that are subscribed to it. PCF(s) and NSSF can collect directly slice specific network status analytic information from NWDAF. This information is not subscriber specific. PCF uses that data in its policy decisions. NSSF may use the load level information provided by NWDAF for slice selection.

NOTE 1:	NWDAF functionality beyond its support for Nnwdaf is out of scope of 3GPP.

NOTE 2:	NWDAF functionality for non-slice-specific analytics information is not supported in this release.

Nevertheless, with the establishment of the Study of enablers for Network Automation for 5G (FS_eNA), it becomes possible to investigate the usage of analytics (i.e., NWDAF) for supporting other NFs and the diverse services that the NFs will have to handle due to the introduction of different 5G scenario requirements as specified in TS 22.261.

Extracted from TR 23.791, Section 1:

# 1	Scope

The aim of this Technical Report is to study and specify how to collect data and how to feedback data analytics to the network functions.

One example of a 5G scenario defined in TS 22.261 is the URLLC. This type of scenario requires E2E QoS requirements (e.g., 5ms E2E latency for remote driving and 99.999% of reliability). The detection of situations that affect the E2E QoS provisioning is important in order to support the 5G user experience as stated in:

Extracted from TS 22.261 Section 6.7.1:

“Further, for QoS control in EPS only covers RAN and core network, but for 5G network E2E QoS (e.g., RAN, backhaul, core network, network to network interconnect) is needed to achieve the 5G user experience (e.g., ultra-low latency, ultra-high bandwidth).”

The E2E QoS in 5G scenarios is affected by parameters from AN, CN, and application, as illustrated in the figure below.

Furthermore, a new study item in V2X have been approved in SA1 (FS_V2XIMP approved in S1-180451_ FeV2XIMP). The objects of this study item show that 5GS needs to support analytics for 5G scenarios, such as V2X.

Extracted from S1-180451 Section 4:

Mobile networks have at least the following problems that difficult the detection of situations that affect E2E QoS:

- Information is compartmentalized inside each one of the parts of the network (e.g., AN, CN, and application Server) limiting the capability of analysing them together. Few mechanisms exist to bring information from different parts of the mobile network to support decision making processes, such as PCC has the RAN Congestion Awareness Function (RCAF) which gives the RAN congestion info to support PCRF decision making (TS 23.203). Nevertheless these mechanisms are not able to provide analytics on E2E QoS as 5G scenarios require.

- The mechanisms used for load and status reporting are entity (i.e., Network Function) based and using aggregated values. For instance, in the GTP-C load and overload control mechanism (TS 29.274) nodes can report their load information which is based on aggregated values such as “used capacity of the node based on activated bearers in relationship to maximum number of bearers the node can handle” (TS 29.274, Section 12.2.5.1.2.2). The problem is that, only based on this type of aggregated information it is not possible to identify situations affecting the E2E performance of specific types of services (e.g., URLLC).

Therefore, in this document we propose the use case where NWDAF is used to collect and analyse information from different sources in 5GS, i.e., UE, AN, CN, and application Server so to provide notifications to 5GS about situations that affected E2E QoS profile.

# 3 Proposal

We propose to introduce the following changes to the TR 23.791.

* * * * Start of Change * * * *

### 5.1.X	Use Case X: NWDA-assisted detection of situations in E2E QoS Profiles

#### 5.1.x.x	Description

Operators already use mechanisms for collection of information that enable CP and OAM to detect situations affecting the QoS of provided services. These mechanisms are designed in a compartmentalized way (i.e., targeting AN, or CN) and limited to per NF information data collection (e.g., TS 32.426 defines measurements related to EPC entities – such as MME, etc - which are aggregated values such as mean number of dedicated EPS bearers in active mode).  In 5G, there will be different scenarios, such as URLLC (as defined in TS 22.261) and V2X (as recently defined in the SA1 FS_V2XIMP), that require, respectively, E2E QoS (which includes at least 5GS, and application parameters) and prediction of QoS profile. Using current mechanisms, operators will have difficulties to detect situations in E2E QoS profiles per slice and/or per type of service (i.e, performance requirements for groups of UEs associated with a type of service, e.g. V2X).

NWDAF can be leveraged to collect information from the different sources, 5GS, and Application Server, and provide analytics about E2E QoS profile per slices and/or per types of services, and not only per NF basis as it is available to operators today. By analyzing together mobile information (i.e., AN and CN) with application information (AF), operators will have the feedback necessary to better tune the network operation to offer the expected E2E QoS profile. For instance, by using feedback that correlates AN, CN, and AF information, operators can trigger changes in their networks and AF only if the problem of E2E QoS resides on their network, because E2E QoS can also be affected by the environment between the mobile network and the AF itself, which is not controlled by operators so far.

This use case is required to study:

- Which information from 5GS and Application Server can be used to enable generation of analytics related to E2E QoS profile per slice basis and/or per type of service basis;

- How can the feedback generated by NWDA hint the specific contributors of situations that affected E2E QoS profile.

* * * End of all change * * *