3GPP TSG-SA WG2 Meeting #126	S2-182662

Montreal, Canada, 26 February – 2 March, 2018                                    (was S2-181478)

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0009 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS clarifications for EPC interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-01-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UE and PGW-C+SMF need to store some association between the EPS PDN connection parameters and 5GS PDU session parameters. One example is the PDU Session ID: the UE and the PGW-C+SMF need to store the mapping between the PDU session ID allocated by the UE and the EBI allocated by the PGW-C+SMF to map the information during handover procedures from EPS to 5GS. Other examples are the association between EPS bearers and 5G QoS Rules, and between 5G QoS Flow and the EPS QoS parameters, they should be stored in the UE and PGW-C+SMF in order to delete the 5G QoS Rule when the EPS bearer is deleted, or delete the EPS QoS parameters when the 5G QoS Flow is deleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE and the network need to store the association between the PDU Session ID and the default EBI, and the association between the QoS flow and the mapped EPS QoS parameters, and association between EBI and the 5G QoS Rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarity of the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR is exactly as the agreed Draft CR |  |  |  |  |  |  |  |  |

## *** Start of the change ***

#### 4.11.1.1	General

N26 interface is used to provide seamless session continuity for single registration mode.

When the UE is served by the 5GC, UE has one or more ongoing PDU Sessions each including one or more QoS Flows. During PDU Session establishment and GBR QoS flow establishment, PGW-C+SMF performs EPS QoS mappings and allocates TFT with the PCC rules obtained from the PCF+PCRF if PCC is deployed, otherwise EPS QoS mappings and TFT allocation are executed by the PGW-C+SMF locally. EPS Bearer IDs are allocated by the serving AMF requested by the SMF if the SMF determines that EPS Bearer ID(s) needs to be assigned to the QoS Flow(s). For each PDU Session, EPS bearer ID(s) are allocated to the default EPS bearer which non GBR flows are mapped to, and allocated to dedicated bearers which the GBR Flows that are mapped to in EPC. The SMF shall be able to selectively determine for some of the GBR flows to allocate EPS Bearer IDs based on QoS profile and operator policies. For Ethernet and Unstructured PDU Session Types, only EPS Bearer ID for the default EPS Bearer is allocated. The EPS Bearer ID(s) for these bearer(s) are provided to the UE, NG-RAN and PGW-C+SMF by AMF. The UE is also provided with the mapped QoS parameters. The mapped EPS QoS parameters may be provided to PGW-C+SMF by the PCF+PCRF, if PCC is deployed. The UE and the PGW-C+SMF store the association between the QoS Flow and the corresponding EBI and the EPS QoS parameters. When the QoS Flow is deleted e.g. due to PDU Session status synchronization or PDU Session Modification, the UE and the PGW-C+SMF delete any possibly existing EPS QoS parameters associated with the deleted QoS Flow.

In this release, for a PDU Session for a LADN, the SMF doesn't allocate any EBI or mapped QoS parameters.

When the UE is served the EPC, UE has one or more ongoing PDN connections including one or more EPS Bearers. During PDN connection establishment, the UE allocates the PDU Session ID and sends it to the PGW-C+SMF via PCO and other 5G QoS parameters corresponding to PDN connection, e.g. Session AMBR and QoS rules, are allocated by the PGW-C+SMF and sent to UE in PCO. The UE and the PGW-C+SMF store the association between the EPS Context and the PDU Session Context to use it in case of handover from EPS to 5GS. During the EPS bearer establishment/modification procedure, QoS rules corresponding to the related EPS bearers are allocated and sent to UE in PCO. The 5G QoS parameters are stored in the UE and are to be used when the UE is handed over from EPS to the 5GS. The 5G QoS parameters may be provided to PGW-C+SMF by the PCF+PCRF, if PCC is deployed. The UE and the PGW-C+SMF store the association between the EPS bearer and the corresponding 5G QoS Rules. When the EPS bearer is deleted e.g. due to EPS bearer status synchronization or bearer deactivation, the UE and the PGW-C+SMF delete any possibly existing 5G QoS Rule associated with the deleted EPS bearer.

In the roaming case, if the VPLMN supports interworking with N26, the UE shall operate in Single Registration mode.

During the 5GS-EPS handover, indirect forwarding may apply for the downlink data forwarding performed as part of the handover. From its configuration data the AMF knows whether indirect forwarding applies and it requests to allocate downlink data forwarding paths on UPFs for indirect forwarding. From its configuration data the MME knows whether indirect forwarding applies and it requests to allocate downlink data forwarding paths on Serving GWs for indirect forwarding. It is configured on AMF and MME whether indirect downlink data forwarding does not apply, applies always or applies only for inter PLMN inter RAT handovers.

## *** End of the change ***