SA WG2 Meeting #127-bis	S2-185158

May 28 – June 1, 2018, Newport Beach, USA

Source:	CableLabs, Charter Communications,

Title:	DOCSIS solution for FN-CRG

Document for:	Discussion and approval

Agenda Item:	6.7

Work Item / Release:	FS_5WWC / Rel-16

Abstract of the contribution: This contribution proposes additional architecture solutions for FN cable RG.

# Introduction

This contribution proposes an additional solution architecture to cover cable fixed access network with legacy RG.

# 2	Proposal

It is proposed to adopt the following proposals and the related changes to TR 23.716.

BEGIN 1st CHANGES

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

The definitions in TS 23.501 [2], TS 23.502 [3] and TS 23.503 [8] are applicable unless differently defined in the following:

-	5G-RG: A 5G-RG is a RG capable of connecting to 5GC playing the role of a UE with regard to the 5G core. It supports secure element and exchanges N1 signalling with 5GC.

-	5G-CRG: A 5G-CRG is a cable residential gateway capable of connecting to 5GC playing the role of a UE with regard to the 5G core. It supports secure elements and exchanges N1 signalling with 5GC.

-	DOCSIS Wireline Access Network: An access network that conforms to CableLabs DOCSIS MULPI [13].

-	RG: The RG is a device capable of providing voice, data, broadcast video, video on demand, specified by BBF.

-	FN-RG: A Fixed Network RG (FN-RG) is a RG playing a role similar of a UE with regard to the 5G core. It does not support N1 signalling. The FN-RG is a RG specified by BBF TR-124 [9].

-	FN-CRG: A fixed Network cable RG (FN-CRG) is a cable RG playing the role similar of a UE with regard to the 5G core. It does not support N1 signaling. The FN-CRG is a cable modem specified by CableLabs DOCSIS MULPI [13].

-	Fixed Access Gateway Function (FAGF): A Network function in W-5GAN that provides connectivity to the 5G Core of 5G-RG.

-	Hybrid Access 5G-RG: it is 5G-RG capable of connecting simultaneously via both 5G RAN and wireline access network to 5GC.

-	Wireline 5G Access Network: The Wireline 5G Access Network (W-5GAN) is a wireline AN that connects to a 5GC via N2 and N3 reference points.

-	Wireline 5G Cable Access Network: The Wireline 5G Cable Access Network (W-5GCAN) is a DOCSIS Wireline Access Network that connects to a 5GC via N2 and N3 reference points.

-	Wireline Access Network: An access network (optical or electrical) which is conform to BBF TR-101 [10] or BBF TR-178 [11].

-	Non-3GPP Access Network: In the context of this TR a Non-3GPP Access Network is an access network that is not defined by 3GPP including for example the Wireline 5G Access Network.

Editor's note:	The definition of Trusted Non-3GPP is FFS.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5G-RG	5G Residential Gateway

5G-CRG	5G Cable Residential Gateway

W-5GAN	Wireline 5G Access Network

W-5GCAN	Wireline 5G Cable Access Network

BBF	Broadband Forum

CPE	Customer Premises Equipment

FAGF	Fixed Access Gateway Function

FN-RG	Fixed Network RG

FN-CRG	Fixed Network Cable RG

HA	Hybrid Access

RG	Residential Gateway

BEGIN 2nd CHANGES

#### 4.2.2.x	Scenario 9

The following architectural principles apply for a FN-CRG (not N1 capable CPEs/CRGs):

1.	FN-CRG does not support any 3GPP procedure.

NOTE:	The scenario of FN-CRG connected to 5GC via NG RAN and the Hybrid Access scenario (i.e. connected via W-5GAN and NG RAN) are not applicable.

2.	The impact on 3GPP reference points shall be minimized.

3	The W-5GCAN shall support an interworking function that enables FN-CRG to be connected to 5GC.

4	The Interworking function interfaces the 5GC using the 3GPP reference points N1, N2 and N3 as depicted in figure 4.2.2.x-1.

Figure 4.2.2.x-1: High level architecture for the FN-CRG case

BEGIN 3rd CHANGES

#### 4.2.2.5	Scenario 10

In addition to scenario 9, the following architectural principles apply for a 5GC capable UE behind a FN-CRG:

1.	5GC capable UE shall use 3GPP procedures and impact on 3GPP reference points shall be minimized.

2	FN-CRG does not support any 3GPP procedure.

NOTE 1:	The scenario of FN-CRG connected to 5GC via NG RAN and the Hybrid Access scenarios (i.e. connected via W-5GCAN and NG RAN) are not applicable.

3.	FN-CRG shall transparently enable direct N1 signalling between the 3GPP UE and AMF/SMF.

4.	FN-CRG shall transparently enable direct UP transport between the 3GPP UE and UPF.

5.	FN-CRG and 5GC shall enable the 5GC capable UE to exchange traffic with the other devices on the Home LAN network.

6.	For the interfaces between the W-5GCAN and the 5GC the same principles defined for scenario 9 bullet 3 and bullet 4 apply.

7.	5GC shall be able to support the functionality for user-plane treatment in the UPF defined in TS 23.501 [2] clause 6.2.3 also for a 3GPP UE behind a FN-CRG. This includes being the anchor point for the PDU session of the 3GPP UE when moving between FN-CRG and 3GPP access.

8.	W-5GCAN may be enabled to support QoS treatment of QoS flows for a 3GPP UE using FN-CRG access. This implies that the W-5GCAN may need to simultaneously manage QoS resources for PDU Session of the FN-CRG as well as PDU Session of 3GPP UEs connecting via the FN-CRG.

Figure 4.2.2.x-1: High level architecture for the 5GC Capable UE behind a FN-CRG case

NOTE 2:	The reference points N2 and N3 might be impacted on protocol level.

9	The 5GC capable UE can be registered and have PDU Sessions with active user planes via multiple accesses at the same time (i.e. via FN-CRG and NG RAN) as depicted in figure 4.2.2.x-2. These requirements apply:

a.	5GC and 5GC capable UE shall be able to support ATSSS functions between 3GPP access and FN-CRG access.

Figure 4.2.2.x-2: High level architecture for the Hybrid access 5GC capable UE connected via NG RAN and FN-CRG

BEGIN 4thCHANGES

#### 4.2.2.6	Scenario 11

In addition to scenario 9, the following architectural principles applies for a Non-5GC capable device behind a FN-CRG:

Editor's note:	In the following it is assumed that a Non-3GPP device, e.g. a PC, tablet, does not support USIM. The exact definition will be an outcome of the study.

1.	A non-5GC capable device may be a 3GPP UE that supports EPC, or a non-3GPP device that supports neither USIM nor 3GPP signalling and consequently does not support 3GPP procedures.

2.	FN-CRG does not support any 3GPP procedure.

NOTE 1:	The scenario of FN-CRG connected to 5GC via NG RAN and the Hybrid Access scenarios (i.e. connected via W-5GCAN and NG RAN) are not applicable.

Figure 4.2.2.x-1: High level architecture for the Non-5GC Capable device behind a FN-CRG

END of CHANGE