SA WG2 Meeting #127bis	S2-185281

28 May - 01 June, 2018, Newport Beach, USA

Source:	Huawei, HiSilicon

Title:	Solution for Key Issue 2: Analytic Information Exposure to AF

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes solutions for Key Issue 2: Analytic Information Exposure to AF.

1 Proposal

It is proposed solutions for Key Issue 2: Analytic Information Exposure to AF.

* * * * Start of Change * * * *

6.X	Solution X: Analytic Information Exposure to AF

6.X.1	Description

This solution is for Key Issue 2: Analytic Information Exposure to AF.

6.X.1.1 Procedure for Analytic Information Exposure to AF by AF Request

Figure 6.x.1.1-1: Procedure for Analytic Information Exposure to AF by AF Request

1. From AF to PCF via NEF if necessary: Npcf_Service Request.

2. From PCF to NWDAF: Nnwdaf_AnalyticsInfo_Request.

3. From NWDAF to PCF: Nnwdaf_AnalyticsInfo_Request response.

4. The PCF determines one or more policies, based on analytic information provided by the NWDAF, information provided by the AF and other available information.

5. From PCF to AF via NEF if necessary: Npcf_Service Response.

6.X.1.2 Procedure to support Analytic Information Exposure to AF by AF Subscription

Figure 6.x.1.2-1: Procedure for Analytic Information Exposure to AF by AF Subscription

1. From AF to PCF via NEF if necessary: Npcf_ServiceSubscription_Subscribe / Npcf_ServiceSubscription_Unsubscribe.

2. From PCF to NWDAF: Nnwdaf_AnalyticsInfo_Request.

3. From NWDAF to PCF: Nnwdaf_AnalyticsInfo_Request response.

4. The PCF determines one or more policies, based on analytic information provided by the NWDAF, information provided by the AF and other available information.

5. From PCF to AF via NEF if necessary: Npcf_ServiceSubscription_Notify.

6.X.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

6.X.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * End of Change * * *