SA WG2 Meeting #127bis	S2-185355

28 May - 01 June, 2018, Newport Beach, USA

Source:	Huawei, HiSilicon

Title:	Solution to support mobile restriction for 5WWC

Document for:	Approval

Agenda Item:	6.7

Work Item / Release:	5G_ph1/Rel-16

Abstract of the contribution: This contribution proposes the solution to support mobile restriction.

Discussion

This paper propose the solution to mobile restriction.

5WWC is the convergence of wireline and wireless network and has similar service requirement with wireline network. In the existing fixed network, it is required that the RG/CPE can only register to the network in one or more specific location, e.g. in specific Home or in specific ROOM. Accordingly, 5WWC can adopt similar requirement as the existing fixed network.

It is proposed to adopt the mechanism of Registration restriction which is similar to the service area restriction mechanism in TS 23.501 and TS 23.502. However some adaptions listed below are necessary.

- In order to be consistent with the existing fixed network mechanism, Registration restriction is the mechanism to define areas in which the 5G-RG can or cannot register to the 5GC which is different from the service area restriction.

- For the scenario 5G-RG is connected to the Broadband Wireline network, the location information used to perform Registration restriction can be one or more Line ID including the “circuit ID” and the “remote ID” defined in RFC 3046.

- For the scenario 5G-RG is connected to the NG-RAN, the location information can be one or more Cell IDs instead of TA since 5G-RG is assumed not support mobility in a large-scale area.

Proposal

* * * Begin of Changes * * *

6.x	Solution #x – Mobile restriction and LADN support

This is the solution to Key issue #13.

NOTE 1:	LADN is not applicable in this solution.

6.x.1 Description

Registration Restrictions is the mechanism to perform management of Mobile Restriction for WWC.

Registration Restrictions for 5WWC:

Defines areas in which the 5G-RG can or cannot register to the 5GC as follows:

-	WWC Allowed Area:

WWC Allowed Area is the area where the 5G-RG is allowed to register to the 5GC based on subscription.

-	WWC Non-Allowed Area:

WWC Non-Allowed Area in an area where the 5G-RG is not allowed to register to the 5GC based on subscription.

NOTE :	The WWC Allowed Area and WWC Non-Allowed Area are defined per Access type, i.e. 3GPP for the case of 5G-RG connected via NG RAN and W-5GAN for the case of 5G-RG connected via W-5GAN.

For the architecture of Figure 6.1.x-1, Figure 6.1.x-2 in clause 6.1, 5G-RG is connected to the Broadband Wireline network. A Registration Restriction may contain Location Information in wireline access network, for example one or more Line ID including the “circuit ID” and the “remote ID” defined in RFC 3046[x].

For the architecture of Figure 6.1.x-3, Figure 6.1.x-4 in clause 6.1, 5G-RG is connected to the NG RAN. A Registration Restriction may contain Location Information in wireless access network, for example one or more Cell ID.

The 5G-RG's subscription data in the UDM includes a Registration Restriction which may contain either WWC Allowed or WWC Non-Allowed Areas specified by using explicit Location Information (Line ID or when 5G-RG is connected to the Broadband Wireline network, or Cell ID when 5G-RG is connected to the NG RAN) and/or other geographical information (e.g., longitude/latitude, zip code, etc). The geographical information used to specify WWC allowed or WWC non-allowed area will be mapped to Location Information (Line ID or when 5G-RG is connected to the Broadband Wireline network, Cell ID when 5G-RG is connected to the NG RAN).

The UDM stores the Registration Restrictions of a 5G-RG as part of the UE's subscription data. The PCF in the serving network may (e.g. 5G-RG to varying conditions such as UE's location, application in use, time and date) further adjust Registration Restrictions of a UE, either by expanding an WWC allowed area or by reducing a WWC non-allowed area. The UDM and the PCF may update the Registration Restrictions of a 5G-RG at any time.

During registration, if the Registration Restrictions of the 5G-RG is not present in the AMF, the AMF fetches from the UDM the Registration Restrictions of the 5G-RG that may be further adjusted by the PCF. The serving AMF shall enforce the Registration Restrictions of a UE.

Editor's note:	whether and how the VLAN ID is used as parameter for Registration Restriction when is dynamically assigned in W-5GAN is FFS

6.x.2 	Procedure

Editor's note:	The impacts to the procedures, e.g. Registration Procedure, are FFS.

6.x.3	Solution Evaluation

Editor's note:	The solution evaluation is FFS.

* * * End of Changes * * *