SA WG2 Meeting #S2-127bis	S2-185438

Newport Beach, CA, USA, 28 May-1 June 2018

Source:	Vodafone

Title:	Evaluation of Solution 4 “AF Control of Make-before-break PDN Connection”

Document for:	Approval

Agenda Item:	6.17

Work Item / Release:	FS_LLC_Mob / Rel-16

# Proposal

It is proposed to make the following revision marked changes to sections 6.4.4. and 6.4.5 of TR 23.739 v0.3.0.

Sections 6.4.1-6.4.3 are provided below just “for information”.

***************** Start of Changes ******************************************

## 6.4	Solution #4: AF Control of Make-before-break PDN Connection

### 6.4.1	Introduction

The solution addresses Key Issue #2 and Key Issue of creation of make-before-break PDN connection. It is supplementary to Solution 3, where the AF triggers the IP address reallocation instead of the PGW.

### 6.4.2	Functional Description

An example deployment to which both the solutions apply is the shown in the figure below.

Figure 6.4.2-1: Architecture of LLC Data network

Details are described in clause 6.2.2.1. In addition to the description in clause 6.3.2.1, the following is assumed:

-	AF is aware that edge compute resources are available in PRA1 (edge data center 1) and PRA2 (edge data center 2) and that there is a PGW-U for breakout to the edge compute resources.

Editor's note:	If is FFS how the AF is made aware of location corresponding to where edge compute resources are placed and there is a PGW-U for breakout to the edge compute resources.

### 6.4.3	Procedure

Depicted in Figure 6.4.3-1 below is the procedure for AF control of "make-before-break" PDN connection mobility in EPS.

Figure 6.4.3-1: "Make-before-break" PDN connection controlled by AF

1a-1d.	UE Requested PDN Connectivity procedure or Attach with PDN Establishment from TS 23.401 [3] is performed for LLC APN. The S/PGW-C selects a user-plane function S/PGW-U1 which is in the central data center. This establishes a PDN connection 1.

2a-2h.	The mobile communicated with AF. The AF creates an Rx session with the PCRF and subscribes to location update change notification for PRA-list. The PRA-list is the list of edge data centers.

3a-3e.	The UE moves (handover or idle-mode) to eNB that is in PRA1. The UE performs tracking area update. The MME then sends location update notification as described in TS 23.401 [3] to the S/PGW-C. The S/PGW-C further notifies the PCRF which then notifies the AF.

4a-4b.	The AF determines that there are edge compute resources in PRA which can benefit from the UE having an IP address allocated from user-plane node in PRA1. The AF provides a new trigger to the PCR that the a LLC  is required for this service  This trigger is then provided to the S/PGW-C.

5.	Steps  5-13 from Figure 6.2.3.1 are performed where the PGW-C triggers a a new PDN connection  This establishes a PDN connection 2.

Editor's note:	It is FFS how to select a PGW-U based on PRA information.

The PDN connection1 is released. The UE is assumed to switch application traffic from PDN connection1 to PDN connection 2.

6.	UL/DL traffic flows through S/PGW-U2.

The following steps are new:

1.	Step 2b, AF subscription to PRA changes.

2.	Step 4a and Step 4b, Provision of an indication that a LLC PGW-U should be allocated change anchor for the PDN Connection.

### 6.4.4	Impacts on existing entities and interfaces

Rx (AF and PCRF):

-	Subscription to PRA change location notification on Rx interface.

Gx:

-	Support of a request for LLC for this service.

PGW-U Selection:

-	PGW-U selection based on PRA-ID.

### 6.4.5	Evaluation

The solution is similar to the SSC mode 3 functionality in the 5G System (refer to TS 23.502 [5] clause 4.3.5.2, with the difference being that the logic for PGW-U relocation is within the AF.

The solution supports roaming with Local BreakOut.

The solution is aware of the use (or non-use) of applications that require low latency. This avoids the PLMN needing to install 'low latency' GW capacity for all UEs that might, occasionally need low latency service.

There is no MME impact.

************* end of changes **************