SA WG2 Meeting #127bis	S2-186289

28 May – 1 Jun, 2018, Newport Beach, CA, USA	(revision of S2-186270, S2-185990, S2-185310)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0480 | rev | 3 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MT-SMS over NAS re-transmission upon delivery failure on one Access Type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (UK) Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 30-04-2018 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current specification in 23.502 subclause 4.13.3.9 reads: …… “If the AMF informs the SMSF that it cannot deliver MT-SMS to UE, and if MT SMS domain selection is supported, SMSF may get another possible entity address for re-attempting the delivery via UDM and forwards the MT SMS to the chosen entity.” It is unclear whether AMF should inform SMSF immediately after the MT-SMS delivery after one Access Type has failed, or AMF should retry the delivery by itself on the other Access Type, and only inform SMSF after the delivery on both accesses have failed, in case UE can access to the AMF via both 3GPP and non-3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AMF can retry MT-SMS delivery through the other Access Type by itself. AMF shall inform SMSF about the delivery failure after the delivery on both access types has failed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how SMSF and AMF should handle the MT-SMS retransmission when both 3GPP and non-3GPP accesses are available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 4.13.3.9	Unsuccessful Mobile terminating SMS delivery attempt

The procedure of Unsuccessful Mobile terminating SMS delivery is defined as follows:

-	If the UE is registered over both 3GPP access and non-3GPP access:

-	if SMS over NAS is supported via both access types, the AMF may re-attempt the MT-SMS delivery. The AMF determines the Access Type to re-transfer the MT-SMS based on operator local policy;

-	the AMF shall inform the SMSF immediately after the MT-SMS delivery on both access types has failed.

-	If the AMF informs the SMSF that it cannot deliver the MT-SMS to the UE:

-	if MT-SMS domain selection is supported, the SMSF may get another possible entity address for re-attempting the delivery via UDM and forwards the MT-SMS to the chosen entity. If SMSF cannot obtain any entity address, or SMSF receives failure report from the chosen entity, the SMSF sends a failure report to the entity from which SMSF receives this SMS message;

-	if domain selection is not supported, the SMSF sends a failure report to the SMS-GMSC as defined in TS 23.040 [7].

-	After the first SMS-GMSC informs the UDM/HSS that the UE is not able to receive MT-SMS, the UDM/HSS shall set its internal Mobile Station Not Reachable Flag (MNRF).

-	If the UDM/HSS has not subscribed UE Reachability Notification, it immediately initiates a subscription procedure as specified in clause 4.2.5.2.

-	When the AMF detects UE activities, it notifies UDM/HSS with UE Activity Notification as described in clause 4.2.5.3. The UDM/HSS clears its MNRF and alerts related SMSCs to retry MT-SMS delivery.