3GPP TSG-SA2 Meeting #127	S2-183371

# April 16 - April 20, Sanya, China

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0243 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configured NSSAI update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_ph1 |  |  |  |  |  |  | Date: |  | 2018-04-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. According to the agreed S2-182835 and S2-182983, the Configured NSSAI for a UE for a PLMN can only be updated when UE has registered in the PLMN. Considering the following scenarios: 1) When the Configured NSSAI for PLMN 1 is changed while UE is roaming in PLMN 2, the new Configured NSSAI for PLMN 1 will not be sent to the UE as it registers in another PLMN. Then when UE registers to the PLMN 1 it still stores the old Configured NSSAI for the PLMN 1 and the UDM or AMF in the PLMN 1 may not trigger update for the Configured NSSAI. The UE may have no chance to include the newly added S-NSSAI, if any, in the Request NSSAI for PLMN 1. 2) If UE's Subscribed S-NSSAIs are changed when the UE RM state in the AMF is RM-DEREGISTERED, the Configured NSSAI for the PLMN UE registers cannot be updated in the UE. As the Requested NSSAI can be a subset of the Configured NSSAI for the PLMN, if the UE Configured NSSAI for a PLMN is not updated successfully on the UE, in some scenarios the network cannot be aware of the inconsistence just based on the received Requested NSSAI when UE registers. Based on the discussion in S2-183344, it is proposed a solution to resolve the problem in the above scenarios. 2. A trigger for Configured NSSAI update is proposed: if the Requested NSSAI includes an S-NSSAI that is not permitted by the Subscribed S-NSSAIs, the AMF or NSSF may also determine to update the UE Configured NSSAI for the Serving PLMN or/and the associated mapping of the Configured NSSAI to the Configured NSSAI for the HPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) When the AMF receives a registration request from the UE, the AMF will provide the UE with the Configured NSSAI for the Serving PLMN and/or the associated mapping with Configured NSSAI for the HPLMN if the registration type is “initial registration”, or if the registration type is “mobility registration update” and the old AMF is from another PLMN (based on 5G GUTI in the registration request). 2) Add a trigger for UE Configured NSSAI update in the TS 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Configured NSSAI in UE will not be updated for those PLMNs which UE doesn’t register to when the Subscribed S-NSSAI(s) changes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** 1st Change *****

### 5.15.4	UE NSSAI configuration and NSSAI storage aspects

#### 5.15.4.1	General

A UE can be configured by the HPLMN with Network Slice configuration information.

The Network Slice configuration information contains one or more Configured NSSAI(s). A Configured NSSAI may apply either to one PLMN or to all PLMNs that do not have a specific Configured NSSAI (e.g. this could be possible for NSSAIs containing only S-NSSAIs with standard values, see clause 5.15.2.1). There is at most one Configured NSSAI per PLMN.

The Configured NSSAI of a PLMN may include S-NSSAIs that have standard values or PLMN-specific values.

The Configured NSSAI for the Serving PLMN includes the S-NSSAI values which can be used in the Serving PLMN and may be associated with mapping of each S-NSSAI of the Configured NSSAI to one or more corresponding S-NSSAI values in the Configured NSSAI for the HPLMN.

The S-NSSAIs in the Configured NSSAI for the HPLMN, at the time when they are provided to the UE, shall match the Subscribed S-NSSAIs for the UE. When the Subscribed S-NSSAI(s) are updated (i.e. removing an old and/or adding a new) and it is applicable to the Serving PLMN the UE is registered in as described in clause 5.15.3 , the AMF may update the UE with the Configured NSSAI of the Serving PLMN and/or Allowed NSSAI and/or the associated mapping with Configured NSSAI for the HPLMN.

When providing a Requested NSSAI to the network upon registration, the UE in a given PLMN only includes and uses S-NSSAIs applying to this PLMN, possibly associated with mapping of each S-NSSAI of the Requested NSSAI to the S-NSSAIs of the Configured NSSAI for the HPLMN i.e. part of the Configured and/or Allowed NSSAIs applicable for this PLMN. Upon successful completion of a UE's Registration procedure over an Access Type, the UE obtains an Allowed NSSAI for this Access Type, which includes one or more S-NSSAIs, from the AMF, possibly associated with mapping of Allowed NSSAI to Configured NSSAI for the HPLMN. These S-NSSAIs are valid for the current Registration Area and Access Type provided by the AMF the UE has registered with and can be used simultaneously by the UE (up to the maximum number of simultaneous Network Slices or PDU Sessions).

The UE might also obtain one or more rejected S-NSSAIs with cause and validity of rejection from the AMF. An S-NSSAI may be rejected:

-	for the entire PLMN; or

-	for the current Registration Area.

While it remains RM-REGISTERED in the PLMN and regardless of the Access Type, the UE shall not re-attempt to register to an S-NSSAI rejected for the entire PLMN until this rejected S-NSSAI is deleted as specified below.

While it remains RM-REGISTERED in the PLMN, the UE shall not re-attempt to register to an S-NSSAI rejected in the current Registration Area until it moves out of the current Registration Area.

NOTE 1:	The details and more cases of S-NSSAI rejection are described in TS 24.501 [47].

The UE stores (S-)NSSAIs as follows:

-	When the UE is provisioned with a Configured NSSAI for a PLMN and/or the associated mapping of this Configured NSSAI to the Configured NSSAI for the HPLMN, the Configured NSSAI and/or if present, the associated mapping shall both be stored in the UE until a new Configured NSSAI for this PLMN and/or the associated mapping are provisioned in the UE:

-	When provisioned with a new Configured NSSAI for a PLMN and/or the new associated mapping of this Configured NSSAI to the Configured NSSAI for the HPLMN, the UE shall:

-	replace any stored (old) Configured NSSAI for this PLMN with the new Configured NSSAI for this PLMN; and

-	delete any stored associated mapping of this old Configured NSSAI for this PLMN to the Configured NSSAI for the HPLMN and, if present, store the associated mapping of this new Configured NSSAI to the Configured NSSAI for the HPLMN; and

-	delete any stored Allowed NSSAI and associated mapping as well as rejected S-NSSAI for this PLMN;

NOTE 2:	It is expected that the UE keeps storing a received Configured NSSAI for a PLMN and associated mapping to the Configured NSSAI for the HPLMN even when registering in another PLMN. However, the number of Configured NSSAI and associated mapping to be kept stored in the UE for PLMNs other than the HPLMN is up to UE implementation.

-	If received, the Allowed NSSAI for a PLMN and Access Type and any associated mapping of this Allowed NSSAI to the Configured NSSAI for the HPLMN shall be stored in the UE. The UE should store this Allowed NSSAI and any associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN also when the UE is turned off;

NOTE 3:	Whether the UE stores the Allowed NSSAI and any associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN also when the UE is turned off is left to UE implementation.

-	When a new Allowed NSSAI for a PLMN and any associated mapping of the Allowed NSSAI to the Configured NSSAI of the HPLMN are received over an Access Type, the UE shall:

-	replace any stored (old) Allowed NSSAI and any associated mapping for these PLMN and Access Type with this new Allowed NSSAI; and

-	delete any stored associated mapping of this old Allowed NSSAI for this PLMN to the Configured NSSAI for the HPLMN and, if present, store the associated mapping of this new Allowed NSSAI to the Configured NSSAI for the HPLMN;

-	If received, an S-NSSAI rejected for the entire PLMN shall be stored in the UE while RM-REGISTERED in this PLMN regardless of the Access Type or until it is deleted.

-	If received, an S-NSSAI rejected for the current Registration Area shall be stored in the UE while RM-REGISTERED until the UE moves out of the current Registration Area or until it is deleted.

NOTE 4:	The storage aspects of rejected S-NSSAIs are described in TS 24.501 [47].

One or more S-NSSAIs in an Allowed NSSAI provided to the UE can have values, which are not part of the UE's slice configuration information for the Serving PLMN. In this case, the Allowed NSSAI is associated with mapping information regarding how each S-NSSAI of the Allowed NSSAI corresponds to the S-NSSAI(s) of the Configured NSSAI for the HPLMN. This mapping information is sufficient to allow the UE to associate for a given application the S-NSSAI as per NSSP of the URSP rules, as defined in clause 6.6.2 of TS 23.503 [45], with the corresponding S-NSSAI from the Allowed NSSAI.

#### 5.15.4.2	Update of UE Network Slice configuration

At any time, the AMF may provide the UE with a new Configured NSSAI for the Serving PLMN, associated with mapping of the Configured NSSAI to the Configured NSSAI for the HPLMN as specified in clause 5.15.4.1.The Configured NSSAI for the Serving PLMN and the mapping information is either determined in the AMF (if based on configuration, the AMF is allowed to determine the Network Slice configuration for the whole PLMN) or by the NSSF. The AMF provides the new Configured NSSAI as specified in TS 23.502 [3], clause 4.2.4 UE Configuration Update procedure.

When the UE performs initiate registration, or mobility registration from another PLMN (i.e. the old AMF indicated by the 5G GUTI in the Registration Request is from a different PLMN), the AMF shall provide the UE with the Configured NSSAI for the Serving PLMN and/or the associated mapping of the Configured NSSAI for the Serving PLMN to the Configured NSSAI for the HPLMN.

If the HPLMN performs the configuration update of a UE registered in the HPLMN (e.g. due to a change in the Subscribed S-NSSAI(s)), this results in updates to the Configured NSSAI for the HPLMN. Updates to the Allowed NSSAI and/or, if present, to the associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN are also possible if the configuration update affects S-NSSAI(s) in the current Allowed NSSAI.

If the VPLMN performs the configuration update of a UE registered in the VPLMN (e.g. due to a change in the Subscribed S-NSSAI(s)), this results in updates to the Configured NSSAI for the Serving PLMN and/or to the associated mapping of the Configured NSSAI for the Serving PLMN to the Configured NSSAI for the HPLMN. Updates to the Allowed NSSAI and/or to the associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN are also possible if the configuration update affects S-NSSAI(s) in the current Allowed NSSAI.

A UE for which the Configured NSSAI for the Serving PLMN has been updated and has deleted the stored Allowed NSSAI as described in clause 5.15.4.1 shall initiate a Registration procedure to receive a new valid Allowed NSSAI (see clause 5.15.5.2.1).

The update of URSP rules (which include the NSSP), if necessary, is described in TS 23.503 [45].

***** 2nd Change all new texts*****

#### 5.15.5.2	Selection of a Serving AMF supporting the Network Slices

##### 5.15.5.2.1	Registration to a set of Network Slices

When a UE registers over an Access Type with a PLMN, if the UE for this PLMN has a Configured NSSAI or for this PLMN and Access Type has an Allowed NSSAI, the UE shall provide to the network in AS layer and NAS layer a Requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wishes to register, in addition to the 5G-S-TMSI if one was assigned to the UE.

The Requested NSSAI shall be one of:

-	the Configured-NSSAI, or a subset thereof as described below, e.g. if the UE has no Allowed NSSAI for the serving PLMN; or

-	the Allowed-NSSAI for the Access Type over which the Requested NSSAI is sent, or a subset thereof and Access Type; or

-	the Allowed-NSSAI for the Access Type over which the Requested NSSAI is sent, or a subset thereof, plus one or more S-NSSAIs from the Configured-NSSAI as described below.

The subset of Configured-NSSAI provided in the Requested NSSAI consists of one or more S-NSSAI(s) in the Configured NSSAI applicable to this PLMN, for which no corresponding S-NSSAI is present in the Allowed NSSAI. The UE shall not include in the Requested NSSAI any, S-NSSAI that is currently rejected (i.e. rejected for the current Registration Area or rejected for the entire PLMN) by the network (i.e. rejected in the current registration area or rejected in the PLMN).

The UE shall include the Requested NSSAI in the RRC Connection Establishment and in the establishment of the connection to the N3IWF (as applicable) and in the NAS Registration procedure messages. However, the UE shall not indicate any NSSAI in RRC Connection Establishment or Initial NAS message unless it has a Configured NSSAI for the corresponding PLMN or an Allowed NSSAI for the corresponding PLMN and Access Type. The (R)AN shall route the NAS signalling between this UE and an AMF selected using the Requested NSSAI obtained during RRC Connection Establishment or connection to N3IWF respectively. If the (R)AN is unable to select an AMF based on the Requested NSSAI, it routes the NAS signalling to an AMF from a set of default AMFs. In the NAS signalling the UE provides the mapping of each S-NSSAI of the Requested NSSAI to the S-NSSAIs of the Configured NSSAI for the HPLMN.

When a UE registers with a PLMN, if for this PLMN the UE has not included a Requested NSSAI nor a 5G-GUTI while establishing the connection to, the (R)AN, the (R)AN shall route all NAS signalling from/to this UE to/from a default AMF. When receiving from the UE a Requested NSSAI and a 5G-S-TMSI or a 5G GUTI in RRC Connection Establishment or in the establishment of connection to N3IWF, if the 5G-AN can reach an AMF corresponding to the 5G-S-TMSI or 5G-GUTI, then 5G-AN forwards the request to this AMF. Otherwise, the 5G-AN selects a suitable AMF based on the Requested NSSAI provided by the UE and forwards the request to the selected AMF. If the 5G-AN is not able to select an AMF based on the Requested NSSAI, then the request is sent to a default AMF.

When the AMF selected by the AN receives the UE Registration request:

-	As part of the Registration procedure described in TS 23.502 [3], clause 4.2.2.2.2, the AMF may query the UDM to retrieve UE subscription information including the Subscribed S-NSSAIs.

-	The AMF verifies whether the S-NSSAI(s) in the Requested NSSAI are permitted based on the Subscribed S-NSSAIs (to identify the Subscribed S-NSSAIs the AMF may use the mapping to the S-NSSAI(s) of the Configured NSSAI for the HPLMN provided by the UE, in the NAS message, for each S-NSSAI of the Requested NSSAI).

-	When the UE context in the AMF does not yet include an Allowed NSSAI for the corresponding Access Type, the AMF queries the NSSF (see (B) below for subsequent handling), except in the case when, based on configuration in this AMF, the AMF is allowed to determine whether it can serve the UE (see (A) below for subsequent handling). The address of the NSSF is locally configured in the AMF.

NOTE 1:	The configuration in the AMF depends on operator's policy.

-	When the UE context in the AMF already includes an Allowed NSSAI for the corresponding Access Type, based on the configuration for this AMF, the AMF may be allowed to determine whether it can serve the UE (see (A) below for subsequent handling).

NOTE 2:	The configuration in the AMF depends on the operator's policy.

(A) Depending on fulfilling the configuration as described above, the AMF may be allowed to determine whether it can serve the UE, and the following is performed:

-	AMF checks whether it can serve all the S-NSSAI(s) from the Requested NSSAI present in the Subscribed S-NSSAIs (potentially using configuration for mapping S-NSSAI values between HPLMN and Serving PLMN), or all the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs in the case that no Requested NSSAI was provided or none of the S-NSSAIs in the Requested NSSAI were present in the Subscribed S-NSSAIs (see clause 5.15.3).

-	If this is the case, the AMF remains the serving AMF for the UE. The Allowed NSSAI is then composed of the list of S-NSSAI(s) in the Requested NSSAI permitted based on the Subscribed S-NSSAIs, or, if no Requested NSSAI was provided, all the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs (see (C) below for subsequent handling). It also determines the mapping if the S-NSSAI(s) included in the Allowed NSSAI needs to be mapped to Subscribed S-NSSAI(s) values. If no Requested NSSAI is provided, or the Requested NSSAI includes an S-NSSAI that is not valid in the Serving PLMN, or the Requested NSSAI includes an S-NSSAI that is not permitted by the Subscribed S-NSSAIs, the AMF based on the Subscribed S-NSSAI(s) and operator's configuration may also determine the Configured NSSAI for the Serving PLMN or/and the associated mapping of the Configured NSSAI to the Configured NSSAI for the HPLMN.

-	If this is not the case, the AMF queries the NSSF (see (B) below).

(B) When required as described above, the AMF needs to query the NSSF, and the following is performed:

-	The AMF queries the NSSF, with Requested NSSAI, mapping of Requested NSSAI to Configured NSSAI for the HPLMN, the Subscribed S-NSSAIs (with an indication if marked as default S-NSSAI), any Allowed NSSAI it might have for the other Access Type (including its mapping to the Configured NSSAI for the HPLMN), PLMN ID of the SUPI and UE's current Tracking Area(s).

NOTE 3:	When more than one UE's Tracking Area is indicated, the UE is using more than one Access Type.

-	Based on this information, local configuration, and other locally available information including RAN capabilities in the current Tracking Area for the UE or load level information for a network slice instance provided by the NWDAF, the NSSF does the following:

-	It verifies which S-NSSAI(s) in the Requested NSSAI are permitted based on the Subscribed S-NSSAIs and the mapping of Requested NSSAI to Configured NSSAI for the HPLMN. It considers the S-NSSAI(s) marked as default in the Subscribed S-NSSAIs in the case that no S-NSSAI from the Requested NSSAI are present in the Subscribed S-NSSAIs.

-	It selects the Network Slice instance(s) to serve the UE. When multiple Network Slice instances in the UE's Tracking Areas are able to serve a given S-NSSAI, based on operator's configuration, the NSSF may select one of them to serve the UE, or the NSSF may defer the selection of the Network Slice instance until a NF/service within the Network Slice instance needs to be selected.

-	It determines the target AMF Set to be used to serve the UE, or, based on configuration, the list of candidate AMF(s), possibly after querying the NRF.

-	It determines the Allowed NSSAI(s) for the applicable Access Type(s), taking also into account the availability of the Network Slice instances as described in clause 5.15.8 that are able to serve the S-NSSAI(s) in the Allowed NSSAI in the current UE's Tracking Areas.

-	It also determines the mapping of each S-NSSAI of the Allowed NSSAI(s) to the Subscribed S-SNSSAIs if necessary.

-	Based on operator configuration, the NSSF may determine the NRF(s) to be used to select NFs/services within the selected Network Slice instance(s).

-	Additional processing to determine the Allowed NSSAI(s) in roaming scenarios and the mapping to the Subscribed S-NSSAIs, as described in clause 5.15.6.

-	If no Requested NSSAI is provided or the Requested NSSAI includes an S-NSSAI that is not valid in the Serving PLMN, or the Requested NSSAI includes an S-NSSAI that is not permitted by the Subscribed S-NSSAIs, the NSSF based on the Subscribed S-NSSAI(s) and operator configuration may derive the Configured NSSAI for the Serving PLMN or/and the associated mapping of the Configured NSSAI to the Configured NSSAI for the HPLMN.

-	The NSSF returns to the current AMF the Allowed NSSAI for the applicable Access Type(s), the mapping of each S-NSSAI of the Allowed NSSAI to the Subscribed S-NSSAIs if determined and the target AMF Set, or, based on configuration, the list of candidate AMF(s). The NSSF may return the NRF(s) to be used to select NFs/services within the selected Network Slice instance(s), and the NRF to be used to determine the list of candidate AMF(s) from the AMF Set. The NSSF may return NSI ID(s) to be associated to the Network Slice instance(s) corresponding to certain S-NSSAIs. NSSF may return the rejected S-NSSAI(s) as described in clause 5.15.4.1. The NSSF may return the Configured NSSAI for the Serving PLMN and the the associated mapping of the Configured NSSAI to the Configured NSSAI for the HPLMN.

-	Depending on the available information and based on configuration, the AMF may query the appropriate NRF (e.g. locally pre-configured or provided by the NSSF) with the target AMF Set. The NRF returns a list of candidate AMFs.

-	If rerouting to a target serving AMF is necessary, the current AMF reroutes the Registration Request to a target serving AMF as described in clause 5.15.5.2.3.

(C) The serving AMF shall determine a Registration Area such that all S-NSSAIs of the Allowed NSSAI for this Registration Area are available in all Tracking Areas of the Registration Area (and also considering other aspects as described in clause 5.3.2.3) and then return to the UE this Allowed NSSAI and the mapping of the Allowed NSSAI to the Subscribed S-NSSAIs if provided. The AMF may return the rejected S-NSSAI(s) as described in clause 5.15.4.1.

NOTE:	As there is a single distinct Registration Area for Non-3GPP access in a PLMN, the S-NSSAIs in the Allowed NSSAI for this Registration Area (i.e. for Non-3GPP access) are available homogeneously in the PLMN.

Upon successful Registration, the UE is provided with a 5G-S-TMSI by the serving AMF. The UE shall include this 5G-S-TMSI in any RRC Connection Establishment during subsequent initial accesses to enable the RAN to route the NAS signalling between the UE and the appropriate AMF.

If the UE receives an Allowed NSSAI from the serving AMF over an Access Type, it shall store this new Allowed NSSAI and the associated mapping of the Allowed NSSAI to the Configured NSSAI for the HPLMN, if any, and override any previously stored Allowed NSSAI for these PLMN and Access Type, as described in clause 5.15.4.1.

When no Requested NSSAI was included or when an S-NSSAI was rejected in the PLMN, the AMF may update the UE slice configuration information for the PLMN as described in clause 5.15.4.2.

If the UE receives a Configured NSSAI for the Serving PLMN and the associated mapping of the Configured NSSAI to the Configured NSSAI for the HPLMN, the UE shall store this new Configured NSSAI for the Serving PLMN and the associated mapping information, and override any previously stored Configured NSSAI for this PLMN and the associated mapping information, as described in clause 5.15.4.1.

***** End of Change *****