SA WG2 Meeting #127	S2-183916

April. 16 – April 20, 2018, Sanya, China	(revision of S2-183651)

Source:	Intel

Title:	Solution of High latency communication and Power Saving Function

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	FS_CIoT_5G / Rel-16

Abstract of the contribution: The contribution proposes solution for KI#3 High latency communication

# 1	Discussion

The contribution proposes solution for KI#3 High latency communication.

It is proposed to add following solution to the TR 23.724. All changes are new.

################### TEXT PROPOSAL FOR THE TR 23.724 ########################

## 6.X	Solution #X: Solution for supporting High Latency Communication

### 6.X.1	Introduction

This solution addresses the Key Issue 3 - "High Latency Communication".

### 6.X.2	Functional Description

Functions for High latency communication can be used to handle mobile terminated (MT) communication with UEs being unreachable while using power saving functions in 5GS.

High latency communication is handled by notification procedures requested by NFs, e.g. AF, SMF, UPF, when a UE wakes up from its power saving state and sends downlink data to the UE when the UE is reachable. This notification procedure is based on notification service operations including UE Reachability, and Availability after DDN (downlink data notification) failure.

For UE allowing high latency communication, the following two notification service operations can be used to optimize system resources and reduce signaling overheads.:

- UE reachability notification for MT communication

- UE availability notification procedure after DDN failure

### 6.X.3	Procedures

6.X.3.1 Support for UE Reachability Notification for Mobile Terminated Communication

The network can accommodate the UE with scheduled MT communication, e.g. the AF/SMF subscribes the notification service from the AMF for the UE reachability.

For AF requesting event exposure from the AMF for the UE reachability notification, the AF can buffer the packets for MT communication before receiving the notification.

Further, the SMF can request event exposure from the AMF for the UE reachability. There are two alternatives to handle DDN due to MT communication when the UPF receives the MT packets.

Alt1:

- The UPF notifies the SMF for the downlink packets by sending DDN.

- The SMF checks the UE’s reachability and determines if it needs to send the DDN to the AMF or not.

If the UE is in power saving mode, the SMF notifies the UPF to buffer the packets and indicates the buffer size, expected time for UE’s availability, e.g. next time to be in idle state.

If the UE is in idle mode, the SMF further sends the DDN to the AMF.

Alt2:

- The SMF has notified the UPF about the buffering information according to the UE reachability.

- The UPF buffers the packets accordingly and sends the DDN only after reaching the maximum size of the buffering

Further, if the UE is in CM-CONNECTED mode with RRC INACTIVE, extended buffering may be done at RAN. If RAN based paging fails or if RAN notification update guard timer expires RAN initiates AN release and send buffered data to AMF. AMF can then initiate CN based paging and take further action as specified in clause 6.X.3.3.2 Step 4 onwards.

6.X.3.2 Support for UE Availability Notification Procedure after DDN Failure

The AF/SMF/UPF can subscribe to UE availability notification after DDN failure.

For the AF requesting UE availability notification after DDN failure, the AF buffers the packets for MT communication before receiving the notification for UE availability.

For the SMF subscribing UE availability notification after DDN failure, the UPF buffers the packets for MT communication until receiving the notification for UE availability from the AMF or reaching the buffering criteria indicated by the SMF/AMF.

For the UPF subscribing UE availability notification after DDN failure, the UPF buffers the packets for MT communication until receiving the notification for UE availability from the AMF/SMF or reaching the buffering criteria indicated by the AMF/SMF.

Figure 3 shows the notification procedure subscribed by the SMF for UE availability after DDN failure.

Figure 3: notification procedure for UE availability after DDN failure

- The UPF receives downlink data packets from data network.

- The UPF sends downlink data notification message to the SMF.

- When a SMF sends DDN (downlink data notification) to the AMF, it indicates also if it subscribes the notification service from the AMF for the UE availability when DDN failure.

- The AMF may initiate paging procedure.

- The DDN failure may be of many causes: paging failure, the paging suspension due to activated MICO mode, the UE activates high latency communication. If the DDN failure happens, the AMF replies to the SMF for the DDN failure with an indication for confirming the notification service for the next UE availability. Further, the AMF may base on UE’s reachability information to calculate buffering info and then include the following information in DDN response message: the buffer size (number of packets), buffering time, expected UE reachable time.

- The SMF forwards the buffering information to the UPF. The UPF starts to buffer packets according to the received buffering info.

- The UE contacts the AMF via registration update procedure.

- The AMF notifies the SMF/UPF for the UE availability.

- The UPF starts to deliver buffered downlink packets.

### 6.X.5	Impacts on existing entities and interfaces

### 6.X.6	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

################### END OF PROPOSAL ########################