SA WG2 Meeting #127	S2-183920

16 – 20 April, 2018, Sanya, China	(revision of S2-181906, 3217)

Source:	Ericsson, CATT

Title:	KI3: Solution for High latency communication

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	FS_CIoT_5G / Rel-16

Abstract of the contribution: High latency communication solution added. Contribution S2-183625 merged into clauses 6.X.4.1 and 6.X.4.2 of this version

# Discussion

This contribution discusses how to handle High Latency communication with extended buffering in CM-IDLE mode and in CM-CONNECTED with RRC Active/Inactive modes. It also proposes how to handle monitoring events related to HLCOM..

#### Extended buffering DL data in CM-CONNECTED with RRC Inactive mode

For UE in CM-CONNECTED with RRC inactive mode, several alternatives are possible. The figure below shows the flow of AMF providing buffering related support information to SMF/UPF and downlink data handling, where 3 different alternatives are discussed.

- Alternative 1: DATA SENT TO RAN DIRECTLY 
The main principle is that, SMF/UPF starts data buffering when RAN/AMF reports UE unreachability and that data is forwarded from RAN back to SMF/UPF in case RAN can’t delivery it.

  - Pros: data buffer in CN is triggered only when needed, i.e. less total notification signalling

  - Cons: new mechanism for forwarding between RAN and CN needed

- Alternative 2: DATA BUFFERED IN CN AND RAN QUERIED FOR UE REACHABILITY
The main principle is that, SMF/UPF starts data buffering always for PDU session that is subject to HLCOM based on indication from AMF and deliver data to RAN only when UE is reachable either based on UE reachability report or MO data/signalling.

  - Pros: N2 notification only needed when there is MT data and not at every active/inactive transition, i.e. less total notification signalling. Data only sent to RAN when UE is reachable. No need to handle duplicates of DL data (as alt 3). Forwarding from RAN not needed (alt 1).

  - Cons: extra signalling needed for MT data arriving in the time window when UE happens to be reachable.

- Alternative 3: DATA BUFFERED IN CN AND A COPY OF THE DATA SENT TO RAN
The main principle is that, SMF/UPF starts data buffering also for PDU session that is subject to HLCOM based on indication from AMF while delivery data to RAN through N3 tunnel and keeps the buffered data only in case a failure delivery is reported from RAN/AMF side. If no failure delivery is reported SMF/UPF assumes the data has been successfully delivered and discards the buffered data copy based on a local timer.

  - Pros: N2 notification only needed when there is MT data and not at every active/inactive transition, i.e. less total notification signalling. DL data Data delivery is always guaranteed without RAN forwarding data back to CN. Forwarding from RAN not needed (alt 1).

  - Cons: handling of duplicate data and dependency of timers to remove the duplicate data.

We recommend alternative 2 based on the pros and cons above.

Figure: Data Buffering in SMF/UPF for RRC_Inactive mode

For alternative 1

- UPF delivery DL data RAN through existing N3 tunnel.

- The RAN checks the UE reachability in RRC_Inactive mode. If UE is not reachable, i.e. due to KI4 (eDRX/DRX) or if RAN paging fails, the RAN sends N2 Notification to indicate the unreachability and data delivery failure. RAN provides also the buffering support information, if there is any.

- AMF send notification to SMF indicating UE unreachability and data delivery failure. AMF provides also extended data buffering support information (e.g. buffering duration) based on known info (e.g. negotiated eDRX interval or DRX interval) and/or buffering supporting info from RAN in step 2. AMF can use existing Nsmf_PDUSession_UpdateSMContext service, or Namf_EventExpousre_Notify, or any new service to deliver the info. UPF is notified by SMF in the similar way as in CM-IDLE mode.

- UPF starts to provide extended data buffering. SMF/UPF may also provide data forwarding tunnel info to RAN via AMF through Namf_Communication_N1N2MessageTransfer. The N2 message contains UL forwarding tunnel info. The message can either be new N2 message or modification of an existing N2 message, i.e. PDU session resource modification.  RAN forwards the data packets received in step 1 to UPF where it is buffered for the duration of extended buffering initiated after step 3.

- At next UE contact with the network e.g. after a paging initiated at step 2 or after a MO event, either AMF receives a N2 notification triggered by an RRC resume, or a N2 Path Switch Request if RRC resume triggers a RAN node change, the AMF notifies SMF/UPF of the UE reachability.

- UPF thus can deliver buffered downlink data to UE.

- If the UE does not contact with the network before expiry of the timer set in the SMF/UPF for the extended buffering duration, the buffered data is discarded in the SMF/UPF.

For alternative 2,

- a) Either when RAN put UE in RRC_Inactive state for UE with power save mode due to KI4 (e.g. eDRX/DRX), the RAN sends a N2 Notification to AMF. RAN provides buffering support information if there is any, and (or)

b) the AMF notifies SMF/UPF that the PDU session is subject for extended data buffering and buffering support info (e.g. buffering indicator and optionally duration) based on known info (e.g. negotiated eDRX/DRX interval) and/or buffering supporting info from RAN. The message used by AMF can be the existing Nsmf_PDUSession_UpdateSMContext service, or Namf_EventExpousre_Notify, or any new service to deliver the info. If data is buffered in the UPF and not in SMF, the SMF can notify the UPF in a similar way as in CM-IDLE mode.

- The application sends downlink data to UPF, and UPF will start to apply extended data buffering based on the indication received in step 0) above e.g. using the buffering indicator for the extended buffering.

- [conditional]The SMF may either request UE reachability report via Namf_MT_EnableUEReachability, based on notification from UPF which can either be similar to the DDN in CM-IDLE mode or a modification of the existing DDN message.

The SMF may optionally just wait for the next MO data from UE in step 5 to trigger the sending of downlink data without invoke enabling the UE reachability reporting above.

- The AMF may use the N2 Notification procedure to retrieve UE reachability info. RAN may provide new buffering support information, if there is any.

- The AMF notifies the SMF/UPF with the latest reachability info (either UE is reachable or still not reachable with new buffering support information)

- At next UE contact with the network, either AMF receives a N2 notification triggered by a RRC resume, or an N2 Path Switch Request if a RRC resume triggers a RAN node change, the AMF notifies the SMF/UPF of the UE reachability.

- UPF thus can deliver buffered downlink data to UE.

- If the UE does not contact with the network before expiry of the timer set in the SMF/UPF for the extended buffering duration, the buffered data is discarded in the SMF/UPF.

For alternative 3,

- a) Either when RAN put UE in RRC_Inactive state for UE with power save mode due to KI4 (e.g. eDRX/DRX), the RAN sends a N2 Notification to AMF, and (or)

b) the AMF notifies SMF/UPF that the PDU session is subject for extended data buffering based on known info (e.g. negotiated eDRX/DRX interval) and/or buffering supporting info from RAN.

- The application sends downlink data to UPF, and UPF will start to apply extended data buffering based on the indication received in step 0) above.

- The UPF also delivers buffered data to RAN through existing N3 tunnel while data is still buffered. UPF may also start a timer to monitor the already transfer data and discard it if not step 4 below is received after timer expires

- [Conditional]The RAN checks the UE reachability in RRC_Inactive mode when data is received. If UE is not reachable, i.e. due to KI4 (eDRX/DRX) or RAN paging fails, The RAN sends N2 Notification to indicate the unreachability and data delivery failure (data is discarded by RAN). RAN provides also the buffering support information, if there is any.

- AMF send notification to SMF indicating UE unreachability and data delivery failure. AMF provides also extended data buffering support information (e.g. buffering duration) based on known info (e.g. negotiated eDRX/DRX interval) and/or buffering supporting info from RAN in step 2. AMF can use existing Nsmf_PDUSession_UpdateSMContext service, or Namf_EventExpousre_Notify, or any new service to deliver the info. UPF is notified by SMF in the similar way as in CM-IDLE mode.

- At next UE contact with the network, either the AMF receives a N2 notification triggered by a RRC resume, or an N2 Path Switch Request if a RRC resume triggers a RAN node change, the AMF notifies the SMF/UPF of the UE reachability.

- UPF thus can deliver buffered downlink data to UE.

- If the UE does not contact with the network before expiry of the timer set in the SMF/UPF for the extended buffering duration, the buffered data is discarded in the SMF/UPF.

# Proposal

It is proposed to select alternative 2 based on the discussion above. That alternative is used in the solution below for high latency communication to the 5G CIoT TR 23.724.

***** First Change *****

# 6	Solutions

## 6.X	Solution #X: High latency communication

### 6.X.1	Introduction

This clause addresses the high latency communication key issue #3.

### 6.X.2	High Latency Functional Description

Functions for High latency communication may be used to handle mobile terminated (MT) communication with UEs being unreachable while using power saving functions e.g. UE Power Saving Mode, extended idle mode DRX or MICO. "High latency" refers to the initial response time before normal exchange of packets is established. That is, the time it takes before a UE has woken up from its power saving state and responded to the initial downlink packet(s).

High latency communication is handled by extended buffering of downlink data in the UPF/SMF. Optionally High latency communication can also be handled by notification procedures.

High latency communication by extended buffering of downlink data in the UPF/SMF is controlled by the AMF/SMF. The AMF asks the UPF/SMF, via SMF, to buffer downlink data until the UE is expected to wake up from its power saving state, see clause 6.X.4.1. If a SMF/UPF change is invoked, the buffered packets are forwarded and will not be lost. The number of packets to buffer is decided by the SMF/UPF, but the AMF may optionally provide supporting information. Optionally a suggested number of packets to buffer may also be provided by the SCS/AS as input to the SMF/UPF, see key issue 10.

An SCS/AS may optionally use notification procedures to handle High latency communication (see clause 6.X.4.2). The SCS/AS requests to receive notification when a UE wakes up from its power saving state to be able to send downlink data to the UE when the UE becomes reachable. Especially for long latencies (e.g. infrequent mobile terminated communication) this is suitable. This notification procedure can be triggered based on following monitoring events (see clause 4.15.3.1 in TS 23.502):

-	Monitoring event: UE Reachability; or

-	Monitoring event: Availability after DDN failure.

An SCS/AS may request a one-time "UE Reachability" notification when it wants to send data to the UE. Alternatively, the SCS/AS may request repeated "Availability after DDN failure" notifications where each notification is triggered by a DDN failure i.e. the SCS/AS sends a downlink packet which is discarded by SMF/UPF but which triggers the AMF to send an event notification to the SCS/AS next time the UE wakes up.

If CM-CONNECTED with RRC inactive state is activated for the UE, the downlink data handling (i.e. extended buffering) and notification is handled in the 5GC with the same way applied for CM-IDLE state, with additional support of information from RAN side. The RAN support information for high latency communication is provided through the N2 notification procedure, see clause 4.8.3 in TS 23.502.

The tools for High latency communication make the behaviour of the 3GPP network predictable when sending mobile terminated data to UEs applying power saving functions. The network will deliver downlink packets with high reliability for both stationary and mobile UEs when the UE wakes up from its power saving state. Therefore SCS/AS can adapt its retransmissions to reduce the load on both the SCS/AS itself and the network.

### 6.X.3	Support of EPC interworking

Editor's Note:	This clause describes if and how EPC-5GC interworking is supported by this solution.

### 6.X.4	High latency communication procedures

#### 6.X.4.1	Extended buffering

Functions for High latency communication may be used to handle mobile terminated (MT) communication with UEs being unreachable while using power saving functions (see KI 4). "High latency" refers to the initial response time before normal exchange of packets is established. That is, the time it takes before a UE has woken up from its power saving state and responded to the initial downlink packet(s).

The High latency communication includes invoking extended buffering of MT data at the UPF when the UE is in a power saving state and not reachable. The handling shall be specified in the Network Triggered Service Request procedure. Establishing the user plane for delivering the buffered data when the UE contacts the AMF by signalling shall be specified in the Registration procedure. The AMF uses its parameter DL Data Buffer Expiration Time in the MM context information to know if there is buffered DL data to be delivered when the UE becomes reachable. When set, the DL Data Buffer Expiration Time shall be cleared at any user plane setup to the RAN, i.e. buffered DL data can been delivered. At Registration procedure with AMF change, the old AMF shall indicate in the context response to the new AMF that buffered DL data is waiting and hence the new AMF shall establish the user plane for delivery of the buffered DL data. When the DL Data Buffer Expiration Time has expired, the AMF considers no DL data to be buffered and no indications of Buffered DL Data Waiting are sent during context transfers at Registration procedure. At Registration procedure with UPF change, the buffered DL data is forwarded to the new UPF.

For HLCOM, it’s the SMF/UPF that provide extended buffering, and the AMF provides buffering related support information towards SMF/UPF.

For UE in CM-IDLE mode and UE in CM-CONNECTED RRC-Inactive mode, AMF provides buffering support information to SMF/UPF based on the NAS level information negotiated between UE and AMF.

-	MICO related parameters

-	PSM related parameters (related to KI 4)

-	eDRX related parameters (related to KI 4). A UE using MICO mode is assumed to be able to benefit from Extended buffering. Any DL data being buffered for the UE can be delivered to the UE when it contacts the network.

Editor’s Note: The details for the handling of extended buffering is FFS and is dependent on KI 4 Power saving functions.

#### 6.X.4.2	DL data in CM-IDLE mode

For UE in CM-IDLE mode, the figure below shows the flow of AMF providing buffering related support information to SMF/UPF and downlink data handling. Extended buffering is used for UE that is using a power saving method (i.e. eDTX, PSM or MICO mode (see KI 4)). Extended buffering should not be used for a UE which has a monitoring event, 'UE Reachability' or 'Availability after DDN failure', configured.

Figure: Data Buffering in SMF/UPF for CM-IDLE mode

The Network Triggered Service Request procedure as specified in TS 23.502 [x] clause 4.2.3.3 would be updated based on the description below.

1.	The application sends downlink data to UPF.

2.	The UPF sends a Downlink Data Notification (DDN) message to the SMF if there is no N3 tunnel.

3.	The SMF initiates Namf_Comm_N1N2MessageTransfer_Reqeust service towards AMF to setup N3 tunnel.

4.	The AMF check the UE reachability in CM-IDLE mode. UE not reachable due to power saving. If UE is not reachable due to eDRX/DRX, the AMF schedules a paging.

5.   The AMF send Namf_Comm_N1N2MessageTRansfer Response to indicate the UE unreachable. If the UE is unreachable due to power saving, the AMF also indicates DL Buffering Requested and provides buffering supporting information (e.g. DL Buffering Duration time and optionally a DL Buffering Suggested Packet Count).

The AMF stores a new value for the DL Data Buffer Expiration Time in the MM context for the UE based on the DL Buffering Duration time. The DL Data Buffer Expiration Time is used for UEs using power saving state and indicates that there are buffered DL data waiting in the UPF. When the DL Data Buffer Expiration Time has expired, the AMF considers no DL data to be buffered and no indications of Buffered DL Data Waiting are sent during context transfers at Registration procedure.

Note: If AMF is configured to support Notify on availability after DDN failure as specified in clause 6.X.4.5.2, Extended data buffering is not invoked.

6.	The SMF send DDN failure with DL Buffering Requested indication and buffering supporting information to UPF. The SMF includes the DL Buffering Duration time and optionally a DL Buffering Suggested Packet Count.

7.	The UPF starts extended data buffering based on the buffering support information. The UPF stores a new value for the DL Data Buffer Expiration Time based on the DL Buffering Duration time and does not send any additional Data Notification if subsequent downlink data packets are received in the UPF before the DL Buffer Expiration Time has expired for the UE.

8.	When UE enters CM-CONNECTED mode, the AMF notifies SMF that previous has received UE unreachable indication in step 3.

At for example periodic or mobility registration update as specified in TS 23.502 [x] clause 4.2.2.2 extended buffering is handled with the following addition to step 22 (Registration Accept). If the DL Data Buffer Expiration Time for the UE in the AMF has not expired, the user plane setup procedure is activated. The message sequence should be similar for the UE triggered Service Request procedure specified in clause 4.2.3.2 of TS 23.502 [x] from the step when AMF activates the User Plane connections of the PDU Session(s).

9.	SMF/UPF trigger the setting of N3 tunnel and delivery the buffered data packets.

#### 6.X.4.3	DL data in CM-CONNECTED mode without extended buffering

The figure below shows the handling of downlink data in CM-CONNECTED mode without extended buffering. This procedure is used for UEs not applying any power saving method. For UEs that do apply power saving methods (i.e. potentially very long buffering times), see clause 6.x.4.4.

Figure: DL data in CM-CONNECTED mode without extended buffering

1.	DL data arriving in UPF is forwarded on the N3 towards RAN.

2a.	If the RRC connection is in RRC Inactive mode, RAN pages the UE. DL data is temporarily buffered in the RAN. If the UE does not respond to paging, e.g. may be temporarily unreachable, the RAN discards the DL data.

2b.	If the RRC connection is in active mode, or transitions to active mode, the DL data is sent to the UE.

#### 6.X.4.4	DL data in CM-CONNECTED mode with Extended buffering

This procedure shows how DL data in CM-CONNECTED mode with Extended buffering is handled. Extended buffering is used for UE that is using a power saving method (i.e.. eDTX/DRX, PSM or MICO mode (see KI 4)). Extended buffering should not be used for a UE which has a monitoring event, 'UE Reachability' or 'Availability after DDN failure', configured. For a UE in RRC inactive mode, the AMF provides buffering related support information to SMF/UPF and downlink data handling. NOTE: The alternative 2 as of the discussion part in S2-183920 is used.

Figure: DL Data Buffering in SMF/UPF for RRC_Inactive mode

0.	Data Buffering is established, either:

0a.	each time when RAN transition a UE using eDRX/DRX to RRC_Inactive state, the RAN sends a N2 Notification to AMF. RAN provides buffering support information if there is any, and (or)

0b.	initially when the PDU Session is moved to CM-CONNECTED and RRC Active/Inactive are used, the AMF notifies SMF/UPF that the PDU session is subject for extended data buffering and buffering support info (e.g. buffering indicator and optionally duration) based on known info (e.g. negotiated eDRX/DRX interval) and/or buffering supporting info from RAN. The message used by AMF can be the existing Nsmf_PDUSession_UpdateSMContext service, or Namf_EventExpousre_Notify, or any new service to deliver the info. If data is buffered in the UPF and not in SMF, the SMF can notify the UPF in a similar way as in CM-IDLE mode.

Editor’s Note: The details of buffering support information are FFS.

1.	The application sends downlink data to UPF, and UPF will start to apply extended data buffering based on the indication received in step 0b above e.g. using the buffering information for the extended buffering.

2.	[conditional] The SMF may either request UE reachability report via Namf_MT_EnableUEReachability, based on notification from UPF which can either be similar to the DDN in CM-IDLE mode or a modification of the existing DDN message.

The SMF may optionally just wait for the next MO data from UE in step 5 to trigger the sending of downlink data without invoke enabling the UE reachability reporting above.

3.	The AMF may use the N2 Notification procedure to retrieve UE reachability info. RAN may provide new buffering support information, if there is any.

4.	The AMF notifies the SMF/UPF with the latest reachability info (either UE is reachable or still not reachable with new buffering support information). If the UE is reachable, step 5 is skipped and the procedure continues in step 6.

5.	At next UE contact with the network, either AMF receives a N2 notification triggered by a RRC resume, or an N2 Path Switch Request if a RRC resume triggers a RAN node change, the AMF notifies the SMF/UPF of the UE reachability.

6.	UPF thus can deliver buffered downlink data to UE.

7.	If the UE does not contact with the network before expiry of the timer set in the SMF/UPF for the extended buffering duration, the buffered data is discarded in the SMF/UPF.

8.	RAN moves the UE to RRC Inactive state.

#### 6.X.4.5	Notification procedures

If SCS/AS selects to send downlink data when UE becomes reachable, it can use one-time UE-Reachability notification (see clause 6.X.4.5.1). If SCS/AS may also select the Notification of UE availability after downlink data delivery failure (see clause 6.X.4.5.2).

##### 6.X.4.5.1	Event Subscription and Notification of UE reachability

If an SCS/AS wants to send downlink packet(s) when UE becomes reachable, the SCS/AS can subscribe for one-time "UE Reachability" monitoring event by following either Event Exposure using NEF specified in TS 23.502 clause 4.15.3, or Core Network Internal Event Exposure specified in TS 23.502 clause 4.15.4. The SCS/AS sends the packet data when it receives notification that the UE is reachable.

Figure: TS 23.502: Event Subscription of UE reachability notification

If UE is in CM-CONNECTED and RRC state is unknown (i.e. N2 notification procedure is not activated yet), the AMF shall trigger N2 Notification procedure to retrieve UE information from RAN, see TS 23.502 clause 4.8.3 for details.

Figure: Event Subscription and Notification for UE reachability

Editor’s Note: Detailed steps and dependencies to KI Power saving are FFS.

If the SCS/AS optionally wants to fine-tune the delivery of the downlink data within the time-window when the UE is reachable, the SCS/AS can configure optional parameters with proper value (as described in clause 6.X.4.2.3).

##### 6.X.4.5.2	Notification of Availability after DDN failure

If an SCS/AS wants to get notifications when the UE becomes reachable after a proceeding downlink data delivery failure, the SCS/AS can subscribe for the “Notification of availability after DDN failure” monitoring event.

This is a different monitoring event from the "UE Reachability" monitoring event specified in 6.X.4.2.1. This information is provided to the AMF at registration through subscription data, see clause 6.X.4.2.3. The AMF notes this and sets a Notify-on-available-after-DDN-failure flag after a DDN failure. If the flag is set when the UE next contacts the network, the serving node notifies the NEF that the UE is reachable and clears the flag.

An important use case for this feature is the application that wants to communicate with a UE that sleeps for a long time. If downlink packets from the application are not delivered, the application recognizes that the UE is not available by lack of response within a reasonable time from the UE, and then await notification from the network (i.e. from the AMF via the NEF) of UE reachability.

For UE in CM-IDLE mode, the figure below shows the flow of AMF providing UE availability to NEF/AF after DDN failure.

Figure: Notification - Availability Notification after DDN Failure (CM-IDLE)

1.	The application sends downlink data to UPF.

2.	The UPF sends a Downlink Data Notification (DDN) message to the SMF if there is no N3 tunnel.

3.	The SMF initiates Namf_Comm_N1N2MessageTransfer_Reqeust service towards AMF to setup N3 tunnel.

4.	The AMF check the UE reachability in CM-IDLE mode. If UE is not reachable, i.e. due to KI4 (such as PSM, MICO) or paging fails, The AMF send Namf_Comm_N1N2MessageTRansfer Response to indicate the failure

5.	The SMF send DDN failure to UPF and the UPF discards data packets.

6.	The AMF shall set the “Notify on availability after DDN failure” flag.

7.	When UE enters CM-CONNECTED mode, the AMF notes that UE is available and that the Notify-on-availability-after-DDN-failure flag for the UE is set.

8.	The AMF trigger Namf_EventExposure_Notification towards the NEF/Application.

9.   The AMF also clear the Notify-on-availability-after-DDN-failure flag for the UE.

10.	The application realizes the UE is reachable and re-send data.

For UE in RRC-Inactive state where the N3 tunnel exists, RAN can discard the data if UE is not reachable. But it shall indicate to AMF that downlink data is discarded, and AMF shall set the “Notify on availability after DDN failure” flag. At next N2 notification with change to RRC-Connected (assuming N2 notification is already activated during configuration in 6.X.4.2.3) or Patch Switch Request, AMF triggers notification to application.

The figure below shows the flow of AMF providing UE availability to NEF/AF after DDN failure.

Figure: Notification - Availability Notification after DDN Failure (CM-CONNECTED / RRC-Inactive)

- The application sends downlink data to UPF/RAN.

- The RAN check the UE reachability in RRC_Inactive mode. If UE is not reachable, i.e. due to KI4 (such as eDRX) or RAN paging fails, The RAN send N2 Notificaiton to indicate the failure

- The AMF shall set the “Notify on availability after DDN failure” flag.

- AMF may optionally send an event notification to the NEF and SCS/AS as a confirmation that the DL data in step 1 was received and that an 'Availability after DDN failure' notification will be sent as soon as the UE becomes available.

- When UE enters CM-CONNECTED mode (RAN notify AMF either through N2 notification in case same RAN node, or Path Switch Request in case of RAN node change), the AMF notes that UE is available and that the Notify-on-availability-after-DDN-failure flag for the UE is set (by step 3).

- The AMF clears the Notify-on-availability-after-DDN-failure flag for the UE and sends a Namf_EventExposure_Notification towards the NEF/Application.

- The application realizes the UE is reachable and re-send downlink data.

- RAN moves the UE to RRC Inactive state.

##### 6.X.4.5.3	Configuration for notification related approaches

###### 6.X.4.5.3.1	Configuration of notifications via UDM

The figure below illustrates the procedure of configuring monitoring events parameters at the UDM or the AMF. The procedure is similar to external parameter provisioning in TS 23.502 clause 4.15.6.2

The procedure is also used for replacing and deleting a monitoring event configuration.

Editor's Note: If this existing procedure shall be updated for High Latency or a new procedure defined is FFS

Figure: Monitoring event parameters configuration and deletion via UDM procedure

1.	AF (SCS/AS) provides one or more configuration parameter(s) related to monitoring even for HLCOM to be updated in Nnef_ParameterProvision_UpdateRequest to the NEF.

The PUI identifies the UE and the Transaction Reference ID identifies the transaction request between NEF and AF (SCS/AS).

2.	If the AF (SCS/AS) is authorised by the NEF to provision the parameters, the NEF requests to update and store the provisioned parameters as part of the subscriber data via Update Request message, the message includes the provisioned data and NEF reference ID.

If the requester is not authorised to provision data, then the NEF continues in step 4 indicating the reason to failure in Nnef_ParameterProvision_Update response.

3.	UDM/UDR resolves the PUI to SUPI and stores the monitoring events parameters as part of the subscription data and responds with Update response message.

If the SUPI cannot be derived from the PUI, then the Update Response indicates the reason in the cause value

4.	NEF responds the request with Nnef_ParameterProvision_update response. If the procedure failed, the cause value indicates the reason.

5.	UDM/UDR notifies the AMF of the updated subscriber data via Nudm_SDM_UpdateNotification Notify message.

6. 	The AMF handling according to 6.x.4.2.4

###### 6.X.4.5.3.2	Configuration directly at AMF

Editor's Note: It is FFS whether the existing procedure for configuration directly at the AMF in 23.502 is sufficient or if it needs updates for HLCOM.

##### 6.X.4.5.4	AMF handling to trigger RAN notification

For UE in UE is in CM-IDLE, or CM-CONNECTED with RRC states known as RRC inactive, AMF triggers notification towards NEF as specified in 6.X.4.2.2 from step 8 when UE becomes RRC active (i.e. when AMF receives NAS signalling from UE or RAN indicates UE is in RRC active through N2 Notification procedure).

For UE in CM-CONNECTED with RRC state known as RRC active, AMF immediately triggers notification procedure towards NEF as specified in 6.X.4.2.2 from step 8.

For UE in CM-CONNECTED with RRC state unknown, AMF shall trigger the N2 Notification procedure to retrieve the UE RRC state info and act accordingly as specified above based on RRC states information from RAN.

### 6.X.5	Impacts on existing entities and interfaces

Editor's Note: This clause describes impacts to existing entities and interfaces.

The AMF, SMF and UPF are enhanced to support extended buffering as described in clause 6.X.4.

The N11 and N4 interfaces are enhanced to support extended buffering as described in clause 6.X.4.

The AMF and NEF are enhanced to support Notification procedures as described in clause 6.X.4.

### 6.X.6	Evaluation

Editor's Note: This clause provides an evaluation of the solution.

***** End of Changes *****