SA WG2 Meeting #S2-127	S2-184357

23 - 27 April, 2018, Sanya, China	was S2-184246 was S2-183590

Title:	Reply LS on QoS

Response to:	-

Release:	Release 15

Work Item:	5GS_Ph1

Source:	SA2

To:	RAN2, RAN3, CT1, CT4

Cc:                              -

Contact Person:

#### Name:	Aeneas Dodd-Noble

E-mail Address:	noblea at cisco dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

SA2 thanks RAN2 on their LS (R2-1801645) and would like to provide further feedback in addition to the SA2 LS response in S2-182856.

This response provides further clarification on the question that RAN2 asked in their original LS:

QFI Size

For the SDAP sublayer, in order to keep the header contained within one byte, RAN2 has agreed to limit the number of QFI which can be signalled to 64. Therefore, RAN2 would like to understand if SA2 expects to use more than 64 reflective flows per PDU session per UE at a time.

SA2 has re-discussed this topic and would like to confirm that also limiting the QFI to 6 bits (i.e. 64 values) in NAS is considered adequate for Rel-15 5GS system.

In SA2#127, following discussions on some companies’ proposal, SA2 concluded that a potential extension of the QFI beyond 6 bits may be considered in a future release (e.g. in Rel-16). SA2 would like to stress that Release 15 should be defined such that future extensibility should not be precluded.

2. Actions:

To RAN2, RAN3, CT1, CT4 group

ACTION: 	SA2 respectfully asks RAN2, RAN3, CT1 and CT4 to take into account that Release 15 should be defined without precluding extensibility of the QFI beyond 6 bits in a future release and let SA2 know of updates.

3. Dates of Next TSG-SA2 Meetings:

| 3GPPSA2#127bis | 28 May - 1 Jun 2018 | Newport Beach | US |
| --- | --- | --- | --- |
| 3GPPSA2#128 | 2-7 July 2018 | Vilinius | LT |