SA WG2 Meeting #128-bis	S2-187748

Sophia Antipolis, France, 20 - 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0557 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unnecessary Slice-based Wait Timer for AN connection release for congestion control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Wistron |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In March 2018 SA2#126, SA2 has approved to introduce a slice based wait timer in RRC connection release message (as defined in S2-182985) in TS 23.501, clause 5.19.5.2. SA2 has sent the LS to inform RAN2 for such design requirement and requested for the technical support for such functionality in RAN. In July 2018 RAN2#AH-1807 ad-hoc meeting, RAN2 has decided not to support the wait timer as proposed by SA2. RAN2 sent a LS (see S2-187657) to inform SA2 for its decision and asked SA2 to further clarify the SA2’s requirement for the slice based wait timer in RRC connection release message. In addition, both RAN2 and CT1 have approved the additional functionality for Unified Access Control (UAC) mechanism according to the approved UAC requirements for slice based access control support defined by SA1 as described in TS 22.261, clause 6.22. UAC mechanisms are now specified in both RAN2 and CT1 to support also slice based congestion control. After some further analysis (see S2-187747), it is concluded that the slice based wait timer proposed by SA2 overlaps the slice based congestion control supported by UAC and could cause in deterministic system behavior for slice based congestion control. Further details of the analysis for the justification of this CR can be referred to S2-187747. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR removes the slice-based wait timer support from the clause 5.19.5.2 in TS 23.501 as such function could conflict with the UAC approach for slice-based congestion management that has been implemented by RAN2 and CT1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The slice based wait timer overlaps the slice based congestion control supported by UAC and could cause in deterministic system behavior for slice based congestion control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.19.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### ***** First Change *****

#### 5.19.5.2	AMF Overload Control

Under unusual circumstances, if AMF has reached overload situation, the AMF activates NAS level congestion control as specified in Clause 5.19.7 and AMF restricts the load that the 5G-AN node(s) are generating, if the 5G-AN is configured to support overload control. N2 overload control can be achieved by the AMF invoking the N2 overload procedure (see TS 38.300 [27] and TS 38.413 [34]) to all or to a proportion of the 5G-AN nodes with which the AMF has N2 connections. The AMF may include the S-NSSAI(s) in N2 overload control message sent to 5G-AN node(s) to indicate the Network Slice(s) with which NAS signalling is to be restricted. To reflect the amount of load that the AMF wishes to reduce, the AMF can adjust the proportion of 5G-AN nodes which are sent NGAP OVERLOAD START message, and the content of the overload start procedure.

The AMF should select the 5G-AN node(s) to which it triggers overload start procedure at random to avoid that multiple AMFs in an AMF Set request reduction of load from the same subset of 5G-AN node(s).

A 5G-AN node supports rejecting or releasing of 5G-AN signalling connection establishments for certain UEs as specified in TS 38.331 [28]. Additionally, a 5G-AN node provides support for the barring of UEs as described in clause 6.22 in TS 22.261 [2]. These mechanisms are further specified in TS 38.331 [28].

Using the overload start procedure, the AMF can request the 5G-AN node to:

a)	reject 5G-AN signaling connection (RRC Connection over 3GPP access or UE-N3IWF connection over N3GPP access) requests that are for non-emergency and non-high priority mobile originated services; or

b)	reject new 5G-AN signaling connection requests for uplink NAS signalling transmission to that AMF;

c)	only permit 5G-AN signaling connection requests for emergency sessions and mobile terminated services for that AMF; or

d)	only permit 5G-AN signaling connection requests for high priority sessions and mobile terminated services for that AMF;

NOTE 2:	The 5G-AN signaling connection requests listed in this clause also include the request from UE in RRC-Inactive state.

The AMF can provide percentage value that indicates how much amount of signalling traffic to be rejected in the overload start message, and the 5G-AN node may consider this value for congestion control.

When rejecting a 5G-AN signaling connection request for overload reasons (cases a and b above) the 5G-AN indicates to the UE an appropriate wait timer value that limits further 5G-AN signaling connection requests until the wait timer expires.

During an overload situation, the AMF should attempt to maintain support for emergency services and for MPS.

When the AMF is recovering, the AMF can either:

-	trigger overload start procedure with new percentage value that permit more signalling traffic to be carried, or

-	the AMF trigger overload stop procedure.

to some or all of the 5G-AN node(s).

#### ***** End of Change *****