SA WG2 Meeting #128	S2-186404

2 – 6 July 2018, Vilnius, Lithuania 	(revision of S2-18XXXX)

Source:	Ericsson, China Mobile, LG Electronics, CATT

Title:	Introduction of SBI-based SMSGW in 5GC

Document for:	Discussion

Agenda Item:	7.1

Work Item / Release:	TEI16 / Rel-16

Abstract of the contribution: Discussion on the need for SBI based SMSGW for SMS transport and for SMS related UDM interactions.

# Introduction

This contribution discusses the need for SBI-based SMSGW for SMS transport and for SMS related UDM interactions within the 5GC.

## Background

The current architecture for SMS over NAS in TS 23.501 looks as follows:

And

It was decided in SA2 that MT SMS domain selection is in IP-SM-GW/SMS Router and that the IP-SM-GW/SMS Router may connect to MSC, MME and SMSF via one of the standardized interfaces as shown in TS 23.040. According to TS 23.040, these interfaces for MT SMS are 3 (E when roaming) and SGd, hence both MAP and Diameter are possible as transport protocol. Similar MO SMS may be sent via 3/E or SGd to SMS-IWMSC. It can be noted that both MO and MT SMS may be sent via the roaming interface, whereby the MT SMS may be via an IP-SM-GW/SMS Router in B HPMN while the MO SMS will be sent to SMS-IWMSC in A HPMN. See TS 23.040 for further details.

Currently, for SMS transport between SMSF and AMF the following services-based interfaces are used:

- Nsmsf: For MO SMS from AMF to SMSF

- Namf: For MT SMS from SMSF to AMF

There is no service based interface defined for MO / MT SMS from or to IP-SM-GW/SMS Router or any other NF that may want to send/receive SMS via interaction with SMSF.

Additionally, SMSF makes use of Nudm services for managing SMSF registration and SMS subscription data with UDM/HSS. No service-based interface is defined either for:

- enabling the retrieval of routing information for the transfer of short messages (used by SMS-GMSC, SMS-Router, IP-SM-GW),

- the report of status of the delivery status of a short message (used by SMS-GMSC) and

- the alerting of the SMS-SC between the UDM/HSS, the SMS-GMSC/IWMSC, SMS Router and/or IP-SM-GW.

The interaction between UDM and HSS is implementation specific.

## Deployment use cases on SMSF

The currently documented architecture allows deployments of SMSF supporting MAP only, Diameter only or MAP and Diameter in combination for legacy interfaces. In any case, one of the legacy MAP or Diameter interfaces must be supported by the SMSF in addition to the service-based interfaces with UDM/HSS and AMF.

Similarly, the interactions from the central SMS functions and UDM/HSS are only based on one of the legacy MAP or Diameter interfaces.

## Full SBI for SMS transport

If a deployment of SMSF wants to avoid supporting MAP or Diameter interfaces, then MO and MT SMS must be transported via a service-based interfaces as follows:

- New Nnew_service/Nxy for MO SMS, e.g. supported by IP-SM-GW/SMS Router and used by SMSF

  - Proposal: Nsmsgw_ SMService

  - Proposal: The SMSGW can be co-located with IP-SM-GW.

- Extension of Nsmsf for MT SMS, supported by SMSF and used by SMSGW

See also the following figure that includes also the interaction between AMF and SMSF for MO and MT SMS.

According to TS 23.040, the SMS-GMSC and the SMS Router interact via reference point 2 (MAP) or S6c (Diameter) with the HLR/HSS for enabling the retrieval of routing information for the transfer of short messages (e.g. to get the current MSC, SGSN, MME and/or SMSF addresses), the report of status of the delivery status of a short message and the alerting of SMS-SC. According to TS 23.204, the IP-SM-GW interacts with the reference point J or with S6c with the HLR/HSS for the same purpose. The use of Sh towards the HSS by the IP-SM-GW is related to SMS over IP and only mentioned for completeness and not further discussed in this paper.

If a deployment of UDM wants to avoid supporting MAP or Diameter interfaces, the corresponding interactions to e.g. retrieve the routing information for the transfer of a short message within the 5GC (i.e. to retrieve the SMSF address), report of status of the delivery status of a short message and the alerting of the SMS-SC could also use a service-based interface (Nudm/Nyz).

This would result in the following architecture.

And

According to the figures above, it is proposed that the SMSGW supports service-based interfaces for SMS transport with SMSF and for interworking with UDM/HSS. The SMSGW can be co-located with IP-SM-GW.

NOTE: 	The SMS-GMSC/IWMSC may still interwork with HSS using legacy MAP or Diameter interfaces to retrieve the MAP address of the SMSGW.

It can also be discussed to also have SBI between SMSGW and SMS-GMSC/IWMSC. However, unless all SMS-GMSC/IWMSC are supporting SBI, the SMSGW has to support also MAP and/or Diameter.

This will allow that legacy MAP or Diameter interfaces are not needed within the 5GC. Supporting only service-based interfaces for SMS transport allows also that the SMSGW in the B-HPMN can handle both MT SMS and MO SMS (like the IP-SM-GW does today already for SMS over IP) and avoids impacting the SMS-IWMSC in the A-HPMN.

## Service Discovery

Current specification of SMSF discovery and selection describes for the AMF both the approach of service discovery (via NRF) and the use of UE context in SMSF data (stored in the UDM) or from an old AMF

If introducing service-based interface for SMS transport between SMSF and SMSGW, then the SMSF needs to discover the new Nxy for MO SMS and the SMSGW needs to discover the SMSF/Nsmsf.

Similarly, the SMSGW needs to discover the UDM Nudm/Nyz service.

# 2	Proposal

In summary, the following aspects are discussed above and for your reference, two companion CRs, one for TS 23.501 (S2-18CCCC) and the other for TS 23.502 (S2-18XXXX) show the proposed changes.

- the need for service-based interface for SMS transport between SMSF and SMSGW, that can be co-located with IP-SM-GW,

- the need for service-based interface for SMS interworking between UDM and SMSGW and

- the related service discovery for

  - the new Nsmsgw_ SMService for MO SMS by SMSF,

  - the extended SMSF/Nsmsf by the SMSGW and

  - the UDM/Nudm by the SMSGW.

It is proposed to allow TEI16 meeting time to discuss a CR to TS 23.501 and a CR to TS 23.502 with the above proposal.