SA WG2 Meeting #128	S2-186445

July 2nd – 6th 2018, Vilnius, Lithuania

Source:	Orange

Title:	Clarification on the establishment procedure of proxyied MPTCP connections

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	ATSSS/Rel-16

Abstract of the contribution: This contribution proposes a method for establishing proxyied MPTCP connections without requiring another encapsulation scheme to transport flows between the UE and the MPTCP proxy. The proposed method allows establishing network-assisted MPTCP connections in a 0-RTT manner avoiding therefore penalizing proxyied MPTCP connections compared to non-proxyied TCP connections.

Introduction

Network-Assisted MPTCP deployment models are designed to facilitate the adoption of MPTCP for the establishment of multi-path communications without making any assumption about the support of MPTCP capabilities by communicating peers.

Section 6.5 of TR23.793 defines an architecture to take advantage of establishing communications over multiple paths (resource bonding, resiliency, etc.) even if the remote endpoint is not MPTCP-capable. To that aim, an MPTCP proxy is introduced to relay MPTCP connections created by the UE into TCP connections and vice versa.

The address of the MPTCP proxy is discovered by the UE in step 13 of TR23.793, Section 6.5.4.1. Nevertheless, TR23.793 does not specify how MPTCP subflows can be established via an MPTCP proxy (that is, how the address of the MPTCP proxy is used by the UE). This contribution fills that void.  It does so by proposing the UE and MPTCP proxy to adhere to the 0-RTT proxy procedure defined in [I-D.ietf-tcpm-convertes].

In order to avoid the burden of encapsulation and additional signaling between the UE and the MPTCP proxy, [I-D.ietf-tcpm-convertes] specifies how a plain transport mode is enabled, so that packets are exchanged between an UE and its upstream MPTCP proxy without requiring the activation of any additional encapsulation scheme to transport MPTCP flows (e.g., IP-in-IP [RFC2473], GRE [RFC1701]). Also, the approach in [I-D.ietf-tcpm-convertes] avoids the need for out-of-band signaling. For example, if SOCKS is used between the UE and the MPTCP proxy, up to 6 SOCKS messages may have to be exchanged between the UE and the MPTCP proxy before actual payload data to be transferred, thereby yielding several tens of milliseconds of extra delay before the connection is established. Such extra delay is not acceptable.

[I-D.ietf-tcpm-convertes] can be used to assist MPTCP connections even if endpoints are MPTCP-capable. Furthermore, it accommodates various deployment contexts, such as those that require the preservation of the source IP address and others characterized by an address sharing design. The solution is compatible with IPv4/IPv6.

### ************************************First Change**********************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[3]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[4]	3GPP TS 23.139: "3GPP system - fixed broadband access network interworking".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[8]	draft-ietf-mptcp-rfc6824bis-10: "TCP Extensions for Multipath Operation with Multiple Addresses".

[9]	draft-ietf-tcpm-converters-01: "0-RTT TCP Convert Protocol".

### ************************************End Change**********************************

### *********************************Second Change**********************************

#### 6.5.4.1	MPTCP PDU session establishment procedure

Figure 6.5.4-1-1: UE-requested MPTCP PDU Session Establishment for non-roaming and roaming with local breakout

Editor's note:	It is FFS whether instead of the above PDU session setup procedure the UE could setup two independent PDU sessions for MPTCP procedures. In this case it is also FFS how the UE receives the address of the MPTCP proxy.

1.	The UE initiates the "UE Requested PDU Session Establishment" procedure as specified in TS 23.502 [6] clause 4.3.2.2.1 on 3GPP access with an additional preference to establish an MPTCP PDU Session. The UE can also first initiate the PDU Session Establishment procedure on untrusted non-3GPP access.

2.	The AMF selects an SMF that supports MPTCP procedures.

3.	The AMF sends an Nsmf_PDUSession_CreateSMContext Request to SMF.

4.	The SMF registers with the UDM.

5.	The SMF sends an Nsmf_PDUSession_CreateSMContext Response to AMF.

6.	Optional Secondary authorization/authentication.

7a.	If dynamic PCC is deployed, the SMF performs PCF selection.

7b.	The SMF may perform a Session Management Policy Establishment.

8.	The SMF selects an UPF that incorporates an MPTCP proxy functionality. As an alternative, the UPF can select a standalone MPTCP proxy.

9.	SMF may perform a Session Management Policy Modification procedure.

10-17.	The user plane of the PDU session over 3GPP access is established and the UE creates the first MPTCP subflow over the 3GPP access following the procedure defined in clause 6.5.4.2.

-	In step 11, the SMF indicates to AMF the MPTCP proxy address.

-	In step 13, the UE receives a PDU Session Establishment Accept message containing the MPTCP proxy address, thus, the UE determines that the MPTCP PDU session over 3GPP access is established.

19-27.	The user-plane of the PDU session over untrusted non-3GPP access is established and the UE creates an additional MPTCP subflow over the untrusted non-3GPP access. The second MPTCP subflow is established following the procedure defined in clause 6.5.4.2.

NOTE:	The steps 10-17 and 19-27 can be executed in parallel.

### ************************************End Change**********************************

### *************************Third Change (New Text) ******************************

#### 6.5.4.2	Procedure for establishing proxied MPTCP connections

The UE and MPTCP proxy shall follow the procedure defined in clause 3.3 of IETF draft-ietf-tcpm-converters-01 [9] for establishing proxied MPTCP subflows.

In particular, the SYN message to create the initial subflow via the MPTCP proxy shall include the Connect TLV defined in IETF draft-ietf-tcpm-converters-01 [9]. Also, the destination IP address of that SYN message shall be set to the one of the MPTCP proxy discovered in step 13 of clause 6.5.4.1. Subsequent subflows of the same MPTCP connection are established directly between the UE and the MPTCP proxy following normal MPTCP procedures [8].

### ************************************End Change**********************************